Cinemaster Driver version 0.940  03/10/97

Quick Installation:

If you have a previous installation of a cinemaster board, Windows 95 will not prompt you again to install the drivers.  Use the "System" dialog of the control panel.  Select the "Device Manager" panel,

and doubleclick on the "Cinemaster" icon in the "Sound, video and game controllers" sub section. Choose the "Driver" panel, and push the "Change Driver... " button.  Then use "Have Disk..." and use this driver disk.  Windows will now install the drivers for you.

_____________________________________________________________________________

Direct Draw Overlay Support

Starting 0.905 the Cinemaster drivers supports the Direct Draw hardware overlay.  This feature is found on several new VGA boards like the Virge VX.  It enables the board to playback video upto the full display resolution.  There are some drawbacks associated with this feature.  Typically the image has to be scaled down before it is sent to the VGA board, to fit the requirements for the VGA scaler.  This is either due to limited PCI bandwidth, limited VGA memory or limited scaler capabilities.  The effect is that the Image looks smeared and blurred.

To have the best looking display in all cases, the driver does not switch to overlay until the maximum resolution of the PCI inlay chip has been reached.

As there are cards that limit the PCI inlay to a smaller size (due to PCI bandwidth limitations like the Virge VX) we added an .ini file entry to set the maximum horizontal resolution used for inlay.  If the image needs to be scaled larger than that, the driver falls back to overlay mode.

You can change the inlay performance value for your display card, using the custom tab in the system configuration manager.

_____________________________________________________________________________

DVD Continuous Playback Support

Starting 0.905 the Cinemaster drivers support parts of the DVD filing system.  DVD specs require a huge video file to be separated into several sub gig size files named „*.vob“.  There is a control file named „*.ifo“ for the complete video sequence.  If this file is attempted to be played, the driver joins all the associated „*.vob“ files to a single clip, and treats it as a single video file, that can be coninuously played.

_____________________________________________________________________________

DVD Programchain Support

Starting with version 0.910 the Cinemaster drivers support the splitting of DVD files into program chains. These programchains can individually be played. Therefore to new MCI commands have been introduces.

setprogramchain to <pgc>

Jumps to the selected programchain.

status current programchain

status number of programchains

These commands return information about the number of program chains and the currently selected programchain.

_____________________________________________________________________________

DVD Parental Control Support

Starting with version 0.910 the Cinemaster drivers support parental control for DVD titles. A new MCI command has been introduced for this purpose.

setparental mask to <mask>

This command sets the parental level for the current playback. Each bit in the mask represents one parental level, that may be playbacked. This command will probably replaced by a command to directly set the parental level based on stream information.

_____________________________________________________________________________

DVD Multi Angle Playback Support

Starting with version 0.910 the Cinemaster drivers support playback of multi angle clips. Two new MCI commands have been introduced for controlling this.

setangle to <angle>

Selects the specified angle.

status current programchain

status number of programchains

These commands return information about the number of available angles and the currently selected angle.

_____________________________________________________________________________

Troubleshooting:

No audio on the audio output:

Check the settings of the Cinemaster mixer, check the settings in the "Properties" dialog in the media player.

Jerky playback, slow mouse response, when playing from DVD:

This driver requires at least a 75MHz pentium with 8MBytes RAM.  It is highly recommended that you use a bus mastering IDE driver.

No video in window but on external monitor:

You probably have a non PCI-Inlay conform display card (you need 32768, 65536 or 16Mio colors).

If you had inlay before, you should have the system recheck this.  Load the "cinemast.ini" file in a text editor and delete all sections that look like [0x1002/0x4758/1152/864/15].  Then reboot.  The driver will rescan the framebuffer.

Maybe the RAM timing limitations on your display board are too high.  Make the display window smaller. If this was the reason, display should appear at some point.

Color video in window, but grey on monitor:

Check the video standard settings in the "Properties" dialog using the media player.  Change it to the video standard of your external monitor.

Errors in video decoding, stopping of playback

Check if you have the latest EPLD installed on your Cinemaster board.

_____________________________________________________________________________

Known Bugs:

_____________________________________________________________________________

BUG #2: No AC3 Settings for Cinemaster

Description :

The AC3 settings in the "AC3 Setup" dialog of the "Properties" dialog have no meaning for the cinemaster board, as this is currently only a two channel version.

Workaround :

Do not use these settings.

Comment :

This dialog will be removed or ghosted for two channel boards.

_____________________________________________________________________________

BUG #6: Position Information in Frames and Milliseconds Wrong.

Description :

For many clips the position information in frames and millisecond units is wrong.  This is due to incorrect bit rate parameters in the clip.  Several clips we checked just use a default value of 10MBit.  As the internal position is calculated based on the bytes consumed, this will lead to wrong values.

Workaround :

Use correct bit stream timing values in the streams.  For accurate seeking use position information in bytes instead of frames or milliseconds.

Comment :

This will either be fixed by correct streams (it does very well with several streams).  Otherwis the position could be extracted using the GOP header.

_____________________________________________________________________________

BUG #8: Noise on AC3 Audio Output

Description:

There is a lot of noise on the AC3 audio output.  This is due to an unstable PLL used as the reference for the audio DACs.

Workaround:

None

Comment:

Next board revision will use a different and more stable PLL.  This fixes the noise problem.

_____________________________________________________________________________

Fixed Bugs from Revision 0.940:

_____________________________________________________________________________

BUG #10: No playback of  PAL MPEG 2 clips.

Description:

PAL MPEG 2 clips can not be played with the current driver version.

Workaround:

None

Comment:

This bug is due to memory limitations.

_____________________________________________________________________________

Fixed Bugs from Revision 0.930:

_____________________________________________________________________________

BUG #12: Problems playing files with more than 4 gig in size.

Description:

Due to some obsolete internal limitations, the current driver has problems playing files with more than 4 gigabyte in size.  This bug is connected to the new DVD filing system support.

Workaround:

none

Comment:

Will be fixed in the next revision.

_____________________________________________________________________________

BUG #13: Problems playing back field based clips

Description:

The driver has problems playing back field based clips. The problems are especially heavy when playing back audio/video multiplexed clips. This bug is due to some synchronisation problems between decoding and displaying of fields.

Workaround:

none

Comment:

Has to be further investigated.

_____________________________________________________________________________

Fixed Bugs from Revision 0.910:

_____________________________________________________________________________

BUG #1: DupField used in MPEG 2

Description :

The dup field MPEG 1 scaling enhancement (assuming odd and even field are identical) is also applied to MPEG 2 / DVD clips (odd and even fields are different).  This leads to vertical distortions in the video window, when scaling between half size and full size.

Workaround :

Uncheck the "DupField" chek box in the "Properties" dialog, using the media player.

Comment :

Will be corrected.

_____________________________________________________________________________

BUG #9: Colorbars always in PAL

Description:

Colorbars used as the idle screen on an external monitor are currently always in PAL.  On NTSC only monitors, this will lead to a black and white image.

Workaround:

None

Comment:

Will have to be fixed.

_____________________________________________________________________________

BUG #11: Problems shutting down system when using the mixer.

Description:

On one machine we had problems shutting down the system, when the cinemaster mixer device was enabled.  This seems to be somehow related to the Direct Draw functions we are using.

Workaround:

Do not use either Direct Draw (delete „\win95\system\VDServ32“) or the cinemaster mixer.

Comment:

Will have to be fixed.

_____________________________________________________________________________

Fixed Bugs from Revision 0.903:

_____________________________________________________________________________

BUG #3: On the Fly Displaymode Change

Description :

When changing the displaymode between PAL & NTSC on the fly, using the configure dialog, the clip advances for about a tenth of a second, but the slider position of the "Media Player" is not changed

correctly.

Workaround :

Reopen the clip after switching display mode.

Comment :

Will be fixed together with #4

_____________________________________________________________________________

BUG #4: Multiple Instance Position Problems

Description :

When using multiple instances (opening more than one clip at a time), and switching between the clips using the play command, the position information of the clip is not maintained properly.

Workaround :

Do not use multiple instances in the current version.

Comment :

Will be fixed together with #3

_____________________________________________________________________________

BUG #5: Single Step in Audio-Video  Clips

Description :

Single step does only work for four to five frames.  This is due to the audio part not stepping.  The effect is that the audio buffer runs full, so the video buffer can not refill and further stepping is prohibited by the starvation logic.

Workaround :

Do not use the "step" command in the current version, use "seek" instead.

Comment :

_____________________________________________________________________________

BUG #7: Wrong Decoding in "I-Frame" rich Clips

Description:

There seems to be a problem in the current version of the MPEG 2 video decoder, when more than two I or P-frames are decoded contiguosly.  This effect typically shows up in I-frame only video clips or scenes (some encoders switch to I and P-Frame only mode, when there is too much motion going on), or in IBP (used in some MPEG 1 clips instead of IBBP) clips, when frame rate conversion from NTSC to PAL is used (which forces the decoder to skip frames).

Workaround:

None

Comment:

The second problem (frame rate conversion) is fixed for MPEG 1 in the driver by not dropping a complete frame but two fields instead (1122334466778899 -> 1122344556778890).  This approach can not be taken for MPEG 2 as the decoder typically needs to fields to decode a frame.

It seems that it is not a decoding but a displaying problem, as consecutive P-Frames are decoded correctly.

We are checking this problem with SGS Thomson, but there is no known bug on there side yet.