��������������������������������ķ

� �                              �

� �   0038  Utility Diskette     �

� �                              �

� �         Release 2.27         �

� �                              �

��������������������������������Ľ

This README.DOC file contains the most update information

about the utility diskette shown below.

EMS driver

Hard Disk Cache

RAM Disk

Print Spooler

I/O Clock Change

HIDOS Driver

Please read the following for description on the utilities

and the installation method.

1. EMS Driver

DEMS.SYS is a LIM 4.0 compatible EMS Driver. This driver is

applicable with at least 1M bytes on board memory installed and

EMS feature (in the BIOS setup menu) must be enabled.

To install DEMS :

1.1  Enter the setup menu by pressing the DEL key during

system boot up.

1.2  Enable EMS MEMORY inside the setup menu. If both

EXTENDED MEMORY and EMS MEMORY are used at the same

time, the size of EMS MEMORY can be altered by means

of changing the EXTENDED MEMORY size inside the setup

menu.

1.3  Execute the installation program called INSTALL.EXE (please

refer to topic 5) which can be found in this diskette.

1.4. Or follow the procedures below.

A. Copy the DEMS.SYS file from this disk to the root

directory of the disk using in starting up the comput-

er.

B. Then add the following line into the CONFIG.SYS file.

DEVICE=DEMS.SYS

C. Reboot the system

A sign-on message from the DEMS.SYS should be seen as

follow :

Expanded Memory Manager Version 4.31

Copyright (C) 1990

Total Page : XXXX    ( YYYYY KB)

Page Frame : ZZZZ

I/O Address: PPPP

2. Hard Disk Cache

CACHE.SYS is an utility for improving your hard disk perfo-

rmace. This program is applicable with at least 1M bytes on

board memory and EMS feature (in the BIOS setup menu) must be

enabled.

To install CACHE :

2.1. Use an installation program called INSTALL.EXE (please

refer to topic 5) which can be found in this diskette.

2.2. Or follow the procedures below.

A. Copy the CACHE.SYS file from this disk to the root

directory of the disk using in starting up the comput-

er.

B. Then add the following lines into the CONFIG.SYS file.

DEVICE=DEMS.SYS

DEVICE=CACHE.SYS <size>

Where size must be 16 kilo bytes or a mutiple of 16

kilo bytes (minimium 64KB).

DEMS.SYS is an expanded memory management driver for

the Hard Disk Cache, RAM Disk, Print Spooler and

other software.

eg. DEVICE=DEMS.SYS          (must install DEMS first)

DEVICE=CACHE.SYS 384     (to get a 384 KB Cache)

C. Reboot the system

A sign-on message from the CACHE.SYS should be seen as

shown below :

Hard Disk Cache Version 1.02

Copyright (C) 1990

Installation Success

3. RAM Disk

The RAM Disk driver is called EMMDISK.SYS in this diskette.

RAM  Disk is a high speed virtual disk. This program is appli-

cable at least 1M bytes on board memory and EMS feature ( in

the BIOS setup menu) must be enabled.

To install RAM Disk :

3.1. Execute the installation program called INSTALL.EXE (please

refer to topic 5) which can be found in this diskette.

3.2. Or follow the procedures below.

A. Copy the EMMDISK.SYS file from this disk to the root

directory of the boot disk.

B. Then add the following lines into the CONFIG.SYS file.

DEVICE=DEMS.SYS

DEVICE=EMMDISK.SYS <size>

Where size must be 16 kilo bytes or a mutiple of 16

kilo bytes.

DEMS.SYS is an expanded memory management driver for

the Hard Disk Cache, RAM Disk, Print Spooler and

other software.

eg. DEVICE=DEMS.SYS          (must install MMD first)

DEVICE=EMMDISK.SYS 384    (to get a 384 KB RAM Disk)

C. Reboot the system

A sign-on message from the TDISK.SYS should be seen as

shown below.

EMM Disk Version 1.23

Copyright (C) 1990

Installation Success

4. Print Spooler

The Print Spooler is called SPOOLER.SYS in this diskette. It

provides a large data buffer for printing so as to reduce The

the idle time of computer. This program is applicable with at

least 1M bytes on board memory and EMS feature (in the BIOS setup

menu) must be enabled.

To install SPOOLER :

4.1. Execute an installation program called INSTALL.EXE (please

refer to topic 5) which can be found in this diskette.

4.2. Or follow the procedures below.

A. Copy the SPOOLER.SYS file from this disk to the root

directory of the boot disk.

B. Then add the following lines into the CONFIG.SYS file

of the disk.

DEVICE=DEMS.SYS

DEVICE=SPOOLER.SYS <size> <port discription>

Where size must be 16 kilo bytes or a mutiple of 16

kilo bytes. The port number must be started with LPT

and followed by 1 to 4 numbers, where the number is/are

the printer port(s) which you want to be buffering.

DEMS.SYS is an expanded memory management driver for

the Hard Disk Cache, RAM Disk, Print Spooler and

other software.

eg. DEVICE=DEMS.SYS               (must install MMD first)

DEVICE=SPOOLER.SYS 384 LPT13  (buffering printer port 1

and port 3 using 384 KB

RAMs)

C. Reboot the system

A sign-on message from the SPOOLER.SYS should be seen shown

as follow.

Print Spooler Version 1.02

Copyright (C) 1990

Installation Success

5. Utilities installation

With the INSTALL.EXE the utilities can be installed to your

hard disk or a floppy disk provided that  they  must be bootable.

After installation, the utilities will be available whenever you

boot up the system using the installed disk.

The installation program can be used for modification after first

time installation.

To do first time installation :

5.1. Place this diskette into drive A

5.2. Type A:

5.3. Type INSTALL

5.4. Follow the instructions shown, and select the source drive,

boot drive, target drive, target path and the size of RAM

Disk, Hard Disk Cache and Print Spooler.

5.5. Press F10 to save and modify the CONFIG.SYS and AUTOEXEC.BAT.

5.6. Restart the system using the installed disk.

6. I/O Clock Change

The default clock is 14 MHz

Under DOS environment by typing the followings to change the

default:

"IOSPEED 14" for I/O Clock is 14 MHz

"IOSPEED 1"  for I/O Clock same as CPU clock

"IOSPEED 2"  for I/O Clock same as 1/2 CPU clock

Please make sure the IOSPEED program is in the current

directory.

7. HIDOS Driver

THIS DRDOS V6.0 driver HIDOS.SYS can relocate DRDOS V6.0 kernel

to upper memory (memory  addressed  between 640KB and 1MB) even

only 1M  memory on board . So more  conventional  memory can be

reserved for large application.

To install HIDOS.SYS :

A. Copy the HIDOS.SYS file from this disk to the root

directory of the boot disk.

B. Then add the following lines into the CONFIG.SYS file

of the disk.

DEVICE=HIDOS.SYS

** This driver can only run under DRDOS V6.0 Operation System.

8. Files in this diskette

README.DOC            (This document)

INSTALL.EXE           (for installation and setup)

README.EXE            (for viewing this document)

SCLEAR.EXE            (for clearing the Print Spooler)

CACHE.SYS             (Hard Disk Cache)

DEMS.SYS              (Driver for expanded memory)

EMMDISK.SYS           (RAM Disk)

SPOOLER.SYS           (Print Spooler)

IOSPEED.EXE           (I/O Clock Change)

HIDOS.SYS             (Relocates DRDOS V6.0 KERNEL to high memory)

Please check if your diskette contains these files. Don't hesitate

to consult your dealer for missing files or any diskette damage.

*** END ***