ST21R.COM Documentation:

I'll try to keep this short, since the included ST21R.ASM file

should provide plenty of documentation.  There are just a few things

I thought I should mention, though.

First, the program, as written, requires that the jumpers on the

controller be set for a BIOS address of D800:0000 and port addresses

of 170-177h and 376-377h.  This means that the jumpers will have to

be set as follows:  JP1 shorted, JP2 open.  This is to prevent con-

flicts with your other hard drive controller.  In addition, JP3 should

also be open to enable the onboard BIOS ROM.

Make sure that you allow for enough drives with the LASTDRIVE=

statement in your CONFIG.SYS file, and then place ST21R.COM in your

AUTOEXEC.BAT file as early as possible, before MSCDEX (if present),

and before any RAM drives or disk cache.  Among other things, this

will allow the disk cache to cache the extra drives.

ST21R.COM is NOT a driver!  It uses the driver already present

in DOS, and therefore will work just as if DOS recognized the drives

in the first place.  This means full INT 13h support, which means

you can even use FORMAT and FDISK.  However, there is one warning

you should be aware of.

Although the current version of ST21R.COM doesn't support more

than one partition per drive (and only a primary partition, at that),

you may have more than one partition on your other drives.  All of

your drives will work properly in this case, but FDISK will show

different drive letters for the drives/partitions than those DOS

identifies them by.  It will also assign volume labels to the wrong

drives.  This is due to the fact that DOS normally assigns drive

letters in the following order:  First, one primary partition on

each drive is assigned a drive letter, if present, and then after

all drives are checked, DOS starts over with the first hard drive

and begins assigning letters to logical drives in the extended

partitions.

Since the drives on the ST21R are assigned after all of this

has already been done, they are out of order.  FDISK assigns letters

to all drives the way DOS does, but FDISK can see the extra drives

that DOS couldn't see when it initialized, and therefore, the drive

letters will be assigned differently if there are multiple partitions

on any drive.  Just be aware of this and you should be okay.  You

may have to give FDISK an incorrect volume label when deleting a

partition, so be VERY careful!  Subsequent versions of ST21R.COM may

fix this by changing the assignments already made by DOS, but for now

just be careful.  I will not be responsible for any lost data if you

fail to heed this warning!

When you first boot the computer, the ST21R's BIOS will give an

error message.  Just ignore it.  When ST21R.COM runs, it will force

the controller to install itself.  ST21R.COM doesn't provide any

messages of its own.  When you see a message that drive C or drive D

is being installed, this message is coming from the BIOS, so the

drive letters will be incorrect.  The controller thinks it is

installing the first drives present, and thus calls them C and D.

DOS will recognize them differently.

Also note that ST21R.COM doesn't presently do any error checking

during boot-up, so if there are any problems with the controller or

drives besides the fact that there are too many drives (as far as the

BIOS is concerned), the computer may lock up.  It's a good idea to keep

a bootable floppy handy just in case this ever happens.

If you can use this program, use it.  There is no mandatory

registration fee, but if you feel like sending me some money to help

compensate me for the many hours I spent on it, feel free.  Send only

what you think it's worth, but no more.  If you can't afford it,

don't worry about it.  Go ahead and use it anyway.  I retain all

rights to it, but I've included the source code so you can see how

it works, and if you have a different controller (or a different

revision of the BIOS--mine is revision 1.2), and would like to

modify it to work with your controller, go ahead.  If you have any

questions, comments, or bug reports, you can contact me in the Fido-

Net 80xxx Assembly Language Echo, via NetMail at FidoNet address

1:2240/176, or at the following address:

Chuck Martin

446 Walnut St.

Mt. Morris, MI  48458