SLFP packet driver info:

param	default 	description

-----	------- 	-----------

/p	0x60		Packet driver interrupt number

/i	4		IRQ for com port. Use 4 for COM1 and 3 for COM2.

/b	0x3F8		I/O base address for com port. Use 0x3F8 for

COM1 and 0x2F8 for COM2.

/s	2400		Line speed bit rate.

/d	-none-		Dial sequence. Use atdt764-4800 for local dial.

/t	60		Initial timeout. Amount of time to wait for

the dialup and initial handshake with Merit.

/h      1               Initial SLFP handshake. This can be turned off

with value 0. This allows one to make the

initial connection with a terminal emulator

like kermit, and enter PCIP at the TERMINAL=

prompt. This is useful if you have problems

with the SCP training up into SLFP protocol

mode.

/f     1               Initial FIFO check value. If set to one,

slfp checks for a 16550a UART for the COM

port. This allows the high performence FIFO

on chip to be used. If set to zero, slfp

assumes a normal 8250 chip set.