The following instruction will install the video driver

for an installed version of Windows 3.0

=============================================================================

step 1 - Install Windows 3.0 on your system as per instruction from MicroSoft.

step 2 - Copy unzipped Windows 3.0 drivers to a blank formatted diskette.

Label diskette "EV-673 Windows 3.0 Drivers".

step 3 - Have your Windows 3.0 install diskettes as well as the Everex

driver diskette containing the Windows 3.0 drivers ready.

step 4 - Change to the directory Windows 3.0 is installed.  From DOS, run

the Windows SETUP program.  (DO NOT run SETUP from within windows.)

step 5 - When SETUP displays your current configuration, move the highlight

bar up to the 'Display:' line, and press [ENTER].

step 6 - Move the highlight down to the bottom of the list of display drivers

to the one marked 'Other (Requires disk provided by a hardware

manufacturer.)' and press [ENTER].

step 7 - Insert your diskette containing these Windows v3.0 drivers in

to your drive A: slot.

step 8 - When SETUP prompts you where the drivers are located, type 'A:\'

step 9 - Select the resolution in which you wish to use Windows v3.0.

step 10- Exit SETUP.  SETUP may prompt you for other diskettes, including

the Windows 3.0 installation diskettes.

step 11- Windows v3.0 is now configured to use the new, higher resolution mode.

=============================================================================