Triones

Bus Master / PIO Device Drivers

for Intel PIIX/PIIX3 Chipset

Quick Installation Guide

(June 01, 1996)

##### 1. MS-DOS / WINDOWS / WFW

- You should install CD-ROM in secondary channel in Windows (WFW).

- Change the current directory to a: or b:.

- Under DOS command line prompt, run the SETUP directly.

After entering the setup utility, just follow the instructions of the setup.

##### 2. WINDOWS NT3.5

- From the Program Manager, double click on "Windows NT Setup" in the Main group.

- Select "Options/Add/Remove SCSI Adapters..."

- Click on Add.

- The "Select SCSI Adapter Option" dialog will appear; select "Other (Requires a disk from a hardware manufacturer)" from the "Adapter:" list box.

- Next, the "Insert Diskette" dialog box will appear; insert the Triones PIIX/PIIX3 Bus Master EIDE/ATAPI Driver disk into Drive A: and type in "a:\winnt35" and <Return>.

- Next, the "Select OEM Option" dialog box will appear; select "PIIX/PIIX3 Bus Master EIDE/ATAPI" and click "OK."

- Next, the "Select SCSI Adapter Option" dialogbox will appear; click on the "Install" button in the dialog box. If installation is successful, the "SCSI Adapter Setup" dialog box will reappear, and "PIIX/PIIX3 Bus Master EIDE/ATAPI" will be listed. That means the driver is installed.

- Reboot your system to load the driver.

##### 3. WINDOWS 95

- Close any running applications.

- Insert the floppy disk into drive A:.

- Open "My Computer", double click "Drive A:".

- Double click "Win95".

- Double click the Setup program.

- Then just follow the instruction.

##### 4. NOVELL 3.X&4.X

- Copy the driver TRIN4X.DSK or TRIN312.DSK to the subdirectory which holds the file SERVER.EXE.

- Boot up the file server.

- On the system console and on the command prompt state, type in:

load TRIN4X.DSK <Enter> or load TRIN312.DSK <Enter>

You can also include the above command line into the start up file STARTUP.NCF to load the driver automatically.

##### 5. SCO UNIX

- Install SCO UNIX 3.2.x or SCO Open Desktop 3.x using the default SCO IDE driver.

- Reboot your SCO UNIX system.

- Insert Triones supplied diskette into the 3.5 inch floppy drive of your system. Use doscp command to copy the file TRISCO.TAR to your /tmp directory. For example, if you are using floppy drive A, type: doscp a:/scounix/trisco.tar /tmp/trisco.tar

- From root directory, type the following commands:

- mkdir /inst

- cd /inst

- tar xvf /tmp/trisco.tar .

- (NOTE: there is a period at the end of the last command.)

- Now, insert a blank diskette into the floppy drive A and type:

- tar cvf /dev/<your floppy drive A device name> .

- (NOTE: there is a period at the end of the last command.)

- Your floppy drive A device name could be:

- rfd096ds15 5.25 DSHD

- rfd0135ds18 3.5 DSHD

- rfd048ds9 5.25 DSDD

- rfd0135ds9 3.5 DSDD

- Now you have already made an installation diskette for the device driver. Start the installation by typing: custom

- Select the "Install" operation and then follow the guided steps. When prompted "Do you want to set the device configuration (y/n)", answer "n".

- Reboot your SCO UNIX system.

##### 6. SCO UNIX 5.0

- Install SCO UNIX 5.0 using the default SCO IDE driver.

- Reboot your SCO UNIX system.

- Insert Triones supplied diskette into the 3.5inch floppy drive of your system. Use doscp command to copy the file TRISCO5.TAR to your /tmp directory. For example, if you are using floppy drive A, type:

- doscp  a:/scounix/trisco5.tar  /tmp/trisco5.tar

- From root directory, type the following commands:

- mkdir  /inst

- cd  /inst

- tar  xvf  /tmp/trisco5.tar  .

- (NOTE: there is a period at the end of the last command.)

- Install the driver by typing:

- ./install

- Reboot your SCO UNIX system.

##### 7. OS/2 2.0 and W A R P 3.X

- Copy TRIOS2.ADD from the floppy diskette to your hard disk under the OS2 directory (i.e., C:\OS2).

- Edit C:\CONFIG.SYS to replace BASEDEV=IBM1S506.ADD with BASEDEV=TRIOS2.ADD

- Reboot the system.

NOTE:

For more information (like parameter settings, driver de-installation, etc.), please refer to the README.TXT file on the diskette.