DEC  1994

SCO UNIX V.3 (TCP/IP V1.1X) Driver Installation

for

DE-650 PCMCIA Ethernet Card

=================================================

The following files will be used in this installation:

Files          Contents

----------     ---------------------------------------

DE65N.O        DE-650 Driver Program

DLK            D-Link Unix script to setup adapter

TCP            For Unix streams

0. In DOS environment prepared diskette for setup65n :

Insert D-Link DE-650 Series Driver Programs in A: drive.

Insert a blank diskette in B: drive. ( for D-Link SCO UNIX driver)

Copy D-Link SCO UNIX Driver into root directory of blank diskette.

C> copy A:\ibmpc\scounix\tcpip1.2x\*.*  B:\

1. Login as the root user.

2. Installing TCP/IP software through the custom utility.

# custom

Select install

Select A New Product

Select entire to install all of the packages

3. Rename /usr/lib/mkdev/tcp to /usr/lib/mkdev/tcp.save if you want to keep

original /usr/lib/mkdev/tcp file. (it will be modified by next step)

4.1 Installation diskette in Dos format.

Put D-Link SCO Unix Driver diskette(in step 0 prepared) to floppy A and use

doscp command to copy D-Link SCO UNIX Driver to following directories.

# cd /

# mkdir /usr/lib/tcprt/dlk

# doscp A:/readme.doc /usr/lib/tcprt/dlk/readme.doc

# doscp A:/de65n.o /usr/lib/tcprt/dlk/de65n.o

# doscp A:/dlk /usr/lib/mkdev/dlk

# doscp A:/tcp /usr/lib/mkdev/tcp

# chmod +x /usr/lib/mkdev/dlk

# chmod +x /usr/lib/mkdev/tcp

4.2 Installation diskette in UNIX tar format.

Put your D-Link Installation Diskette to floppy A and use tar command to

copy D-Link SCO UNIX Driver.

# cd /

# tar xvnf /dev/fd048ds9

5. Installing D-Link SCO UNIX Driver.

# mkdev dlk

Do you wish to install or delete the dlk driver?  Type i to install

A display of information about driver.            Type y to continue

Select interrupt vector number [5].              Type value

Select I/O base address [300].                   Type hexadecimal value

Select memory base address [d0000].              Type hexadecimal value

Select PCMCIA slot number [0].                   Type value

Do you wish to relink the kernel now?             Type y to relink

Do you want this kernel to boot by default?       Type y to default

Do you want the kernel environment rebuilt?       Type y to rebuilt

6. Configuring Network interface.

# mkdev tcp

Which driver(s) will TCP/IP be using?             Type dlk

DOMAIN name.                                      Enter key

IP address.                                       n.n.n.n

Broadcast address of all 0's?                     [y/n]

Confirm broadcast address.                        Enter key

Confirm netmask.                                  Enter key

Which driver(s) will TCP/IP be using?             Type quit

7. Reboot UNIX.

# init 6