STAR9E.DOC

----------

The STAR (ADI) Printer Driver

(STAR9E.EXE 1.0) for AutoCAD (TM) ADI (TM)

Copyright 1989, 1990, Autodesk, Inc.

Revision 1.0

April 27, 1992

License is granted for the use of this software and documentation

on a no-charge, sharing basis.  You may distribute and copy this

software freely as long as it remains a completely unaltered

package and is not sold for profit.

INTRODUCTION

This document explains the installation and use of STAR9E.EXE ver

1.0, an Autodesk Device Interface (ADI) driver designed to run

with STAR 9 pin printers in conjunction with AutoCAD. This driver

was developed by STAR Micronics Deutschland GmbH and does not represent

an endorsement of any particular product, including any of the STAR printers.

Autodesk, Inc.  makes no warranty, either expressed or implied,

including but not limited to any implied warranties of

merchantability or fitness for a particular purpose, regarding

these materials and makes such materials available on an "as-is"

basis.

HARDWARE REQUIREMENTS

The hardware requirements specific to using the STAR9E printer ADI

driver with AutoCAD.

o This driver is suitable for use with all 9 pin STAR

printers in the "LC", "FR" and "ZA" series, but some printers can't

use all of the options provided by the driver.  For example, a

narrow carriage printer can't handle wide plots and some

models can't print in color.  Please refer to your STAR

user's manual for details.

o A parallel card and cable is required as described in the

AutoCAD Installation & Performance Guide.

o Serial output is not supported because the time required to

plot would be unacceptably long.

HARDWARE INSTALLATION

The STAR printer must be connected to the computer with a

standard Centronics-type parallel cable.  Chapter 8 of the

AutoCAD Installation & Performance Guide contains all the

information regarding pin connections and switch settings.

COPYING THE DRIVER SOFTWARE

The STAR9E printer ADI (ver 1.0) driver for AutoCAD is 16k bytes

long.  The disk also contains a file STAR9E.HLP for on-line command line

options.  You will probably want to install the driver in the

same directory where you store AutoCAD.  Here's how to copy the

driver software onto your hard disk.  (The example assumes an

AutoCAD directory is on drive C and is named ACAD).

1. Change to the directory where you store AutoCAD:

C:> CD ACAD [Return]

2. Insert the ADI Driver diskette in drive A.

3. Copy the driver files to your hard disk:

C:> COPY A:STAR9E.*  [Return]

INSTALLING THE ADI STAR9E DRIVER

The AutoCAD STAR printer ADI driver must be installed (loaded

into memory) prior to entering AutoCAD.  It is a "terminate and

stay resident" (TSR) software program which can use up to 77K of

RAM depending on how it is configured.  Once loaded, it remains

in memory until you turn off or reboot your computer.

To install the driver:

C:> STAR9E [Return]

The STAR ADI driver will then be installed with the following

defaults:

o  Interrupt 7b(hex)

o  Monochrome

o  180 x 180 Dots per inch print density

o  Output directly to printer

o  Input from AutoCAD

o  form feed after every plot

To change this setup, you can choose to include one or more

options on the command line.  These options are:

-lx parallel port x

-f plot from a file     -v interrupt #      -c color      -p no form feed

-o plot to a file       -w wide paper       -dx density x

To install the driver to do color printer plots (if your STAR is

color capable and you have the color ribbon installed):

C:> STAR9E -C  [Return]

(default is monochrome)

To set the horizontal plot density

C:> STAR9E -Dx [Return]  (where x is 1 to 6 in the following table):

(default is 180 x 180)

x     Horizontal   Vertical

Density    Density

1         60         72

2         120        72

3         240        72

4         240        144

You can also install the driver to write plot files directly to

your disk drive for later printing (this will create a file named

'fname' on your default drive for later printing).  This is

useful for printing multiple copies of a plot.  See the section

titled "OUTPUTTING STAR9E PRINTER FILES FOR DIRECT PRINTING FROM

DISK" for more details:

C:> STAR9E -Ofname [Return]

You can use the driver to read AutoCAD Binary printer plot files

directly from your disk drive.  This allows you to use plotting

modes which would use too much memory for plotting from AutoCAD

as a TSR.  This option does not install the driver as a TSR, and

may be used repeatedly.  See the section titled "USE AS A

POSTPROCESSOR" for more details:

C:> STAR9E -Ffname [Return]

Note:  These filenames must conform to DOS standards.

To install the driver for printing on wide paper (will create a

plot up to 13.6" wide):

C:> STAR9E -W [Return]

(default is 8")

To install the driver to use a different parallel port:

C:> STAR9E -Lx [Return] (where x is the new port assignment 1, 2 or 3)

(default is LPT1)

To install the driver to not send a form feed after each plot:

C:> STAR9E -P [Return]

(default is a form feed after every plot)

To install the driver for a different interrupt vector (default

is 7B hex):

C:> STAR9E -Vxx [Return]  (where xx is the new interrupt number)

To display a list of these options:

C:> STAR9E -H or -? [Return]  Prints a help message from star9e.hlp

To install the driver using a combination of the above options:

C:> STAR9E -Oplotdata.plt -c -v7b   This will do the following:

1.  Create a plot file 'plotdata.plt' of your drawing on your

default drive for plotting at a later time.

2.  Make a color print of the plot.

3.  The interrupt vector will be 78 (in hex).

MEMORY REQUIREMENTS FOR DIFFERENT CONFIGURATIONS

RAM Memory required by driver (8" wide plot)

n   density                Black & White     Color

-----------------------------------------------------------

X   x  Y

1 = 60  x 72  dpi              19k             27k

2 = 120 x 72  dpi (default)    20k             26k

3 = 240 x 72  dpi              20k             29k

4 = 240 x 144 dpi              22k             35k

RAM memory required for wide carriage   (13.6" wide plot)

n   density                 Black & White     Color

-----------------------------------------------------------

X   x  Y

1 = 60  x 72  dpi               20k            28k

2 = 120 x 72  dpi (default)     21k            33k

3 = 240 x 72  dpi               22k            38k

4 = 240 x 144 dpi               25k            48k

** Note: There are only 2 densities in the vertical direction.

If you do not have AutoCAD and this driver configured for the

same dots per inch, your drawing may be expanded or compressed in

the y-axis.

** Note: Each time you type in the command STAR9E (except for the

special case of the "F" option for plotting binary files),

another copy of the driver software is loaded into memory, using

up to 82k more bytes of RAM.  You need to "reboot" the system (to

clear out the any copies of STAR9E.EXE).

CONFIGURING AUTOCAD FOR THE STAR9E 9 PIN PRINTER DRIVER It is

necessary to reconfigure AutoCAD for an installed ADI printer

plotter driver.  You will only need to do this before you want to

use the STAR9E ADI driver for the first time, or when you change the

paper width or the horizontal or the vertical plot densities of

your STAR ADI.  Note the present configuration of STAR9E.EXE when

you install it.  AutoCAD's printer plotter configuration

densities must be equal for your plots to be correct.

Reconfigure AutoCAD for an installed printer plotter ADI driver <2>.

Be sure that you:

1.  Select output format as an Installed ADI driver <2>.

2.  Set the maximum horizontal (X) plot size in inches.

These values are 7.99" for the standard size or 13.59"

for the wide carriage printers.  The vertical (Y) axis

should be set to your paper size, 10.5 is a good value.

3.  Set the density equal to the installed density of the

driver STAR9E.EXE.  If AutoCAD's density (dots/inch) is

greater, your drawing will be expanded along that axis

and may be clipped.  If the drivers density is greater,

your drawing will be compressed along that axis.

4.  The vertical or y-axis has only 2 densities, 72

dots/inch and 144 dpi.  For most printer plotting, 72

dpi should be fine.

See the Chapter 8, the printer configuration section of the

Installation & Performance Guide to AutoCAD, Chapter 13 of your

main Reference Manual for AutoCAD details, and your STAR9E Users

guide for printer details.

AutoCAD will now be configured to do a printer plot using the

installed STAR9E ADI driver.

CREATING PRINTER PLOTS WITH THE STAR9E 9 PIN PRINTER DRIVER

To send plots to the STAR printer from AutoCAD's drawing editor

select Plot then Printer from the AutoCAD menu or type 'prplot'

at the command line.  From AutoCAD's Main Menu select Printer

Plot <4>.  Make sure your printer is connected, turned on and on

line and hit return when ready.  Your plot will be produced on

your STAR 9 pin printer.

** Note: printer plots take a long time to produce, so don't

worry if nothing happens for a while.  They can take a very long

time depending on the complexity of the drawing, if you chose

color or if the plot density is high.

OUTPUTTING STAR9E PRINTER FILES FOR DIRECT PRINTING FROM DISK

You may wish to create a file that may be printed multiple times

directly from disk.  You can do this with the DOS copy command

without running AutoCAD or STAR9E driver after the file is created.

CAUTION: This option should be used with care.  It is possible to

create a file larger than 8 Megabytes if all the settings are at

their maximum.  If it is used with some thought it will produce

good results.

Here is an example:

Say you want to produce a 240 x 72 dpi color plot of the

site-3d.dwg on 13.59" wide paper.

Install the STAR9E ADI printer driver by typing:

C:> STAR9E -d3 -Osite-3d.eps -C -W [Return]

Configure AutoCAD as a ADI printer at 240 x 72 dpi, and a

horizontal width of 13.59".  Run AutoCAD and printer plot the

site-3d.dwg file.  The file site-3d.eps will be created on the

default disk for later printing from the command line.

You can print the file from the DOS command line by typing:

C:> COPY SITE-3D.EPS PRN /B [Return]

In the above example, if you plotted the <E>xtents it would

create a file of 3 Megabytes and would take over an hour to be

produced on a IBM system 30.  But if you created a file of the

<D>isplay at 240 x 72 dpi for 7.99" wide paper by typing the

following:

C:> STAR9E -C -Osite-3d.eps

the file site-3d.eps would be only .3 Megabytes.  The creation

time for this file is only 5 minutes on an AT clone running at 10

Mhz.

USE AS A POSTPROCESSOR

The ADI driver STAR9E.EXE can work as a postprocessor, reading

AutoCAD ADI binary files from your disk and sending them directly

to the printer.  Printer plot to an ADI binary printer file from

AutoCAD.  Then, when the Autodesk product is no longer running, you

may use STAR9E.EXE to plot from the binary file to your STAR printer.

Here's an example:

You want to produce a 240x144 dpi color plot of site-3d.dwg on

13.59" wide paper.  First you run AutoCAD's configuration program

to select the ADI Printer, plot to binary file, 240x144 dots per

inch, and 13.6" horizontal paper width.  Use the PRPLOT command

in AutoCAD.  This produces a binary printer plot file called

"site-3d.prp".  Next, exit AutoCAD and plot the drawing by

invoking:

STAR9E -Fsite-3d.prp -C -D4 -W

This will cause STAR9E.EXE to make a plot of the contents of the file

"site-3d.prp" in color at 240 x 144 dpi on wide paper.  When the

plot is done, STAR9E.EXE will vanish from memory (it does not install

itself as it would if you wanted to plot directly from AutoCAD).

**Note: If you added -Osite-3d.epp to the above command line,

the data wouldn't be sent to the printer but a file named

site-3d.epp would be created.  See the above section titled

"OUTPUTTING STAR9E PRINTER FILES FOR DIRECT PRINTING FROM DISK"

for details.

NOTES ON OPERATION:  AutoCAD TIPS AND HINTS for the STAR9E ADI Driver

1.  If you will use the STAR9E ADI driver with every session of

AutoCAD, you may want to place the STAR9E.EXE command in your

AUTOEXEC.BAT file with the correct options set, because you

must install the Printer driver after every reset of your

system.

2.  The Printer must be connected, turned on, and on line before

you plot a file using the STAR9E ADI driver.

3.  Do not turn the Printer on and off in the middle of a

plotting session.  An error message will be displayed if you

try to use the ADI STAR9E driver while the Printer is turned off.

4.  If you're using the tractor feed, make sure you've advanced

the paper far enough for the paper bail contact the platen.

5.  Only Colors numbered 1 through 7 are printed, colors 8 to 15

are converted to black (#7), so, don't use colors numbered

greater than seven.

6.  You will find that high density and/or color plots take much

longer to complete than low density and/or monochrome plots.

7.  Note that the different density options use different amounts

of memory in your computer.  High density plots need a lot of

memory.  They also take a long time to complete!  Color adds

greatly to the memory and time requirements.  Memory used up

by drivers is subtracted from the memory your drawings need

when you run AutoSketch.  AutoCAD doesn't have that problem,

but there still is limited memory available for use by

drivers.  The highest density color modes use so much memory

that it isn't possible to plot directly from AutoCAD, so this

driver offers an optional file input mode.

8.  AutoCAD 386 places a limitation on real mode ADI printer plotter

drivers such as this one. A maximum of 4096 color pixels can

be printed in the X direction. If you configure this driver

for TSR operation, color, density 5 (180x360dpi) and wide

paper, you must limit the width of the plot to less than 11.3".

This limitation is not necessary with 640k DOS AutoCAD.

AutoCAD and ADI are registered trademarks of Autodesk, Inc.  IBM,

PC, XT, AT are trademarks of International Business Machines

Corporation.