CONTENTS

1.  ZMODEM Features at a Glance....................................   1

2.  TUTORIAL INTRODUCTION..........................................   3

3.  CALLING DSZ FROM COMM PROGRAMS.................................   4

3.1    USE AN INTERFACE PROGRAM?...............................   4

3.2    MSKermit 3.xx...........................................   4

3.3    PROCOMM PLUS............................................   5

3.4    Qmodem..................................................   6

3.5    TELIX...................................................   8

4.  BULLETIN BOARD CONSIDERATIONS..................................  10

5.  DESCRIPTION....................................................  11

6.  COMMANDS.......................................................  13

6.1    List of Commands........................................  13

6.2    Interrupt Keys..........................................  19

7.  NUMERIC PARAMETERS.............................................  20

8.  ENVIRONMENT VARIABLES..........................................  24

9.  EXIT STATUS....................................................  28

10.  EXAMPLES.......................................................  29

11.  BACKGROUND OPERATION...........................................  30

11.1   Microsoft Windows.......................................  31

11.2   VM......................................................  32

12.  TERMINAL SERVERS...............................................  33

12.0.1  PORT WATCHERS  34

13.  FLOW CONTROL...................................................  35

14.  ERROR MESSAGES.................................................  36

14.1   Got ZRPOS...............................................  37

14.2   Subpacket too Long......................................  38

14.3   Bad CRC.................................................  38

14.4   Garbled Data Subpacket..................................  38

14.5   Serial Input Error: Line Status Register xx ............  39

14.6   TIMEOUT.................................................  42

14.7   No Carrier Detect.......................................  42

14.8   Port %d (%X) defective..................................  42

14.9   COMMAND NOT FOUND.......................................  42

14.10  Out of Environment......................................  42

15.  SLOW TRANSFERS.................................................  44

16.  IN CASE OF DIFFICULTY..........................................  45

16.1   LOCKUPS.................................................  45

16.1.1  Insufficient Memory  45

16.1.2  Environment TOO BIG  45

16.1.3  Bad COM Port Configuration  45

16.1.4  LAN Manager Conflicts  46

16.2   Incorrect Batch Files...................................  47

16.3   Cheap Internal Modems...................................  47

16.4   Commands Entered in Upper Case..........................  47

16.5   No Carrier Detect.......................................  47

16.6   DOS 3.x.................................................  48

16.7   Perstor Controllers.....................................  48

16.8   IBM PS/2 Model 50Z......................................  48

16.9   Brain Damaged UARTS.....................................  48

16.9.1  Everex Internal Modems  51

16.10  Weird Iron..............................................  51

16.11  PKARC 3.6...............................................  51

16.12  Will the real YMODEM please stand up?...................  51

16.13  286 and 386 Extended Memory.............................  52

16.14  Disk Caches.............................................  52

17.  ASSOCIATED DOCUMENTS...........................................  53

18.  FILES..........................................................  54

19.  Other Omen Technology Products.................................  56

19.1   Professional-YAM COMMS TOOLS............................  56

19.2   ZCOMM: Industrial Strength COMMS Shareware..............  56

19.3   ZMODEM Developer's Collection...........................  56

20.  FAIR PLAY......................................................  57

20.1   Sharing DSZ Files.......................................  58

20.2   REGISTRATION............................................  59

20.3   Files on Registration Disk..............................  61

20.4   Bulletin Board Registration.............................  62

21.  CHANGES........................................................  64

22.  ACKNOWLEDGEMENTS...............................................  75

Chuck Forsberg

Omen Technology Inc

ABSTRACT

ZMODEM-90(TM) - Not just another XMODEM hack

DSZ is one of many available programs that implement the ZMODEM file

transfer protocol.  DSZ features the ZMODEM-90(TM) extensions including

ZMODEM compression and MobyTurbo(TM) accelerator.

DSZ's ZMODEM provides advanced file management features including two

levels of Intelligent Crash Recovery(TM), and flexible control of

selective file transfers.

The ZMODEM file transfer protocol provides reliable file and command

transfers with complete END-TO-END data integrity between application

programs.  DSZ's 32 bit CRC protects against errors that sneak by "error

free" modems and even the most advanced networks.

ZMODEM safeguards all data and supervisory information with effective

error detection.  (XMODEM and many recently introduced protocols do not

protect vital supervisory data.)

ZMODEM rapidly transfers files, particularly with modern modems,

timesharing systems, satellite relays, and packet switched networks.

ZMODEM's streaming operation eliminates delays caused by

Kermit/XMODEM/YMODEM/JMODEM block acknowledgements.  ZMODEM eliminates

traditional protocols' PACKET LENGTH compromises between transfer

efficiency and error recovery; ZMODEM's packet length is the entire

file.

User Friendliness is an important ZMODEM feature.  ZMODEM

AutoDownload(TM) (Automatic file Download without user intervention)

provides a level of convenience unimaginable to users of traditional

protocols.

DSZ - a ZMODEM-90(TM), True YMODEM(TM), XMODEM File Transfer Program

Omen Technology Inc

1.  ZMODEM Features at a Glance

_______________________________________________________________________

|   Feature              | ProCm | RZSZ | DSZ  | GSZ | ZCOMM | Pro-YAM |

|________________________|_______|______|______|_____|_______|_________|

|Security Verification   | no    | no   | no   | no  | no    | YES     |

|Cybnertic Data Recovery | no    | no   | YES  | YES | YES   | YES     |

|                        |       |      |      |     |       |         |

|Command Download        | no    | YES  | no   | no  | YES   | YES     |

|                        |       |      |      |     |       |         |

|Update Directory Tree   | no    | YES* | no   | no  | YES   | YES     |

| Create Directories     | no    | YES* | no   | no  | YES   | YES     |

| Newer files Only       | no    | YES  | YES  | YES | YES   | YES     |

| Protect files          | ?     | YES  | YES  | YES | YES   | YES     |

| Common files Only      | no    | YES  | YES  | YES | YES   | YES     |

|                        |       |      |      |     |       |         |

|Collision Avoidance     | ?     | no   | YES* | YES | YES   | YES     |

|                        |       |      |      |     |       |         |

|Crash Recoverey         | YES   | YES  | YES  | YES | YES   | YES     |

| Verified Accuracy      | no    | SZ   | YES  | YES | YES   | YES     |

|                        |       |      |      |     |       |         |

|ZMODEM-90(TM)           | no    | YES  | YES  | YES | YES   | YES     |

| Compression            | no    | YES  | YES  | YES | YES   | YES     |

| MobyTurbo(TM)          | no    | YES* | YES  | YES | YES   | YES     |

| 7-bit Paths OK         | no    | YES* | YES* | YES | YES   | YES     |

|                        |       |      |      |     |       |         |

|OPERATING SYSTEMS       |       |      |      |     |       |         |

| Text Translation       | no    | YES  | YES  | YES | YES   | YES     |

| Record Format Support  | no    | YES  | YES  | YES | YES   | YES     |

| DOS Shared Files       | ?     | -    | YES  | YES | no    | no      |

| VMS                    | no    | YES  | no   | no  | no    | YES     |

| Unix/Xenix             | no    | YES  | no   | no  | no    | YES     |

|                        |       |      |      |     |       |         |

|True YMODEM(TM)         | no    | YES  | YES  | YES | YES   | YES     |

|Demand Upgrade(TM)      | no    | YES  | YES  | YES | YES   | YES     |

|File Transfer Display   | YES   | no   | no   | YES | YES   | YES     |

| TOTAL File Xfer Data   | no    | YES  | YES  | YES | YES   | YES     |

|________________________|_______|______|______|_____|_______|_________|

ProCm = ProComm Plus

* Not available on all flavors or platforms

Chapter 1 Pg 2         DSZ Reference Rev Jun-7-95         Typeset 6-7-95

*************************************************************************

DSZ registration is special.

With most programs, registration only applies only to today's version.

Not DSZ!

DSZ registration bypasses the shareware announcement and unlocks

advanced features in all future versions of DSZ.  You don't have to pay

and pay again for new DSZ versions.  Omen's Demand Upgrade(TM)

technology allows you to download and activate future DSZ versions

without further registration!

*************************************************************************

Chapter 2 Pg 3         DSZ Reference Rev Jun-7-95         Typeset 6-7-95

2.  TUTORIAL INTRODUCTION

If you are just starting out with DSZ, or if you have difficulty getting

DSZ to work, please follow the procedure given below.

Do not run any TSR or other COMM program.  This includes LAN programs.

Reboot a clean DOS if necessary.

Start with the following command given directly from the DOS prompt:

dsz port 1 speed 2400 d t

Type it exactly as it appears above; do NOT use upper case.

(Change the port number and speed as necessary.  Use only COM1 or COM2

for testing.)

This connects you to the modem.  Dial a bulletin board by typing AT

commands to the modem ("ATDT621-3746 <enter>")[1] to make it dial the

BBS.  Log in as usual.  Then, request a ZMODEM download of a file you

don't already have.  The file will download automatically, you don't

have to do anything.  This procedure verifies the correct operation of

DSZ.

Next you might try calling the BBS using a higher "baud rate".  This

requires setting a higher speed with the "speed" command.  A "handshake

on" command may be necessary along with commands to the modem to enable

hardware handshaking in the CTS and RTS leads.

Perhaps you got some error messages before or during the download.  The

"ERROR MESSAGES" chapter explains all the commonly seen error messages

and suggests solutions to the underlying problems.

__________

1. If your modem has a non standard setting the ATDT621-3746 command

you type to the modem may not echo on the screen.

If your telephone line requires pulse dialing, use ATDP621-3746 (DP

instead of DT).

Chapter 3 Pg 4         DSZ Reference Rev Jun-7-95         Typeset 6-7-95

3.  CALLING DSZ FROM COMM PROGRAMS

If you need to define a trigger string for automatic downloads, make the

program look for "Control-X B 0 0" without the spaces.

It may be necessary to add the DOS command: SET COMSPEC=C:\COMMAND.COM

(change as needed) to your AUTOEXEC.BAT file to allow programs to call

DSZ.

Note: NO SPACES in the SET command!

Please note that dsz reads the hardware to get the speed.  The dsz speed

command is hardly ever needed when calling dsz from another program.

3.1  USE AN INTERFACE PROGRAM?

A number of third party programs have been developed to provide a

congenial interface to DSZ if you have enough memory to spare.  The

XFER, POE, PCALL, DSZ_ZAP, and PROZ programs have been popular in

interfacing DSZ to ProComm's editor gateway.  Many of these support GSZ

as well.  These programs are available on many bulletin boards.

Hint: the DOS PATH environment variable must include ProComm's directory

to allow POE to locate all the POE files,.  The POE files should be

stored in the ProComm directory.  Setting the "PROCOMM" environment

variable is insufficient.

Calling DSZ with these programs requires more memory than calling DSZ

directly.

3.2  MSKermit 3.xx

The "push" command (^[p) in MSKermit 3.xx sends an XOFF character to the

modem before calling DOS.  Using this to activate DSZ's term mode with a

"dsz t" command causes most remote hosts to lock up.

The remote host can be unwedged by using the dsz G command, or by

keyboarding XON (Ctrl-Q) from DSZ's terminal mode.

You can add these macros to your MSKERMIT.INI file.  Courtesy Keith

Petersen W8SDZ

define sx run dsz F port 2 G sx 1 2,define 1,define 2,

define rz run dsz F ha on port 2 G rz -r

define sz run dsz F ha bo port 2 G sz 1 2,define 1,define 2,

define t run dsz F ha on port 2 G t -r

Chapter 3 Pg 5         DSZ Reference Rev Jun-7-95         Typeset 6-7-95

3.3  PROCOMM PLUS

It's easy to add DSZ to ProComm Plus 2.X.

Datastorm has not licensed ZMODEM-90(TM) technology despite allegations

that DSZ is now "built in" to ProComm Plus version 2.  Many users of

ProComm Plus version 2.x use DSZ to get MobyTurbo(TM), Intelligent Crash

Recovery(TM), and other ZMODEM-90(TM) features.

Make sure the DSZ program is in your search path.  If you don't see DSZ

announce itself when called from ProComm, DSZ is not in your search

path.

Here are some handy settings for ProComm Plus.

First, you may need to set the DOS environment before running ProComm.

Give these commands to DOS before running ProComm Plus.  (These lines

may be added to your AUTOEXEC.BAT file.)

Note: NO SPACES in the SET command!

SET DSZPORT=2                   (Only needed for COM2)

SET DSZLOG=DSZ.LOG      (Only if you wish a log file)

Then set ProComm's external protocol strings.  From terminal mode hit

ALT-S, then select the PROTOCOL OPTIONS menu item.

From that menu select EXTERNAL PROTOCOL OPTIONS.  Then set ProComm's

external protocol strings to:

A- External protocol 1 upload filename:  ZMODEMU

B- External protocol 1 download filename: ZMODEMD

C- External protocol 2 download filename: ZMODEMMD

The following batch files are included in the DSZ zipfile:

ZMODEMU.BAT (contains:  DSZ sz %1 %2 %3 %4 %5 %6 %7 %8 %9)

ZMODEMD.BAT (contains:   DSZ rz)

ZMODEMDR.BAT (contains:   DSZ rz -r)

ZMODEMMD.BAT (contains:   DSZ rz -mr)

ZMODEMAD.BAT (contains: DSZ t)

!!!!IMPORTANT USE lower case (small letters) EXCEPT WHERE SHOWN!!!!

*******************************************************************

Select the ZMODEM PROTOCOL OPTIONS menu and turn off Auto downloading!

*******************************************************************

When sending (uploading) files with ZMODEMU.BAT give the ProComm window

the full pathname (c:/foo/bar/file.ext) for *each* file not residing in

the current directory.

Chapter 3 Pg 6         DSZ Reference Rev Jun-7-95         Typeset 6-7-95

When downloading files with one of the other batch files shown above,

Procomm will also prompt for parameters; just hit ENTER.  Or, you can

enter options such as "-mr" if they are't included in the batch file

itself.

Once you've got the hang of things the "ProComm way", try the "dsz t"

command described above to give you automatic ZMODEM-90(TM) downloads

(Look Ma, no keystrokes!!!)..

You can also set the editor hot key (ALT-A) or view program hot key

(ALT-V) by using the ALT-S "FILE/PATH OPTIONS" menu.  The strings for

these keys are long enough to store an entire DSZ command line without

need for a batch file.

COM3 and COM4 on Procomm Plus are non standard; in case of difficulty

use COM1 or COM2, or use the portx command.

DSZ may not operate properly when called from ProComm if ProComm has

files open at the time.

A number of reports have been received of problems running DSZ from

ProComm Plus because of insufficient memory.  Please refer to the

subchapter on LOCKUPS.

When running under ProComm's Host mode, failure to use the DSZ CON

command may cause ProComm to interfere with file transfers or lock up

the computer.

EXAMPLE: DSZ CON sz %1

Hint: the DOS PATH environment variable must include ProComm's directory

to allow POE to locate all the POE files,.  The POE files should be

stored in the ProComm directory.  Setting the "PROCOMM" environment

variable is insufficient.

3.4  Qmodem

Changes in the Qmodem interface (arguments passed to the batch file) may

require changes in the batch files shown below, depending on which

version of Qmodem is used.

If you need to define a trigger string for automatic downloads, make the

program look for "Control-X B 0 0" without the spaces.

If you define a protocol with 'Z' as its letter, the latest Qmodem will

detect automatic ZMODEM downloads.  Defining an external 'Z' protocol

disables Qmodem's internal ZMODEM, allowing DSZ to be called.

The following is based on a message from Mike Cody, sysop of HomePlate

Chapter 3 Pg 7         DSZ Reference Rev Jun-7-95         Typeset 6-7-95

313-385-9969 3/12/24/96 HST

Go into Qinstall and go to frame 3 Directories. Then select External

protocols #3 I think. Now you will see a definition area to tell Qmodem

the Letter,batchfile combo to use.

Now put Z in the letter definition, Zup.bat in the u/l batch zdown.bat

in the d/l batch, and Y in the batch protocol. Now save it at the menu

with f10 and S.

Then go to DOS and create the following batch files in your Qmodem

directory...

zdown.bat: DSZ port %2 rz -rr

zmdown.bat: DSZ port %2 rz -mrr ((FOR MOBYTURBO!!))

zmdown2.bat: DSZ port %2 est 0 %3 rz -mrr

zup.bat: DSZ port %2 sz %4

!!!!IMPORTANT USE lower case (small letters) EXCEPT WHERE SHOWN!!!!

These are the batch files that will be called by Qmodem.  If you did

everything right when you hit PGDN or PGUP you will see an external

protocol called Z) Zmodem in the box. For an easier time do a search for

either External.arc or maybe the string "Qmodem" in the file database as

there is a file with several external protocols and batch files

available in the Mahoney file section (EXEC-PC BBS).

The following batch files for Qmodem are included in the DSZ zipfile:

ZDOWN.BAT contains:     DSZ port %2 rz -rr

ZUP.BAT contains:       DSZ port %2 sz %4

!!!!IMPORTANT USE lower case (small letters) EXCEPT WHERE SHOWN!!!!

A variation on the receive batch file:

c:elecomqmodem\*y.com ha slow est 0 %3 rz -mrr

A variation on the send batch file:

c:elecomqmodem\*y.com ha cts est 0 %3 sz %4

If Qmodem says "unable to allocate swap space": QMODEM 4.2 will swap all

but approx. 5K to memory or disk when you shell out to DSZ. Go into

QINSTALL and make sure that /O /R Optimum Shell and Disk Shell are both

set to ON.

For older Qmodem versions (before 4.2) change the %4 in upload scripts

to %3.

Chapter 3 Pg 8         DSZ Reference Rev Jun-7-95         Typeset 6-7-95

3.5  TELIX

Telix has an internal ZMODEM, but some users prefer to use DSZ for

performance and/or reliability reasons.  Mat Fisher has published a

Telix script to call DSZ for automatic downloads.

For those of you who use DSZ with Telix, here is a sample script

file to use instead of batch files:

//////////////////////////////////////////////////////////////////

//Protocol salt file for Telix's external protocols by Matt Fisher

//Last edited (02/24/90)

//used for the DSZ ZMODEM protocol.  (for u/ling & d/ling)

//////////////////////////////////////////////////////////////////

str upload[80] = "c:\telix\DSZ port 1 ha on sz -m -rr ";

str download[80]= "c:\telix\DSZ port 1 ha on rz -b -m -rr ";

//////////////////////////////////////////////////////////////////

Main ()

{

if (_ext_filespec == "")

{ strcat(download,_down_dir); dos(download,0); }

else

{ strcat(upload,_up_dir); strcat(upload,_ext_filespec); dos(upload,0); }

alarm(1);

}

//////////////////////// THE END //////////////////////////////////

Simple but effective...

--- TAGMAIL v2.20

* Origin: CRIMP/John Galt [Node 1] (1:120/144)

INTERNET: Matt.Fisher@f144.n120.z1.FIDONET.ORG

NOTE: The DSZ command strings given above are quite unusual.  Instead of

the  "pW1  ha  off"  one  would be more likely to use "ha on" or "ha sw"

depending on the modem's flow control settings.

From : Bill Logan

Subject : Help With Gsz For Telix

I test and use a variety of terminal programs for individuals.

One of which is telix.  I found the script language to be less

than user friendly and installed GSZ via batch files and as an

external protocol.  Here are the batch files:

ZMUP.BAT

Chapter 3 Pg 9         DSZ Reference Rev Jun-7-95         Typeset 6-7-95

GSZ pV1 port %2 sz -mr %3

ZMDN.BAT

GSZ pV1 port %2 rz -mrr %3

Then go to ALT O in Telix and insert those batch names in the protocols.

They worked fine for me.

Bill

---

* Origin: PCConsultingAgencyII {=Telecom Node=} (1:300/22)

Use the Telix ALT-O screen to disable Telix's automatic ZMODEM download.

Chapter 4 Pg 10        DSZ Reference Rev Jun-7-95         Typeset 6-7-95

4.  BULLETIN BOARD CONSIDERATIONS

Note to the wise BBS operator: be sure you understand the restrict

command, how and WHY to use it!

When installing DSZ on a bulletin board or host system, be sure to check

for proper operation when the caller hangs up while a file is being sent

or received.

Some bulletin boards do not call DSZ to send a file until they have

verified the file's presence by actually opening the file.  If this

checking is done in DOS's default exclusive open mode, dsz's file

sharing will be defeated.

If using a FOSSIL driver, you may need to call DSZ thru a batchfile.

That file should call XU to disable X00 before running DSZ and restart

X00 afterwards. Failure to take this precaution may cause FOSSIL to

steal the COM port from DSZ before file transfers are completed.

Chapter 5 Pg 11        DSZ Reference Rev Jun-7-95         Typeset 6-7-95

5.  DESCRIPTION

DSZ is a "protocol driver" that uses ZMODEM-90(TM), True YMODEM(TM), or

XMODEM to transfer files over a serial port with a variety of programs.

DSZ has two main commands for ZMODEM transfers: "sz" sends file(s), and

"rz" receives them.

When calling bulletin boards to download files with ZMODEM you can avoid

the bother of starting DSZ for each transfer by DSZ's terminal emulator,

before asking the BBS for the download.

dsz t

The "t" command allows you to keyboard the file request directly to the

BBS.  (The "t" stands for "terminal emulation".) Just give the "dsz t"

command from DOS or your comms program's DOS GATEWAY.  DSZ's ZMODEM

AutoDownload then receives the file automatically, using the filename

provided by the BBS, without any wasted keystrokes or delay.  Try it,

you'll like it!  (The "t" command is described below.)

YMODEM downloads are almost as easy.  Use the "t" command as shown

above, and hit the F3 key after telling the board to download a file

with YMODEM.

ZMODEM and True YMODEM(TM) transfers preserve exact file length and

modification time.

DSZ supports XMODEM-1k (XMODEM with 1024 byte blocks, sometimes

mislabeled as ymodem) and CRC-16 with the sx -k and rc commands.

DSZ is usually called from a communications or bulletin board program.

DSZ uses its own modem I/O.  The original interrupt vector, UART parity,

and interrupt configuration are restored on exit.  The original state of

the 8259 interrupt mask bit for this vector is also restored on exit.

DSZ uses DOS standard output (stdout) for messages.  DSZ never writes

directly to the screen.  DSZ expects its standard output to point to the

console, a logging file, or NUL, but never the modem port.  If dsz's

standard output is redirected to the modem (as with ProComm's Host mode

DOS shell), the combination will lock up the computer.  When necessary,

dsz's standard output can be redirected back to the console device with

the CON command.

DSZ and GSZ automatically support Doorway 2.2 without extra commands.

The RZ.BAT and SZ.BAT files are supplied for convenience when operating

under Doorway.  RZ.BAT allows calling programs that emit an "rz" command

to automatically start DSZ from the DOS command prompt.  SZ.BAT is a

convenient command interface that complements RZ.BAT.  You may modify

Chapter 5 Pg 12        DSZ Reference Rev Jun-7-95         Typeset 6-7-95

SZ.BAT and RZ.BAT with your favorite options.

Chapter 5 Pg 13        DSZ Reference Rev Jun-7-95         Typeset 6-7-95

6.  COMMANDS

***********************************************************

NOTE WELL

Except as noted, all commands and options must be typed in lower case.

DO NOT USE UPPER CASE!!

Case is significant!

***********************************************************

Optional commands or arguments are shown in square brackets [].

When used (in any combination), the A, CON, port, speed, handshake,

estimate, and d commands must be used in exactly this order.

dsz [A] [CON] [F] [port N] [speed S] [handshake XX] [pxN... ] [z pxN...

] [D] [d] [restrict] [G] command

The shareware program DOORWAY 2.2 interfaces well with DSZ, and provides

many of the same functions, including the ability to run direct screen

writing applications remotely.  DSZ and GSZ automatically support

Doorway 2.2 without extra commands.

6.1  List of Commands

CON   Redirect message output to the DOS "CON" (console) device,

overriding any previous redirections.  This is necessary to avoid

lockups caused by ProComm host mode.  Most programs do not require

this hack.

F     Leave the 16550A UART FIFO enabled on exit.  Since DSZ has no way

to read the 16550A DMA mode and trigger level bits, these values

are not restored.

port N select port N (n = 1,2,3,4,5,6,7,8, or 9) Default is 1 (COM1) or

whatever is read from the DOS DSZPORT dos environment variable.

COM11 to COM18 refer to the COM1 to COM8 ports on the PS/2.

*****************************************************************

NOTE

Only COM1 and COM2 are standard on all PC compatible computers.

Multiple standards exist for COM3 and above, so use only COM1 or

COM2 when first installing DSZ.

*****************************************************************

DSZ COM3 uses 3E8 and IRQ4.  DSZ COM4 uses 2E8 and IRQ3.

DSZ COM7 uses 3E8 and IRQ5.  DSZ COM8 uses 2E8 and IRQ5.

If you wish to use COM3 or COM4 and your computer is not wired as

shown above, you MUST use the "portx" command to specify the

Chapter 6 Pg 14        DSZ Reference Rev Jun-7-95         Typeset 6-7-95

actual base address and interrupt vector.

EXAMPLE: to use COM2:

dsz port 2 rz

In multitasking operation, accessing a port used by another

running program will hang the machine.

portx base,irq[,divisor] To select an arbitrary port address and

interrupt vector, the portx command must be used instead of the

port command.  The portx command replaces the portcommand in the

same place on the command line.  Please refer to DSZBG.DOC

(provided on the Registration Diskette) or the Professional-YAM or

ZCOMM (ZCOMMDOC.ZIP) manuals for details on this command.

When the portx command is used, dsz will indicate COM9.

EXAMPLE: dsz portx 3e8,15 speed 19200 handshake on rz

Newer UART chips can use raw bit rate sources up to 8 MHz or more,

more than 4 times as fast as the IBM standard 1.84320 MHz.  Some

386 and 486 machines support MobyBaud(TM) operation above 115200

bps.  To support a nonstandard bit rate source, multiply 5760 by

the new raw bit rate source frequency in MHz divided by 1.84320.

EXAMPLE: dsz portx 3e8,15,57600 speed 19200 handshake on rz

supports the 18.432 MHz clock option on the Boffin Electronics

"WinSport II" 2 port serial board.  Your hardware configuration,

BIOS firmware, operating system, device drivers, and TSR programs

will determine the maximum useful speed.

speed S Set the specified speed.  This command is rarely needed because

DSZ reads the port's current speed by default.  Please refer to

the FLOW CONTROL, MAIN COMMANDS and SOFTWARE COEXISTENCE chapters

in DSZBG.ZOO or the ZCOMM or Pro-YAM manual for important

information about the speed command.

Please study all the chapters of this document when using speeds

above 2400 bps.

With standard clocking, DSZ supports a maximum speed of 115200.

Not all hardware can support that speed without errors.

handshake XX Enable one of several choices of hardware handshake.  (The

default is XOFF/XON software handshake).  This command is

documented in the ZCOMM or Pro-YAM manual, and in DSZBG.DOC.  (The

ZCOMM manual is available on all bulletin boards registered for

Chapter 6 Pg 15        DSZ Reference Rev Jun-7-95         Typeset 6-7-95

DSZ or GSZ as ZCOMMDOC.ZIP.) On DSZ the "handshake" command may be

abbreviated to "ha".

D     Disables the modem (drops Data Terminal Ready, DTR) if carrier is

lost.  This may be used to prevent spurious events (phone ringing)

from interfering with the calling bulletin board's recovery from

carrier loss.

d     Disables carrier dropout monitoring, for modems that do not

properly drive the "Carrier Detect" input on the PC.  If you see a

No Carrier Detect message, you need to give the d command.  Before

using the d command, make sure you have the proper port

selected!!!

estimate length [speed] Use the specified length and/or optional

specified effective transfer speed to estimate transfer time for

the next file to be sent or received.  The length argument is

mainly used with XMODEM downloads; YMODEM and ZMODEM normally

transfer the file length in the filename packet.  The estimate

command must come after any port, speed, or handshake commands.

The estimate command does not affect actual throughput.  It only

affects DSZ's prediction of how much time a file transfer will

take.

The following makes DSZ calculate file transfer times based on 290

character per second throughput.  Some error correcting 2400 bps

modems can attain this throughput when MobyTurbo(TM) is used.

EXAMPLE: dsz port 2 speed 9600 estimate 0 2900 rz

EXAMPLE: dsz port 2 speed 9600 estimate 0 2900 sz bigmama.zip

pxN   Set numeric parameter x to N.

DSZ has two sets of numeric parameters, a base set and a "zmodem"

set.  Parameters in the base control such things as date stamping

of files, error exit value, etc.  The "zmodem" numeric parameters

control timeouts, window sizes, and other ZMODEM, YMODEM, and

XMODEM protocol parameters.

These parameters are mostly a subset of the Pro-YAM/ZCOMM numeric

parameters, and are listed below.  They assume their defaults each

time DSZ is called.  Without an argument, p displays the numeric

parameters and their values.

Chapter 6 Pg 16        DSZ Reference Rev Jun-7-95         Typeset 6-7-95

EXAMPLE: dsz p

z pxN Set zmodem numeric parameter x to N.  All further numeric

parameter commands are taken as zmodem parameters.  The zmodem

numeric parameters correspond to the Pro-YAM or ZCOMM "zmodem"

parameters and rarely need changing.  These parameters assume

their defaults each time DSZ is called.  Without an argument, z p

displays the zmodem numeric parameters and their values.

restrict Restrict incoming pathnames (YMODEM/ZMODEM) to the current disk

and directory tree, and disallow modification or overwriting of

existing files.  This command is vital for bulletin boards

uploading files.  The restrict command causes partially received

files to be deleted.  When DSZ is restricted, it will refuse to

transfer files containing the string autoexec.bat and command.com

in upper or lower case.  This provides some defense from malicious

uploaded files.

EXAMPLE: dsz restrict rz

G     Send an XON character to the modem.  Useful when calling dsz from

MSKermit to undo the XOFF sent by MSKermit.

EXAMPLE: dsz port 2 G t

t     [-T] [-8g] [-h] [-H] Terminal function with ZMODEM AutoDownload.

The optional -T forces DSZ to exit after it has recognized a

ZMODEM AutoDownload(TM) request.

The optional -8g (available on registered copies only) enables

reception of "BBS (ANSI) graphics" displays.  (A video device

driver such as ANSI.SYS must be loaded to support some types of

graphics.)

The optional -h enables local echo of keyboarded characters ("half

duplex").  In addition, -H makes the Enter key perform a local

newline (CR/LF).

A small subset of ZCOMM/Pro-YAM term function subcommands are

recognized:

Keyboard F1 or ALT-X to exit.

F3 receives file(s) with True YMODEM(TM),

ALT-P toggles parity between the 8n default and 8g (enables ANSI

Chapter 6 Pg 17        DSZ Reference Rev Jun-7-95         Typeset 6-7-95

"graphics").

ALT-H toggles two local echo ("Half Duplex") modes; the second

displays a newline (CR/LF) when Enter is keyboarded.

Keyboarded ASCII characters are sent to the modem.

ALT-B sends a break.

Characters from the modem print on the screen.  A device driver

such as ANSI.SYS may be used to recognize escape sequences.

Standard ANSI.SYS is limited to a small subset of ANSI escape

sequences; NANSI.SYS, FANSI-Console and other replacement console

drivers provide faster screen updates and better ANSI terminal

emulation.

DSZ's ZMODEM AutoDownload(TM) detects the start of a ZMODEM file

download and accepts the file(s) without operator intervention.

rz    [-abeEmMnpPrt+YZy] [dir] [file ...]     (ZMODEM-90 receive) **

rz    [-abenprt+Yy] [dir] [file ...]          (ZMODEM receive) **

rb    [-abt+y] [dir] [file ...]     (YMODEM receive) **

rb -g [-abt+y] [dir] [file ...]     (YMODEM-g receive) **

rc    [-abt+y] file1                (XMODEM/CRC receive)

ro    [-abt+y] file1                (XMODEM OverThruster receive)

rx    [-abt+y] file1                (XMODEM receive)

rx -g [-abt+y] file1                (Qmodem-G receive) **

** Download directory specification and -g ignored w/o

registration

sz    [-abefnprtFYQS+yZ] [-w[N]]  [PREFIX=p | ONAME=n] file ...(ZMODEM

send)

sz    [-abeEfmMnpPrtFYQS+yZ] [-w[N]]  [PREFIX=p | ONAME=n] file

...(ZMODEM-90 send)

Pathnames are separated by spaces.  Wildcards are OK.

EXAMPLE: dsz port 2 sz -r /tmp/file1 /tmp/file3 *.c

The "sz" and "sb" commands exploit DOS's tree structured file

Chapter 6 Pg 18        DSZ Reference Rev Jun-7-95         Typeset 6-7-95

system.  A directory or disk specifier expands to all files in

that directory.

EXAMPLE: dsz port 2 sz -r C:srcdir

If you use PREFIX= or ONAME= with the "sb" or "sz" commands, these

pathnames must be entered in lower case unless the underlying file

system supports upper AND lower case.

EXAMPLE: dsz port 2 sz -r ONAME=newsrc.c temp.c

sb    [-ft] [PREFIX=p | ONAME=n] file ...     (YMODEM or YMODEM-g send)

sb -k [-ft] [PREFIX=p | ONAME=n] file ...     (YMODEM-1k or YMODEM-g

send)

sx    [-t] file1                    (XMODEM, XMODEM/CRC, Qmodem-G send)

sx -k [-t] file1                    (XMODEM-1k send)

v     Verify the serial number by writing a log entry (see DSZLOG).

v      1     0 bps    0 cps   0 errors     0  128 s/n 666

The letter "v" indicates a verification entry.  The first numeric

field is 1 if YMODEM-g is supported.  The last field is the serial

number.  Unregistered copies show a s/n of 0.

o     Turn off Data Terminal Ready (DTR), terminating ("offing") the

connection if the modem responds to DTR.  Useful with the "t"

command to disconnect afterwards.

EXAMPLE: dsz t o

LARG  Convert ARG to lower case, then parse arg.

The meanings of optional arguments to the file transfer commands are

described in DSZBG.DOC and the ZCOMM/Pro-YAM manual.

The s (SlugBait) option prevents a receiver from receiving a file

without accounting for it.  Attempts to do so are flagged with a Q

status in the log file.  This option can cause a transfer that

legitimately fails near the end of the file to be reported as

Chapter 6 Pg 19        DSZ Reference Rev Jun-7-95         Typeset 6-7-95

Questionable rather than ERROR.  SlugBait causes DSZ to return success

if at least one file is transferred.

Send files may be specified indirectly:

dsz sz @C:files.lst

sends the files in C:files.lst.  This provides another means of

bypassing the DOS command line limit.  The first letter must be "@" and

the third letter must be ":" to distinguish this from an ordinary

pathname.  Pathnames in the indirection file may be separated by any

combination of space(s), tab(s), CR and/or LF.  No spaces may appear

inside a pathname; "FOO     .BAR" won't work.

A list of files to send may be built up with a command like:

EXAMPLE: dir/w *.zip > files.tmp

This file must be edited to remove extraneous messages and unwanted

files.  Then you send the files with:

DSZ sz -m @c:files.tmp

6.2  Interrupt Keys

ALT-B Cancels the current ZMODEM session.  Normally this is the best way

to cancel a ZMODEM session.

ALT-F Skips the current file in ZMODEM transfers.

Ctrl-Break Terminates the transfer in progress.

ALT-N Abruptly terminates (NUKES) the transfer in progress.

Ctrl-X If the local program is in its terminal function, typing a dozen

Ctrl-X characters to the remote ZMODEM program terminates ZMODEM

operations with most programs.  (Some machines do not allow ZMODEM

programs to check for this.)

Chapter 7 Pg 20        DSZ Reference Rev Jun-7-95         Typeset 6-7-95

7.  NUMERIC PARAMETERS

DSZ uses the following main numeric parameters, settable with the pxN

command.

EXAMPLE: dsz pQ0 rz -rr

**************************************************************************

NOTE: Parameter setting commands such as "pd0" cannot be added to

DSZOPT, they must be included in the command line.

**************************************************************************

In addition, DSZ uses the Pro-YAM/ZCOMM zmodem numeric parameters

described in the Pro-YAM/ZCOMM reference manual (ZCOMMDOC.ZIP) and in

DSZBG.DOC.

B   Set the size of buffer used for disk reads and writes.  The default

is 1024, 4096 maximum (16384 max on GSZ and DSZ.EXE).  If your copy

of DSZ is running on a floppy disk system or a very slow turkey hard

disk, a larger value may give better results.  Too large a value may

exhaust memory, induce receiver timeouts, cause disconnects(!), or

generate spurious CRC errors.  If performance logging doesn't work,

this parameter is too big.

EXAMPLE: dsz pB4096 sz A:floppy.fil

Setting the buffer size may lock out the performance log, DSZFILES,

and/or indirect pathname list in DSZ.COM.

IF YOU AREN'T BLOODY SURE YOU NEED A BIGGER BUFFER, DON'T USE ONE.

Please understand that the B parameter has nothing to do with

anything you've heard about 128, 1024 or 8192 byte XMODEM style

blocks.  Normally, ZMODEM's block size is the entire file!

G   This parameter controls the number of bytes loaded into the 16550A

transmit FIFO (First In First Out) storage register on each

interrupt.

For maximum efficiency, DSZ fills the 16 byte transmit FIFO when the

UART generates a FIFO Empty interrupt.  Once loaded, data

transmission from the transmit FIFO cannot be stopped, so many bytes

may be sent after the modem sends XOFF or asserts hard flow control

restraint.  Good engineering practice requires that modems accept a

large number of bytes after asserting flow control restraint, but

some modems and operating systems won't.

This parameter is effective only with PCDOS machines using FIFO

buffered UART devices.  To optimize dsz's responsiveness to flow

Chapter 7 Pg 21        DSZ Reference Rev Jun-7-95         Typeset 6-7-95

control restraint, set this parameter to 0.

EXAMPLE: pG0

O   This parameter controls XMODEM OverThruster(TM).  It is described in

DSZBG.DOC.

P   After a "speed" command, if you give a p command (display

parameters), the P parameter value will indicate the type of serial

I/O UART chip detected.  DSZ automatically detects and enables the

FIFO on the NS16550A, NS16550AFN, Goldstar GM82C550, and Intel 82510

chips.

EXAMPLE: dsz port 1 speed 19200 p

0  Indicates a standard 8250 or 16450 UART with no special features.

1  Indicates an NS16550AN or NS16550AFN with 16 deep receive FIFO.

These are the preferred chips.

2  Indicates an Intel 82510 with 3 deep receive FIFO.

3  Indicates a Startech ST16C650 UART with 32 byte FIFOs and on chip

hardware flow control support.

The sequence "port 1 pP1 speed 19200" bypasses DSZ's verification

checks normally performed on the FIFO configuration and may allow

defective FIFO logic on certain chips to be used.  This incantation

is not required for the NS16550AN or NS16550AFN.

The sequence "pP-1 speed 2400" should be used with the Western

Digital WD16550 chip to disable the brain damaged FIFO on that chip.

The WD16550 FIFO has been observed to lose data when transmitting at

2400 bps and slower speeds.  The minimum speed for normal transmit

FIFO operation with the WD16C550 may vary with different computers.

Q   Error code returned to DOS (default 1).  (Success always returns 0.)

Some programs toss cookies when confronted with certain exit codes.

EXAMPLE: dsz pQ-1 sz foo.bar

DSZ reports an error if no files have been transferred, even if the

command used wasn't supposed to transfer any file(s).

R   If set to 1, causes partially received files to be deleted.

Chapter 7 Pg 22        DSZ Reference Rev Jun-7-95         Typeset 6-7-95

EXAMPLE: DSZ pR1 rz

S   Timeout in seconds (default 15) waiting for restraint release (XON

and/or handshake).

V   (DSZ.COM and GSZ only) Set this to 1 to use direct to screen video.

This speeds up displays but may cause serial input errors when CGA

equipped computers are used at high speeds.

EXAMPLE: GSZ pV1 rz

sets direct video output.

W   Wait for Enter key before exiting after an error.  Useful when

calling DSZ from DESQview, allows the window to close unless there

is an error message that should remain visible.

EXAMPLE: dsz pW1 sz foo.bar

Y   Control byte for the NS16550AN/NS16550AFN FCR register.  If you

don't know what the FCR does, don't mess with this parameter.  It

has been suggested that a different value (129) may improve

multitasking under DESQview when multiple copies of DSZ are active.

a   Calibrates certain protocol timeouts used with XMODEM and YMODEM

transfers (Cybernetic Data Recovery(TM)).  This parameter does not

affect ZMODEM transfers.

The a numeric parameter is automatically set to 2000 for a 4.77 mHz

PC or XT, 4000 for a PS2/30, 6000 for an 8 mHz 1ws AT, and 12000 for

a PS2/80.  If your computer has a speedup card or otherwise runs

unusually fast (or slow!) for its type, you may set the a numeric

parameter to a value that reflects its speed.

EXAMPLE: dsz port 2 pa4000 rb

NOTE: Setting this parameter does not suppress the "Unknown Machine

Type" message which is displayed before the command line is scanned.

d   A non zero value (the default) causes incoming files received with

ZMODEM and full YMODEM transfers to have their Date set to that sent

in the file header.  ZMODEM's file management features depend on the

transmission of each file's modification date.  Files will have the

same creation date, even if the two machines are in different time

zones.  When transferring files to/from a Unix system, creation

dates are interpreted according to GMT or Universal Coordinated

Time.  File dating may be disabled with a pd0 command, which causes

Chapter 7 Pg 23        DSZ Reference Rev Jun-7-95         Typeset 6-7-95

files to assume the date and time they were received.

NOTE: Backdating file creation times may confuse some backup

programs.

v   Verbose level (debugging output) Negative values inhibit some

routine messages.  Setting v to 1 or greater causes serialized

copies to ring the bell when a transfer is done.

EXAMPLE: dsz port 2 pv1 rz

z   Timezone in minutes behind GMT, used for ZMODEM and True YMODEM file

transfers to coordinate the actual date of the file with computers

in other time zones.  PDT or MST corresponds to 420.  This may be

preset with the ZONE DOS environment variable.

Chapter 8 Pg 24        DSZ Reference Rev Jun-7-95         Typeset 6-7-95

8.  ENVIRONMENT VARIABLES

The following DOS environment variables may be set to modify DSZ

operation.  DOS environment variables are set by giving a command to DOS

before running DSZ.  You can add these commands to your autoexec.bat

file.

EXAMPLE: set DSZPORT=2

The following form may be used for arbitrary address/vector

combinations:

EXAMPLE: set DSZPORT=3e8,4

These DOS "set" commands may be placed in your AUTOEXEC.BAT file or

given just before you run your communications program.

IMPORTANT! Do not add spaces to these commands!!

set DSZPORT  =  2 WILL NOT WORK!

DIRRX (Registered copies only.) Specifies the directory to receive

(download) files.  If the sting in DIRRX ends with "/", directory

and/or disk designators in the incoming pathname are stripped, and

only the stem (name and extension) is used.  Otherwise, if an

incoming file has an absolute pathname, the incoming pathname is

used.

EXAMPLE: set DIRRX=C:/modem/download/dsz/

dsz rz -r

downloads files to the /modem/download/dsz directory on the C:

drive, with ZMODEM Crash Recovery..

EXAMPLE: set DIRRX=B:/

The DOS "set" command defining the receive directory may be placed

in your AUTOEXEC.BAT file.  Make certain there is enough environment

space to hold this value.  Confirm this by typing "set" to the DOS

prompt and verifying the DIRRX value is listed correctly.

DSZFILES May contain a list of up to 9999 pathnames to transmit, with

entries separated by spaces.  If present, DSZFILES is used in place

of a normal command line list of files to send with the sb and sz

commands.

EXAMPLE: set DSZFILES=file1 file2 file3 *.c

dsz port 2 handshake on sz -rr

Chapter 8 Pg 25        DSZ Reference Rev Jun-7-95         Typeset 6-7-95

In practice, this string would be built up by a calling program and

passed to DSZ with a "spawnvpe", "execvpe", or similar subroutine

call.

SEE ALSO: Pathname list specification @C:files.lst

DSZLOG log file to receive entries for files sent and received.

(Default null).  Sample entries:

Z  46532 38400 bps 3324 cps   0 errors    66 1024 DSZ.COM 1177

Z 124087 19200 bps 1880 cps   0 errors     6 1024 MXY.TMP 1177

Transfer times are based on the DOS time of day clock and have up to

two seconds' uncertainty.  As with all protocol throughput reports

generated by Omen Technology programs (and contrary to the false

claims of some critics), DSZ's throughput figures for ZMODEM,

Kermit, and True YMODEM(TM) transfers do not take "credit" for

pathname and/or protocol overhead characters.  DSZ's throughput

reports do not include the time required to send the pathname.  This

understates the true time required to transfer files, but the time

required to send the pathname often depends on the alacrity of the

operator, and thus does not always reflect the performance of the

protocol itself.

Transfers measuring less than one second are reported as having 9999

characters per second throughput.

The unlabeled numeric field after the error (retry) count is the

number of flow control stoppages encountered sending the file.  Flow

control restrictions are normal when the interface speed exceeds the

communications speed (DTE > DCE).  This number gives an indication

of flow control restrictions (overloaded networks, etc.) encountered

during the transfer.

The last numeric field before the file name is the YMODEM block size

or ZMODEM subpacket length used for the last block in the file.

The last field is the serial number of the other program, or -1 if

this information is not available.  When sending files, the

receiving program's serial number is not normally available unless

the -S option is used.

The code letter indicates the type of transfer, Z (ZMODEM), S

(X/YMODEM), R (X/YMODEM), E (error), or L (transfer terminated by

loss of data carrier).

WARNING: If DSZ encounters a loss of carrier or other fatal error

condition before a file COMMENCES transmission, a log entry may not

be made.

Chapter 8 Pg 26        DSZ Reference Rev Jun-7-95         Typeset 6-7-95

EXAMPLE: set DSZLOG=C:/yam/dszlog

IMPORTANT! NO SPACES!!

DSZOPT (Registered copies only) String containing default options for

DSZ file transfers.  Legal option letters for DSZOPT are the

optional modifiers to the DSZ sz, rz, sx, rx, sb, rb commands.

A T may be appended to the string to force DSZ to exit after it has

recognized a ZMODEM AutoDownload(TM) request.

An 8g may be appended to the string to make BBS (ANSI) graphics the

default for DSZ's t command.  The ability to preset these options is

especially helpful for controlling ZMODEM automatic downloads from

DSZ's t command.

EXAMPLE: set DSZOPT=r

Enables DSZ's Intelligent Crash Recovery(TM) as a default.

Let's say you wish ZMODEM crash recovery, and BBS "graphics" for the

DSZ term function:

set DSZOPT=r8g

If you're downloading files from GEnie with an MNP modem, the

following will speed up your downloads with Genie's ZMODEM-90Tm.

(You can recognize ZMODEM-90 on GEnie two ways: 1) MobyTurboTm is

used to speed ZIP and ARC downloads, 2) "sz 4.xx" is displayed at

the end of transfer.  As of this writing, ZMODEM-90 may not be

available in all libraries.) Note that (as usual with DSZ) case is

significant: the "r" must be lower case, the "W" upper case.

set DSZOPT=rW16384

WARNING: Do NOT include "ena" or any other commands in the DSZOPT

string.  With the exception of the "8g" mode, only **options** may

be placed in the DSZOPT string!  (Options are the letters that

modify the sz, rz, sb, etc. commands.  In "rz -r", "r" is an

option.)

Parameters can not be set in DSZOPT.

MAXBYTES Maximum number of bytes allowed for transfer.  If this number

is exceeded, DSZ skips the offending files (ZMODEM) or aborts

remaining files in the batch (YMODEM).

DSZPORT Default communications port override, default 1.

EXAMPLE: set DSZPORT=2

Chapter 8 Pg 27        DSZ Reference Rev Jun-7-95         Typeset 6-7-95

IMPORTANT! NO SPACES!!

ZONE sets the z numeric parameter to the number of minutes the current

local time lags GMT time.  PDT or MST corresponds to 420.  PST

corresponds to 480.

EXAMPLE:

set ZONE=420

Chapter 9 Pg 28        DSZ Reference Rev Jun-7-95         Typeset 6-7-95

9.  EXIT STATUS

DSZ exits with 0 status for successful transfers or the value of the Q

numeric parameter (default 1) if a file transfer failed, or if no files

were transferred.  Many languages and "door" programs do not return this

information to the calling program.  Some languages and "door" programs

freak out (hang) on certain non-zero exit values.

A batch file that calls DSZ directly may be used to detect the DSZ exit

status with the DOS "errorlevel" construct.  Please refer to the Q

numeric parameter below for more information.

DSZ's log file entries may also be examined to determine the success or

failure of a file transfer.

DSZ commands are an improper subset of Pro-YAM/PowerCom/ZCOMM commands.

The file transfer commands are fully described in ZCOMMDOC.ZIP, the

ZCOMM User's Manual.  (ZCOMM is a full featured shareware communications

program with autodialing, autolearn scripting, and terminal emulation.)

Portions of the ZCOMM User's Manual are also provided in DSZBG.ZOO.  You

may refer to one of those documents for details on the following

commands.

Chapter 10 Pg 29       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

10.  EXAMPLES

dsz sz c:foo.zip              (send 1 file with ZMODEM)

dsz sz -m c:*.zip             (send all .ZIP files with MobyTurbo(TM))

dsz sz f1.c f2.c f3.c         (send 3 files with ZMODEM)

dsz sz c:foo.* b:*.c          (send multiple files with ZMODEM)

dsz sz -Z *.lst               (send text files with ZMODEM compression)

dsz sz -r bigone.zip          (CRASH RECOVERY: resume interrupted

transfer)

dsz rz -r                     (CRASH RECOVERY: resume interrupted

transfer)

dsz rz -rr                    (CRASH RECOVERY with file comparison)

dsz handshake both sz yamdemo.zip       (Hardware+Software handshake)

dsz sz -n *.*                 (send files that are newer or rx has no

copy)

dsz sz -Yn *.*                (Only send files that are newer than

receiver's copy)

(Do not send files that do not exist on

receiver's disk)

dsz rz C:/UPLOADS             (receive files to specified directory *)

dsz port 2 speed 38400 handshake both restrict rz

(Restrict provides protection for BBS's)

dsz rz  -y prog.zip           (ZMODEM, single file pathname override)

dsz pR1 rx foo.dat            (XMODEM receive, remove file if error)

dsz pO60 ro foo.dat           (XMODEM/OverThruster receive, GEnie MNP *)

DSZ LRC L-Y NEWPROG.ZIP       (generate lower case "rc -y" command)

dsz sz foo >debug.dmp         (Debugging data dump)

dsz sz @C:/bbs/files.lst      (Indirect file list)

dsz sz -u *.lst               (Unlink (Delete) files as they are sent)

* Registered copies only

Chapter 10 Pg 30       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

11.  BACKGROUND OPERATION

Three DESQview description files are provided to demonstrate background

operation with modest (66 kb window) memory requirements.  DSZ uses

tight C/ASM code, table driven CRC generators, and buffered receive and

transmit for good performance in the background without hogging CPU

cycles.  On a QIC Labs 8 mHz PC-AT clone, DSZ can download files in the

background (3 background clock ticks, 9 foreground clock ticks) with a

throughput better than 9600 bps.

GSZ operates under DESQview with 128k of memory.

HINT: make sure "Separate Shift State" is set to "N".

2S   Prompts for file name(s) and sends them with ZMODEM protocol.

File: 2S-PIF.DVP

2T   Talks to the modem and allows manual YMODEM (F3) or automatic

ZMODEM downloads.  File: 2T-PIF.DVP

2A   Like 2T, but using Quarterdeck's dvansi.com memory resident ANSI

CRT device driver to process ANSI screen codes.  File: 2A-PIF.DVP

DS.BAT is the batch file used by 2A.

These files are written for a port 2 local connection and \TMP

directory, but may be modified to suit your requirements using the

DESQview CP command.  The W numeric allows DESQview to close the window

after DSZ finishes, providing a pleasant user interface.

Other DVPs, Scripts, and Batch files are available on the DESQView

conference on the Sound of Music as DVDSZ20Z.ARC.

Please consult the DESQview documentation for instructions on

installation procedure for the DVP files.

Operation at high speed (above 9600 bps) may require the DESQview

"Optimize communications" performance Advanced Setup option.

Newer versions of DESQview may require more "common memory" allocation

(6-8 k bytes) to operate with DSZ.  Some versions of DESQview do not

operate properly with DSZ; DESQview 2.0 is known to work, and DESQview

2.25 is reported to have fixed bugs in intermediate versions.

On 286 and 386 machines, some DESQview operations involve swapping

memory contents with buffers in extended memory.  As with extended

memory ramdisks and disk caches, this causes errors with high speed

communications.  The considerations discussed in the "286 and 386

Extended Memory" subchapter apply here.  Since this DESQview extended

memory swapping is asynchronous with respect to DSZ's file transfers,

"handshake slow" doesn't help here.  The solution is to force DESQview's

memory swaps to use smaller blocks (with interrupts allowed between

Chapter 11 Pg 31       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

blocks) and/or use NS16550AN or NS16550AFN or Goldstar GM82C550 chips

for the serial interface.

If you see funny characters on the screen while a program is loading,

you have a conflict between the display adapter and QEMM memory manager

which may cause lockups.  If so you must explicitly exclude the display

adapter's memory with an EXCLUDE clause in QEMM's config.sys entry.

***********************************************************

WARNING

When running DSZ under a multi tasking system such as DESQview, do not

allow two programs to access the same comm port at the same time.  At

best, such actions will disrupt communications.  At worst, your computer

will attempt to open a subspace channel to V'ger.

Tell DESQview to use the SPECIFIC port number you are using, and not "Y"

to the question: "Uses serial Ports (Y, N, 1, 2)"

When running a comm program under DESQview or any related multitasking

executive, DSZ must be called directly from the comm program, in the

same window.  DO NOT open a separate window for DSZ unless you were

about to power down the computer anyway.

***********************************************************

With some communications programs, you can make the call, log in, and

then exit (closing the window) without disconnecting the modem.  At this

time you can fire up DSZ with the "t" command to allow YMODEM or ZMODEM

downloads in a small window.

Example File: 2A-PIF.DVP

Alternatively, you can start DSZ from the DOS prompt, then type "AT"

commands to the modem to call the bulletin board.  DESQview's "learn"

facility may be helpful here.

11.1  Microsoft Windows

Communications programs do not operate well under Windows 3 because

Windows does not support true multitasking.  The severity of this

interference depends on the version of Windows and what other programs

are active or loading.

First, uprade to Windows 3.11 or use one of the aftermarket replacement

for the windows comm driver.

You MUST use a 16550A FIFO UART if you are using windows, even if  runs

just fine under DOS with a plain UART.

Set  and the modem to use hardware flow control.

Add the following lines to your system.ini file in the [386enh] section,

where n is replaced by the COMn digit:

Chapter 11 Pg 32       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

MinTimeslice=20

WinTimeslice=100,50

COMnFIFO=1

COMnBuffer=4096

For best results when running  in the MS-DOS shell, try the "Exclusive"

and "Background" items in the "Settings" menu.  Also, bring up the

"Control Panel" "386 Enhanced" menu, and set the "Minimum Timeslice" to

a smaller values such as 2.

OS/2 and Chicago provide better multitasking than Windows 3.1.

11.2  VM

According to Marshall Sutherland, Be sure to set IOPL3 mode to YES.

Chapter 12 Pg 33       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

12.  TERMINAL SERVERS

Often users must use terminal servers or multiplexers to access Unix,

VMS, and other timesharing systems.  Terminals on some timesharing

systems are connected indirectly to the host with TERMINAL SERVERS

operating over networks.  Terminal servers often interfere with file

transfers by "eating" control characters and/or losing data.

Accessing a distant system using telnet on a local Unix system entails

the same problems.  If telnet does not work well, try rlogin with the -8

option.  Set the escape character to ^P (control P) which is protected

by ZMODEM.

When terminal servers interfere with file transfers, server commands are

often available to help the problem.  When possible, enable the server

and modems for hardware flow control in and out.  Set the terminal

server to telnet transparent mode.  Some typical commands are "SET

TERMINAL DOWNLOAD", "set session passall", and "SET TERMINAL NO ESC",

but there is no standard set of commands suitable for all terminal

servers.  Normally these commands are given before attempting file

transfers, but one user has reported that the command must be given

after the file transfer is started(!).  Until terminal server vendors

improve the quality of their product, you may have to get on their case

until they adequately support high performance file transfers.

Escaping control characters with ZMODEM (sz -e) may help if the transfer

gets stuck in the same place on every attempt.  Professional-YAM and

ZCOMM allow selection of which control characters need to be escaped.

In extreme cases 7-bit transmission may be required (see the

Professional-YAM/ZCOMM manual).  If you encounter problems with control

characters you should identify which control characters are causing the

problem.  Please refer to the sz -T command in sz.doc for more help.

Flow control between the server and modem is often defective.  This

rarely causes a problem in interactive applications whose short bursts

(a screenfull at most) fit within the available memory buffers.

Streaming protocols such as YMODEM-g, long packet SuperKermit, and

ZMODEM can overload the available buffering.  Some terminal servers

support commands to enable hardware flow control such as "TERM

FLOWCONTROL HARDWARE IN OUT"; this assumes the modem attached to the

server is properly configured.

If the terminal server is too brain damaged to support flow control,

ZMODEM lets you fake it by setting a window size.  Try

sz -w2048 file...

to enable software flow control.  Experiment with different window sizes

for best results.

Terminal servers are designed to accept keyboard input from human

operators.  They may lose data when a program sends more data to the

host faster than humans type.  Some servers allow larger input buffers

Chapter 12 Pg 34       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

to support file uploads.  If you can upload short files (less than 100

bytes) but not longer files, try

sz -w1024 file...

with your uploading program.  (Not all programs support this option; use

ZCOMM or Professional-YAM.)

When both flow control and character transparency are problems, use

sz -ew1024 file...

as a starting point.

Sometimes the terminal server software is defective and must be

corrected.  Please contact the network's vendor for corrective

procedures.  An alternative is to install direct serial ports for users

desiring to upload files.

We have also encountered problems when networking software fails to pass

data after a large amount of data has been sent to the host.  This has

happened on a PPP internet connection which prevented uploads of large

amounts of data via either FTP or a ZMODEM upload (via TELNET).  The PPP

connection grinds to a standstill and not even PING works.  The same

ZMODEM/TELNET combination easily uploaded ten times as much to a nearby

machine connected by Ethernet.

12.0.1  PORT WATCHERS

Some systems enforce a timeout which disconnects a user after several

minutes of keyboard inactivity.  Long, error free ZMODEM transfers do

not generate keyboard activity (this is why ZMODEM is so efficient).

Restricting the window size with

sz -w16384 file...

option generates terminal input at regular intervals even when no error

correction is needed.  If problems persist, try setting a packet length.

sz -l1024 file...

(The default ZMODEM packet length is the file length.)

Chapter 13 Pg 35       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

13.  FLOW CONTROL

If you plan to send files over Telenet (PC-Pursuit), you must refer to

the Pro-YAM or ZCOMM Reference manual (in ZCOMMDOC.ZIP) FLOW CONTROL

chapter for vital information on flow control considerations necessary

for proper operation of ZMODEM and other streaming protocols.  (ZCOMM is

a full featured shareware communications program with autodialing,

autolearn scripting, and terminal emulation.)

Chapter 14 Pg 36       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

14.  ERROR MESSAGES

COM2 1200 bps Carrier Detect enabled Handshake slow

Command line = (commands)

This is a normal message displayed to help resolve problems with

incorrect batch files.  The port number, speed, carrier detect

("enabled" or "DISABLED") and "handshake" indicate the selected mode of

operation.

DSZ FINISHED! exit code = 0

This is DSZ's normal exit when at least one file has been transferred

and no unrecoverable errors have been detected.  If an error such as a

failed file transfer had been detected, the exit code would be 1, or the

value set in the Q numeric parameter.

As the message implies, DSZ has done all of its processing, and has

return the communications port and vector to its original value before

displaying this message.  If the computer locks up after this message is

displayed, there's something wrong with the program that called DSZ, not

with DSZ itself.

Unknown Machine Type XX, 'a' parameter preset to 6000 indicates a ROM

BIOS that does not follow the IBM specification for identifying the

machine type.  If your computer causes DSZ to display this message, you

may set the a numeric parameter described elsewhere in this document.

This message is generated before any commands are read, so do not be

alarmed when you "discover" that setting the a parameter doesn't make

this message go away.  The a parameter does not affect ZMODEM transfers.

Please send us a postcard or letter indicating the reported hex value

for machine Type, name of the computer, and its speed so we can include

that machine type in a future release.

Need more memory to run DSZ or Abnormal program termination is displayed

by the TurboC startup routine if DSZ is run in too small a memory

partition.  If you see this message, use a larger memory partition for

DSZ or remove some memory resident programs.  This message may also

appear if the calling program fails to free memory before calling DSZ.

No Carrier Detect is displayed when DSZ recognizes a lack of carrier

detect signal from the modem.  Most support calls concerning "No Carrier

Detect" are caused by the user failing to tell DSZ to use the correct

port.  DSZ does not magically divine the COM port number; if the default

COM1 is incorrect, DSZ must be told what port to use with the port

command or the DPORT environment variable.

The No Carrier Detect message may also be caused by a line disconnect,

defective cable, or incorrect modem strapping.  Sometimes this is caused

by setting the B parameter to too large a value.

The No Carrier Detect message may be disabled with the d command.  The d

Chapter 14 Pg 37       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

command must be given after any port or speed commands.

Garbage Count Exceeded Indicates ZMODEM has not detected a valid start

of header in a reasonable number of characters.

If DSZ becomes "stuck" on emitting this messages, it is possible the

sending program may not be able to recognize and act upon transmission

requests (ZROPS headers).  The sending program should be corrected.  In

the meantime, setting a ZMODEM packet length with the "zmodem pl4096"

("l" is the small letter l) command will force the sender to wait for

the receivers' progress reports.

Permission denied comes from the Microsoft C Library.  This is caused by

attempts to modify a read only file or directory, or to create a file in

a nonexistent directory.

Appearance of XOFF or FLOW during ZMODEM or YMODEM-g file sending

signifies DSZ is responding to flow control restraint.  This is normal

operation when error correcting modems or packet switched networks are

used.  ZMODEM's robustness and reliability allow creative users to

transfer files over unusual multiple hop computer lash-ups, causing

modem data lights and computer status displays to dance in unfamiliar

rhythms.

Output FLow Control Restraint RELEASED Indcates DSZ has not received a

positive flow control signal (XON and/or CTS as specified by the

handshake command) within the timeout specified by the S numeric

parameter.  This message may appear as the result of line noise.  If it

appears often, it may indicate improper flow control settings, defective

modem and/or cable, or (rarely) too small a value for the S numeric

parameter.  A TSR program that monitors ring detect has also caused this

problem.

Not allowed to overwrite file.ext By default, DSZ will not overwrite an

existing file.  Use the -y or -n or -r option to overwrite/update

existing files.

14.1  Got ZRPOS

Got ZRPOS indicates the receiving program has detected a transmission

error and has requested retransmission.  In other words, ZMODEM is doing

its job of detecting and correcting missing and garbled data.

If this happens with error correcting modems, or more often than

observed line noise would suggest, the receiver's error messages must be

studied.  "Line Status Register 01" messages indicate a flow control

problem.  "Line Status Register 02" messages suggest a receiver problem

with interrupt latency.

Otherwise, the sender may have a flow control problem in one of four

Chapter 14 Pg 38       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

areas:

+ Between the sending program and its modem.  If using an interface

speed higher than the transmission speed, or modem to modem error

correction (MNP, LAPB, PEP, etc.), check that the modem and the

sending program are configured for the same flow control methods.

Omen Technology's software is faster than most other programs, and

may expose flow control problems not seen with slower software.

+ Between the sending modem and the network.  Correcting this requires

"handshake both" to allow the sending program to honor both hardware

(CTS) flow control from the modem and software flow control

(XON/XOFF) from the network.

+ Between parts of the network, or between the host computer and its

terminal server(s).  We have received a number of reports of

problems with Cisco terminal servers.

+ Between the receiving modem and the receiving program.  This can

happen at high transmission speeds when the receiver has specified a

large I/O buffer size with the B numeric parameter.

Excessive interrupt latency from extended memory ramdisks or disk

caches, and poorly written hard disk drivers also wreak havoc on

streaming protocols but do not affect start-stop protocols (XMODEM,

etc.).  Problems with extended memory disk caches may cause errors

at regular intervals; check the receiver's error messages.

To work around these problems, give the "handshake slow" command to

the receiver.

14.2  Subpacket too Long

This error is often caused by flow control problems at the sender (see

above).

14.3  Bad CRC

This error is often caused by flow control problems at the sender (see

above).  If an odd Length is routinely reported, the cause is almost

certainly incorrectly configured flow control at the sending end.

14.4  Garbled Data Subpacket

This error may be caused by flow control problems at the sender (see

above).

Byte NNNN retry M: <error message> These messages appear when the

Chapter 14 Pg 39       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

protocol has detected an error and is attempting to retransmit the

garbled data.  These are normal messages.

14.5  Serial Input Error: Line Status Register xx

Serial Input Error: Line Status Register HH indicates a hardware

problem, slow device driver, TSR program, or other software is disabling

interrupts too long for the program to accept incoming data, and some

characters are lost.  A slow response to flow control also triggers this

message.

The value of the line status register is displayed in HEX.  The

following error conditions are OR'ed into the Line Status Register.

01   The Interrupt level circular buffer has been overrun.  This

indicates a flow control problem.  The remote did not respond to

DSZ's XOFF or hardware flow control signal, and continued to send

characters after being told not to.  This problem is alleviated by

matching flow control between the sending computer, modems, and the

receiving computer.  Usually, the mismatch exists between the

sending computer and its modem.

02   An 02 value (or any even number) for the LSR indicates Data

Overrun.  "Data Overrun" means the computer was not able to respond

to an incoming character from the UART (modem) in time to make room

for the next incoming character.  Data Overruns are caused by

poorly designed software and/or ROM BIOS "firmware" locking out

interrupts for excessive periods.  These lockouts increase the

interrupt latency, the amount of time which transpires between the

arrival of an interrupt from the modem and transfer of control to

the interrupt service routines that read the modem data to memory.

These lockouts prevent communications routines from responding to

the incoming characters in time to prevent data loss.

Other chapters in this document give suggestions for working around

excessive interrupt latency by using the NS16550AN or other

buffered UART chips.  Please refer to the "Brain Damaged UARTS"

subchapter.

Please try all the suggestions mentioned here before contacting

Omen Technology about data overruns.

Common causes of excessively long interrupt lockouts ("critical

code regions") are mouse drivers, disk drivers (BIOS), EIDE disk

block transfers, window managers (Windows 3 is terrible), disk

cache programs including Microsoft SMARTDRV, TSR programs, and

EGA/VGA board auto-select.

Chapter 14 Pg 40       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

Slowing the CPU to allow games and other time dependent programs to

be used can generate excessive interrupt latency.

Some BIOS routines are written without regard to their effect on

high speed communications.  Better versions of the offending BIOS

ROMS may be made available if enough users complain about poor

performance.  In some cases disabling "multiple block transfer" in

the BIOS improves communications.

Interrupt latency caused by EIDE block transfers can be reduced by

disabling EIDE block transfers.

Interrupt latency caused by EGA/VGA board auto-select can be often

corrected by disabling the board's Auto-Select.

Device drivers and TSRs loaded in high memory sometimes cause

interrupt latency problems that can be corrected by moving them

into low memory.

Poorly written disk caches and disk drivers cause interrupt latency

that affects protocols.  Some disk cache programs can be set to

cache disk reads only, avoiding the massive delayed writes that

interfere with file downloads.  With Microsoft SMARTDRV.EXE, try

using the /E option.

Mouse drivers have been reported to cause loss of serial data.  If

mouse movements interfere with file transfers, do not move the

mouse when transferrng files.

To correct Data Overruns, you must correct the offending

condition(s) to allow the communications program to operate

properly.

The handshake slow command can be used to stop reception during

disk transfers.  However, handshake slow is not always an effective

alternative to correcting the real problem.  When using the

handshake slow command, experiment with locking the modem to

different speeds.  Sometimes a higher speed actually works better.

The zmodem k numeric parameter can be set to flush the SMARTDRV

cache under controlled conditions.  This may be used to reduce

errors caused by SMARTDRV.  When used with SMARTDEV, set the zmodem

k numeric parameter to the number of kilobytes the should be

received before flushing SMARTDRV.  This number should be low

enough to flush SMARTDRV before SMARTDRV periodically flushes

itself.

Downloading files temporarily to a RAMDISK may help.  Files on the

RAMDISK must be copied to a real disk before they are safely

stored.

Chapter 14 Pg 41       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

A CASE STUDY: NEC UltraLite VERSA The NEC UltraLite VERSA provides

a case study of how brain damaged hardware and software interferes

with high speed communications, and what can be done about it.  A

NEC UltraLite VERSA with 8 MB of memory contains about 70 million

transistors and costs upwards of $5000.  Unfortunately this machine

lacks the hundred transistors necessary to buffer input on its

built-in serial port.  This port is built in, so replacing it with

a better UART ship is out of the question.  In normal operation

interrupt latency caused by the NEC's device drivers and DOS cause

errors at 38400 bps and above.

Setting the zmodem k parameter to 50 or 100 reduced overruns by

stopping transmission briefly before disk writes.

The best operation was obtained by hitting the F5 key while DOS 6

loads to bypass all device drivers and TSR programs.  Operating

this way there were no data overruns (LSR 02 errors) at 38400 bps.

Operation at 115200 bps was possible by setting a large I/O buffer

and using the zmodem k numeric parameter.  I was able to download

AM.DAT (a 15762427 byte FCC database of North American AM broadcast

stations provided on the "Ham Call" CDROM) with 3471 cps throughput

using ZMODEM compression.  While nowhere near as compressible as

Columbia University's Kermit News compressible benchmark file, this

highly compressible file provides a stress test for compressed

transfers because of the resulting heavy disk I/O.

08   Framing Error generally caused by line noise or an incorrect

transmission speed ("baud rate") or format (number of bits).  Also

caused by some cheap internal modems.  Most protocols require 8

bits no parity (-8n) for proper operation.  This is set

automatically by Omen Technology programs, but other programs may

not be so smart.

10   Break Interrupt generally caused by line noise or an incorrect

transmission speed ("baud rate") or format (number of bits).  Also

caused by some cheap internal modems.

18   Framing Error and Break Interrupt (see above) in combination.

This error may also be caused by defective 8250 UART chips.  Some

internal modem cards have slow 8250 chips that do not operate

properly with computers faster than 4.77 mHz.  Replacing the 8250

with a NS16550AN is recommended.  Please refer to the "Brain

Damaged UARTS" subchapter for details.

Please refer to the In Case of Difficulty chapter of this document for

solutions.

Chapter 14 Pg 42       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

14.6  TIMEOUT

Indicates the other program is not responding or the phone line has

disconnected.  If this message appears at the start of a transfer, you

may be using the wrong speed or the wrong port.

Less commonly encountered DSZ error messages are explained in the

Professional-YAM manual.

14.7  No Carrier Detect

If you see a No Carrier Detect message, and you haven't lost the

connection, you should fix your modem cable or modem settings to allow

proper operation of the Carrier Detect signal (CD).

A poorer alternative is to give the d command:  dsz port 1 speed 2400 d

t

Selecting the wrong port can provoke this message.

14.8  Port %d (%X) defective

When DSZ selects a serial port, it performs a quick hardware check to

verify the presence and minimal functionality of the 8250 family UART.

This message usually means a nonexistent port has been selected.

Reports have been received of functional but nonstandard UART chips

triggering this message.

14.9  COMMAND NOT FOUND

If you see this message, make sure DSZ is in the DOS PATH.  Sometimes

this error is caused by having too many environment variable strings.

14.10  Out of Environment

means there is not enough room in the program environment to accept more

data.

DOS prints this message when it cannot allocate more memory to

accommodate a "set" command, such as "set dirrx=c:/downloads/".

Try moving the "set" commands to the batch file that calls your comms

program, or to your AUTOEXEC.BAT file.

To increase environment, use the /E switch. The default is 160 bytes.

Chapter 14 Pg 43       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

An example for your CONFIG.SIS file: SHELL=C:COMMAND.COM/E:256

NOTE: Large environments may cause DSZ.COM to lockup.  This is less of a

problem with DSZ.EXE.

Chapter 15 Pg 44       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

15.  SLOW TRANSFERS

Some users report slower than expected file transfers even when few or

no errors are reported.  Some possible causes are:

If "handshake slow" is used, throughput may suffer.  Try one of the

other solutions to the "Line Status Register = 02" errors discussed

above.  When using "handshake slow" it may be possible to increase the

modem's interface speed to 19200 bps or faster.

Bad phone lines will silently slow MNP modems, sometimes without any

error indications.  If the modems disconnect fairly often you should

suspect marginal phone line conditions that are being hidden by MNP

error correction.

One of the programs may have set a parameter or option that acts to slow

the transfer under certain conditions.

Chapter 16 Pg 45       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

16.  IN CASE OF DIFFICULTY

16.1  LOCKUPS

16.1.1  Insufficient Memory

DSZ complains when it detects insufficient memory to operate.  When

calling DSZ from another program, there may be ALMOST enough memory for

proper operation, but DSZ locks up during a transfer or the calling

program freezes after DSZ displays the "DSZ Finished" message.

Sometimes there is a message about insufficient memory, sometimes just a

lockup and no message.  To verify this problem, call CHKDSK from within

your comms program (not from DOS!) by defining it as an external

protocol.  If you use an interface program (Hotkeyz, Prodoor, etc.), you

must call CHKDSK from that program, as those programs also reduce the

memory available to DSZ.

CHKDSK should report at least 64k free memory.  If less memory is

reported, reduce the size of the COMMS program's capture buffer, or

reboot the system with fewer/smaller device drivers and TSR programs.

Calling DSZ directly from the comms program instead of from an interface

also saves memory.

16.1.2  Environment TOO BIG

A large environment may cause DSZ.COM to lockup.  The size of the

environment (the amount of text you get when you give the DOS "set"

command) is subtracted from the data space available to 8086 C programs.

This can be a problem with DSZ.COM, whose 8086 tiny model limits code,

data, stack, and environment to 64 kb TOTAL.  Use DSZ.EXE (compiled with

8086 small model) to alleviate this problem.

16.1.3  Bad COM Port Configuration

DSZ is fastidious about which interrupt vectors it takes over.  Unlike

some programs, DSZ uses only the correct interrupt vector for its

interrupt service routine.  (This allows multitasking.)

DSZ obtains and releases this vector with standard DOS function calls.

This allows multi line operation under DESQview and other multitasking

systems.

Incorrectly configured serial ports that work with programs that pig out

both IRQ3 and IRQ4 will lock up with DSZ.  If DSZ locks up the machine

as soon as it tries to send or receive data, this is usually the fault.

Make sure the comm port is strapped correctly; COM1 uses IRQ4, COM2 uses

IRQ3.  DSZ will lock up if the comm port is not properly set up.

In addition, DSZ will not work properly if two devices are connected to

the same IRQ line, even when some other programs (that don't use

interrupts as intensively) do work normally on the same machine.  Make

Chapter 16 Pg 46       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

sure no other device uses the same IRQ as DSZ even if that device is not

active.

COM3 and above are not widely standardized.  If you have problems with

COM3 or COM4, chances are your "COM3" is non standard.

16.1.4  LAN Manager Conflicts

Some Local Area Network (LAN) managers take over comm ports using code

that interferes with DSZ.

The following hint is from Tom Louvaris, LANtastic Sales/Support - New

York.  Other LAN managers may require similar incantations to keep the

LAN manager from messing with DSZ's comm port.

Problem: When the lan is running, DSZ intermittently fails.  DSZ starts

the transfer by displaying "Ready to send pathname".  The receiving

system does noting.  Eventually, DSZ exits with code 1, and no file is

sent.

To release one or more Com Ports with LANtastic 3.x, enter the

following...

Net Queue Halt \Sever-name Comn

Note: You must be logged on to "Server-name" with Queue privileges to

Chapter 16 Pg 47       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

enable the operation of this command.

16.2  Incorrect Batch Files

Incorrect batch files are the number one source of problems using DSZ

for uploads and downloads.  Bulletin board programs with convoluted

batch file mechanisms exacerbate the problem.

DSZ's command line echo was added to assist in correcting defective

batch files.  You should see a display like:

Command line = 'rz '

or

Command line = 'port 2 speed 9600 sz foo.baz '

You may have to add a "pause" command at the end of the batch file to

allow DSZ's messages to be seen.  Another possibility is to turn on the

printer with ^P before running DSZ, so DSZ's message can be saved on

paper.

In general, the Command line display should look like one of the

examples in this document or those displayed by DSZ itself.  (The

Command line display does not include the name of the DSZ program

itself.)

Try running DSZ by itself, without any batch files or TSR programs,

calling it directly from DOS as described above.

16.3  Cheap Internal Modems

Numerous trouble reports have been received lately caused by cheap

internal modems causing errors on ZMODEM transfers but not XMODEM.  A

similar problem was experienced in the mid 80's during beta testing of

some 2400 bps modems.  Perhaps defective LSI chips left over from those

days were cloned or recovered from the trash bin.

16.4  Commands Entered in Upper Case

Most DSZ commands and options must be entered in lower case (small

letters, not big letters).  Where the documentation or help screen

indicates "rz", you must use "rz" and not "RZ".

16.5  No Carrier Detect

If you see a No Carrier Detect message, and you haven't lost the

connection, you should fix your modem cable or modem settings to allow

proper operation of the Carrier Detect signal (CD).

Chapter 16 Pg 48       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

A poorer alternative is to give the d command:  dsz port 1 speed 2400 d

t

16.6  DOS 3.x

DOS 3.x systems may need to have a "STACKS = 8,256" line in CONFIG.SYS

to prevent antisocial behavior. This is required on the NEC Multispeed,

or if certain TSR's are used.

16.7  Perstor Controllers

Several reports have been received indicating Perstor disk controllers

interfere with data communications.  Perstor is reportedly working on a

fix.

16.8  IBM PS/2 Model 50Z

Early samples of this machine have a design error on the motherboard

that causes comms programs to lock up the machine.  Replacing the

motherboard with a correctly functioning unit from current production

corrects the problem.  The part number of the replacement PS2/50 Z

motherboard is 35F5928.  (Information provided by John M. Choma

73047,3566) U.S. IBM stonewalled on this issue, but in December 1988

agreed to replace the defective motherboards under warranty.

16.9  Brain Damaged UARTS

Omen Technology has received reports of problems with buggy 8250 type

UART integrated circuits in Leading Edge modem boards, serial port

interfaces, and computers.  The defective chip logic affects high

performance software.  Replacing the buggy chip with a newer chip

(NS16550AFN) corrects the problem.

The UM82450 chip in the Zoom 2400HC also appears to cause problems.

Zoom Telephonics advises that this problem is limited to their old

model.  The old model has an external volume control.

You can identify an old chip by inspecting the serial interface board

for 8250 or UM82450 type devices.  A service technician or hardware

hacker can help if you are not familiar with reading chip numbers.

Dsz users can identify old chips with the following software command:

EXAMPLE: dsz port 1 speed 2400 p

The resulting value for the P numeric parameter should be 1.  A value of

0 indicates a non-buffered chip or a chip with a defective buffer.

Chapter 16 Pg 49       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

ZCOMM and Professional-YAM users can look at the printout produced by

the s command;  old UART chips register as TYP 0.

Some internal modems and PCMCIA cards use a firmware simulated UART that

is not detected as a buffered UART.  Properly designed firmware UARTs

provide excellent buffering.

The National Semiconductor NS16550AFN is a pin compatible plug in

replacement for 8250 and 16450 serial chips.  Omen Technology PCDOS

software enables the FIFO buffer in the NS16550AFN to prevents loss of

data from poorly written device drivers, TSR programs, etc.

We are also using the Goldstar GM82C550 chip with a 9025 date code.

This Goldstar chip is plug compatible with the National chips.  No

problems have been discovered with this chip.

The NS16550AFN has better electrical characteristics than the older UART

chips.  386 and fast 286 machines should use the NS16550AFN for best

results.  Omen Technology software has been written to allow time for

the older UART chips to operate. However, there is a limit to how much

the software can rearrange chip accesses without resorting to

performance robbing wait loops.  Some other software is not as forgiving

of slow chips.

Fortunately the FIFO UART chip situation continues to improve.  As of

this writing, the following plug compatible chips are known to work.

+ National Semiconductor NS16550AN and NS16550AFN, Goldstar GM82C550,

Startech 16550

Startech ST16C650CP This chip has 32 byte FIFO buffers and special logic

to support improved response time when using hardware flow control.

This chip may cause lockups when used in a Windows or Windows-95 DOS

Box.

********************

ERROR:input line 3138:LE:mismatched

********************  Texas Insturments TL16C550C This chip has special

logic to support improved response time when using hardware flow

control.  This seems to reduce overruns when used in a Windows DOS Box

compared to regular FIFO devices.

********************

ERROR:input line 3144:LE:mismatched

********************  Current Western Digital WD16C550 chips do not

function properly at low speeds.

The Startech 16C552 contains two UARTS and a buffered parallel

interface.

If your UART chip is soldered in or otherwise not replaceable, you will

need a new plug-in board with a good UART chip.  Byte Runner

Technologies 800-274-7897 offers a variety of boards and chips.

Chapter 16 Pg 50       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

Another is the KW-524E-1 Serial/Parallel/Game card, $30.90 from TSD

Systems, 800-331-9130 (FAX 407-331-8130).  A board populated with two

FIFO UARTS is $41.85.

The Hayes ESP board comes with two serial ports with NS16550AFN devices.

The ESP definitions for COM3 and COM4 agree with the industry standard

definitions used by Omen Technology.  All option switches on the ESP

board should be OFF except those used to select the port number (COMn).

The $99 "WinSport II" from Boffin LTD 612-894-0595 has two NS16550AFN

FIFO UARTS and also supports MobyBaud (18.432 MHz clock).  This card

also supports higher IRQ numbers, freeing up the low IRQs for other

applications.

Software Electronics 503-640-1946 sells a $59 board with two FIFO UARTS

and a bidirectional enhanced parallel port.

The following information comes from a number of people:

"I have located a small dealer in New York who has the cards with or

without the 16550.  I believe the price is around $50.00.  The company

is RJK Consulting and the number is 718-968-9167. The gentleman to speak

to is a Mr. RJ Kulman. ...  I got this information off the "Interlink"

BBS system." Fm: Nick Braak [TRICON] 75156,1254

Fry's - 415-770-FRYS.  This is a Silicon Valley electronic department

store.

Jameco Electronics at 415-592-8097 800-538-5000 sells National

NS16550AFN chips mail order (credit card) in small quantities.

Unfortunately, we received a number of reports (none recently) that

their serial i/o card was supplied with the defective WD16C550 soldered

in, even when ordered with specific instructions to include 16550AFN

chips.

TSD Systems advertise a 2 serial/parallel/game card with one or two

NS16550-AFN chips.  800-331-9130(voice) 407-331-8130(fax)

Microprocessors Unlimited, 24000 South Peoria, Beggs, OK. (918) 267-

4961. The price for the chip is $13.50 plus shipping costs.

Consolidated Computers 4 Shoppers Lane Suite 121 Turnersville, NJ  08012

(609)751-9595(voice) (609)751-3843(bbs) (609)751-3840(fax)

Arrow Electronics at 800-932-7769 (516-467-1000).

Willies Computer Software Company: 800/835-4832.

N.B.: Current Western Digital WD16C550 chips do not function properly at

low speeds.  This has been confirmed with samples received directly from

Chapter 16 Pg 51       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

Western Digital.  WD is aware of this problem, but has not informed us

of a corrected chip.

16.9.1  Everex Internal Modems

Numerous reports have been received concerning difficulties with Everex

modem cards.  Some of these cards contain buggy 8250 chips which are not

up to the demands of full duplex streaming protocols such as ZMODEM.

Replacing the buggy 8250 chip with NS16550AN chip corrects these

problems.

16.10  Weird Iron

A few machines, some with V20 CPUs, reportedly don't like DSZ.COM.  We

have not been able to repeat this; TeleGodzilla (using a V20) runs

DSZ.COM without a hitch.  Switching to DSZ.EXE has cleared up the

problems on other machines.

16.11  PKARC 3.6

It has been reported that PK36 grabs some interrupts and doesn't restore

them on exit in an attempt to prevent hacks of the program.  Since

eliminating that particular version and cleaning up memory, high speed

transfers even with 8250 UARTS work properly again.

The problem with PK 3.6 appears to be most severe if a BBS operator has

doorware, etc that accesses the PK programs from a shell of some sort.

This info may be helpful to users who are having otherwise inexplicable

file transfer problems.

Phil Katz has written me that this problem has been corrected in

subsequent versions.

16.12  Will the real YMODEM please stand up?

Some XMODEM programs claiming to support YMODEM neither send nor accept

the YMODEM pathname block.  1024 byte XMODEM is XMODEM-1k, not the

YMODEM defined by Ward Christensen in 1985 when he invented the term

YMODEM.  1987 and later versions of YMODEM.DOC remind the reader of the

official YMODEM definition, which has not changed since YMODEM's coinage

in 1985.

Chapter 16 Pg 52       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

16.13  286 and 386 Extended Memory

Virtual memory disk caches and ramdisks using 80286 or 80386 extended

memory require a kludge hardware reset of the CPU chip to return to the

normal "real mode" after accessing extended memory.  This interference

to the interrupt response time causes lost characters on comm ports, and

may be minimized by specifying shorter memory transfers.

Switching to a Goldstar GM82C550, National NS16550AN or NS16550AFN UART

chip helps tremendously, allowing transfers at 115200 bps instead of

9600 bps with a standard 8250.  Please refer to the subchapter on "Brain

Damaged UARTs".  The DSZ "handshake slow" command may also help.

16.14  Disk Caches

Cache programs often interfere with ZMODEM transfers.  In one instance,

replacing Microsoft's SMARTDRV.SYS with Multisoft's SUPERPCK.EXE helped.

Norton NCACHE and and Multisoft PCQWIK have also caused errors.

Some cache programs can be turned on and off; try turning it off when

running DSZ.

The current SMARTCACHE can be used to advantage by using "handshake

slow" and the 'zmodem k" numeric parameter described in dszbg.doc.

Switching to a National NS16550AN or NS16550AFN UART chip helps

tremendously, allowing transfers at 115200 bps instead of 9600 bps with

a standard 8250.  Please refer to the subchapter on "Brain Damaged

UARTs".  The DSZ "handshake slow" command may also help.

Chapter 17 Pg 53       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

17.  ASSOCIATED DOCUMENTS

Consult the Professional-YAM, PowerCom, or ZCOMM manual chapter "OPTIONS

FOR FILE TRANSFERS" for descriptions of the file transfer options.

(ZCOMM is a full featured shareware communications program with

autodialing, autolearn scripting, and terminal emulation.) The "FILE

TRANSFER BASICS" chapter explains the relative merits of XMODEM, YMODEM,

ZMODEM, and other protocols and gives tips on their use.  The "MAIN

COMMANDS" chapter describes the rb, rx, rc, rz, sb, and sz commands.

The "MODEM INTERFACING" and "FLOW CONTROL" chapters contain important

information on the use of buffered modems with DSZ.

Information on the ZMODEM Inter Application File Transfer Protocol is

available in Omen Technology's "ZMODEM DEVELOPER's COLLECTION" ($89.00).

Unix/Xenix C Source code for XMODEM, YMODEM, and ZMODEM transfers

to/from Omen Technology Products is included in RZSZ.ZOO.[1] This file

includes a Pro-YAM/ZCOMM script to upload a bootstrap file to a Unix

system, compile it, and (if the compilation is successful) upload the

rz/sz source files.  (ZCOMM is a full featured shareware communications

program with autodialing, autolearn scripting, and terminal emulation.)

__________

1. Technical support for rzsz is available on a paid consulting basis.

Chapter 18 Pg 54       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

18.  FILES

Some of the batch files mentioned in this document are not included in

the archives.  Those that are needed can be readily created with your

favorite editor.

DSZ.COM, DSZ.DOC, 2A-PIF.DVP, 2S-PIF.DVP, 2T-PIF.DVP, BBS_REG.TXT,

DS.BAT, FIX.BAT, MAILER, ZMODEMAD.BAT, ZMODEMD.BAT, ZMODEMDR.BAT, and

ZMODEMU.BAT constitute Omen Technology's DSZ.ZIP.  These same files are

in DSZ.ZOO on the distribution diskette mailed to registered users.[1]

2S-PIF.DVP, 2A-PIF.DVP, DS.BAT, and 2T-PIF.DVP are used with Quarterdeck

Office Systems' DESQview multitasking executive for background

operation.  Their use is described in a previous chapter of this

document.

DGSZ.ZOO is distributed to GSZ registrants.  It contains GSZ.EXE and

GCOLORS.EXE in addition to the above.

ZCOMMDOC.ZIP (manual), ZCOMMEXE.ZIP (programs), ZCOMMHLP.ZIP (help

processor and database), make up the ZCOMM communications tools

shareware distribution.  The ZCOMM files are available on finer bulletin

boards.  (ZCOMM is a full featured shareware communications program with

autodialing, autolearn scripting, and terminal emulation.)

RZSZ.ZOO contains ZMODEM "C" source code for COMPLETE Unix/Xenix

programs licensed for use transferring files to/from licensed Omen

Technology products.[2]

RZSZ.TLB The ZMODEM rz/sz programs are available in native VMS flavors

with wild cards, record format support, etc..  Pre compiled VAX/VMS

files are contained in the RZSZ.TLB and RZSZAXP.TLB compressed text

libraries, available in the TeleGodzilla (503-621-3746) upgrade/vms

subdirectory, Compuserve VAXFORUM, GEnie, and other bulletin boards.

(RZSZAXP.TLB is for VMS on the Alpha machines.) The programs in RZSZ.TLB

are part of the extra value provided by Omen Technology communications

software.

DSZ.EXE compiled with the Xenix to DOS XC may give moderately higher

maximum throughput noticeable on slower machines.  Sending from a faster

__________

1. DSZ.ZOO is supplied to registrants instead of DSZ.ZIP because not

all registrants have copies of unarchiving programs.  The

registration disk includes a copy of BOOZ, a simple dearchiver for

ZOO files.

2. Technical support for rzsz is available on a paid consulting basis.

Chapter 18 Pg 55       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

machine to an 8 MHz IBM AT with 16550AFN, using 115200 bps and "sz -yZ

rttypix":

Program                       Throughput

DSZ.EXE                       12735 cps

DSZ.COM                       7164 cps

ProComm+2.0                   5928 cps

(Compare these speeds to those listed on Page 159 of the April 30, 1991

issue of PC Magazine.  Results are even better with the compressible

file used in the PC Magazine tests.)

DSZ.EXE supports ZMODEM over 7 bit paths (E and P options), and

specified control character quoting (-Q<string> option).  The DSZ.EXE

requires somewhat more memory than DSZ.COM.  The maximum DSZ.EXE I/O

buffer size available with the B numeric parameter is 16384.  DSZ.EXE is

available in DSZEXE.ZIP and DSZEXE.ZOO.

ZCOMM and Pro-YAM also run faster than DSZ.COM.  (ZCOMM is a full

featured shareware communications program with autodialing, autolearn

scripting, and terminal emulation.)

DSZBG.ZOO contains DSZBG.DOC.  Part of the DSZ registration disk, this

file provides background information on the ZCOMM/Pro-YAM commands

provided in DSZ and GSZ.

The ZOO archive format is a machine independent improvement on the PC-

DOS specific .ARC format. The ZOO format has been precisely defined, and

stable ZOO archivers/dearchivers are available on a wide variety of

systems.

BOOZ.EXE unpacks .ZOO archives.  BOOZ.DOC describes BOOZ.EXE.

Chapter 19 Pg 56       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

19.  Other Omen Technology Products

19.1  Professional-YAM COMMS TOOLS

Omen Technology makes Professional-YAM, a comprehensive comms program

with unmatched power.  (Professional-YAM is a full featured commercial

communications program with autodialing, autolearn scripting, and

terminal emulation.) Flavors are available for PC-DOS, Unix, Xenix, and

OS/2 Protected mode.  Unix source code is available under license.

19.2  ZCOMM: Industrial Strength COMMS Shareware

ZCOMM is a full featured shareware version with a $40.00 basic

registration.  Registered DSZ users upgrading to ZCOMM get a free

binder!

19.3  ZMODEM Developer's Collection

Omen Technology sells the "ZMODEM Developer's Collection" for $89.00.

This diskette includes the most recent public domain versions of

YMODEM.DOC, ZMODEM.DOC and the Unix/VMS rz/sz C source code for YMODEM

and ZMODEM.  Porting to the intended application is the sole

responsibility of the purchaser.  For many developers, this collection

affords a significant labor savings in obtaining these materials.

Chapter 20 Pg 57       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

20.  FAIR PLAY

Unlike snazzy menu interfaces, reliable high speed transfer protocols

take many hours of testing, often over expensive phone connections, to

develop.  Two decades of communications experience and protocol testing

are distilled in DSZ.

DSZ is a user supported program developed and supported by Chuck

Forsberg, owner of Omen Technology.  Countless hours of work and

expen$ive resources went to develop and support DSZ for the myriad

environments users use it in.  The fact that most of the problems people

encounter with DSZ are caused by other software, hardware problems,

failure to read the supplied documentation, or pirating has not kept

users from seeking and getting help, even if it means calling the author

in the middle of his sleep.

Beyond this, the world of communications is constantly evolving, and it

requires a significant ongoing effort to keep DSZ and its documentation

up to date with the current state of the art.  Individuals with little

knowledge of the dynamic nature of telecommunications (or hidden

agendas) have misrepresented the frequency of enhancements to DSZ as a

sign that DSZ is "unstable" and unworthy of "serious consideration".  If

you think such sniping is fair play, you are more than welcome to use

one of the many other protocol programs available, each of which claim

to offer speed and reliability comparable to DSZ.

Non registered users are welcome to use DSZ for a reasonable evaluation

period.  "Reasonable" means "until you've decided whether the program is

worth $20.00." It does not mean "until some competing program gets

around to adding ZMODEM or True Ymodem(TM)".  Once you've evaluated DSZ,

ethical use of DSZ requires registering the copy.

DSZ is not and never has been Public Domain.  Every version of DSZ has

included a Copyright Notice and a prohibition against sale or

modification.  There is no usage counter or other Trojan Horse in DSZ,

but if you use this program, you have a moral and ethical obligation to

register it.

Paid registration enables t -8g, DIRRX, DSZOPT, OverThruster(TM),

YMODEM-g, and other advanced features that are present in DSZ.ZIP in

encrypted form.  The registration package serializes and decodes all

past and future versions of DSZ.COM and DSZ.EXE.  You can get a fully

featured upgrade of DSZ simply by downloading a newer version from

ftp.cs.pdx.edu, TeleGodzilla (503-621-3746), BIX, GEnie (IBM

Roundtable), CompuServe, and trusted bulletin boards.

Chapter 20 Pg 58       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

20.1  Sharing DSZ Files

You are welcome to share unmodified copies of DSZ.ZIP or DSZ.ZOO with

your friends and associates for their evaluation.  This documentation

file and the MAILER file must be provided with all copies of DSZ except

by prior written permission from Omen Technology Inc.  Because of piracy

problems, Omen Technology must INSIST that DSZ be distributed only as

part of the DSZ.ZIP file EXACTLY as provided by Omen Technology Inc, or

in an archive which displays a CRC value for DSZ.COM equal to that shown

in the Changes chapter.  If DSZ.ZIP is repacked, no files may be added

to or removed from the archive.

If you see an improperly packed or otherwise modified DSZ archive or

parts thereof in a bulletin board, please ask the board's operator to

replace such files with DSZ.ZIP exactly as provided by Omen Technology

Inc.  Flame them until they behave ethically.

Unmodified copies of Omen Technology shareware and demo files including

GSZ.ZIP, DSZ.ZIP and DSZBG.ZIP may be distributed by "shareware

distributors" provided 1) the files are clearly identified as

unregistered shareware, 2) the cost of the distribution is nominal.

Otherwise, no version of DSZ may be sold without the prior written

permission of Omen Technology Inc.

DSZ is not and never has been Copy Protected.  "DSZ unprotect" files

have no function whatsoever except to aid and encourage the theft of

Omen Technology's Intellectual Property.

Distribution or use of programs or procedures that modify, patch,

disassemble, decompile, or otherwise reverse engineer DSZ without the

prior written permission by Omen Technology Inc is a bloody theft of

intellectual property and services and is a violation of Federal

Copyright Law.  The same applies to debug scripts and documentation

written for the same purposes.  These perform the same function as

pirate satellite and/or cable descramblers, to obtain benefit from

encrypted communications without proper payment.  Piracy has been

investigated by the FBI and prosecuted under federal statutes.

Applicable law provides for damages and recovery of attorney's fees.

Omen Technology has received reports of serious problems caused by

unauthorized patches to DSZ, and cannot assume liability for the

performance of the resultant code.  So far, none of the slime producing

such bilge have seen fit to sign their real name and address to their

cretinous attempts at ego gratification.

Distribution of DSZ in any medium without this documentation file is an

unauthorized modification to DSZ that deprives the rightful author of

registration revenues.  That is a theft of intellectual property.  Such

distribution damages the author by forcing the user and author to waste

time and phone bills scrounging for the documentation, and answering

questions that are fully covered in the documentation.

Chapter 20 Pg 59       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

If any version of DSZ is distributed with any other product, software or

hardware, the product documentation must clearly and prominently state

that the customer's continued use of DSZ requires registration directly

with Omen Technology or its duly authorized agent(s).  When unregistered

copies of DSZ are distributed on disk, a contemporary copy of

ZCOMMDOC.ZIP should be included on the same disk.

Thank you for your support.

20.2  REGISTRATION

After a short evaluation period, ethical users must register their copy

of DSZ with a check for $20.00 in U.S. funds sent to:

Omen Technology Inc.

P.O. Box 4681

Portland, OR 97208

The address on your check is sufficient, no need to fill out forms.

For VISA/MC Registrations, you can also:

Call VOICE: 503-621-3406

Or register online: 503-621-3746

Or FAX the INSTANT MAILER to: 503-621-3735

Don't bother scrounging for an envelope, just print the INSTANT MAILER

included in this archive!

Overseas registrations are sent out International Air Mail.  Users,

especially those overseas, may choose to register their copy of DSZ for

$25.00 U.S.  by providing their VISA or Mastercard card number with

expiration date and their name as it appears on their credit card.

Purchase orders for less than $150 must be prepaid.  Omen Technology Inc

Federal TAX ID #: 930858776 Oregon TAX ID #: 536565

.ne 12

"Das Recht zur Benutzung von DSZ in Deutschland, Oesterreich und der

Schweiz erwerben Sie durch Bezahlung der Lizenzgebuehr von DM 57,00

(incl. Mwst) an RzK

Doris Koepke

Bitzenstr. 11

D-5464 Asbach

Tel. 02683-4547, FAX 49 2683 4537, Telex 86 3705 rzk d, CIS

[70126,1660]. Sie erhalten dann von dort sowohl die neueste Version, als

auch Ihren individuellen Software-Key, mit dem Sie auch kuenftige

Shareware-Versionen "aufschliessen", also die in der Shareware Version

nicht zugnglichen Programmfunktionen nutzbar machen koennen, per Post.

Chapter 20 Pg 60       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

Die Benutzung nicht registrierter Versionen stellt einen Verstoss

geltenden Rechts dar und wird strafrechtlich verfolgt."

*************************************************************************

The user registration package includes a printed serial number password

(SNP) card.  The registration "care package" diskette contains the

latest versions of the DSZ programs, additional user documentation, and

the installation program which uses your personal SNP to enable advanced

features and bypass the opening advertisement.

The registration package unlocks the extra value in all past and future

versions of DSZ.COM and DSZ.EXE.

*************************************************************************

The unique serial number on the registration card is registered to the

user, and is acceptable proof of registration to corporate auditors.

The putsnp program inserts your unique DSZ Serial Number Password (SNP)

into you copy of DSZ with the following command:

putsnp dsz.com

Putsnp prompts for your SNP and asks appropriate questions before

inserting the SNP to enable the extra value in DSZ.

GSZ is serialized with the GCOLORS program described above.

Some TSR programs and device deivers interfere with putsnp.  If you have

a lockup running putsnp, run it without any device drivers or TSR

programs (boot a plain DOS from floppy disk).

Your disk directory entry for DSZ will show the date and time you

serialized your copy of DSZ.  This does not affect the revision date DSZ

announces when DSZ is run.

The only legitimate way to disable the opening advertisement in DSZ is

to insert an authorized SNP obtained from Omen Technology into DSZ with

the putsnp program.  Any other suppression or modification of the

opening screen is an illegal, unethical theft of services.  Omen

Technology has received reports of serious problems caused by

unauthorized patches to DSZ, and cannot assume liability for the

performance of the resultant code.

Piracy is stealing.

Chapter 20 Pg 61       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

20.3  Files on Registration Disk

The following files are currently included on the Registration Disk sent

to paid registrants.

BOOZ.EXE       Program to unpack .ZOO archives.  BOOZ.DOC describes

BOOZ.EXE.

DSZEXE.ZOO     Contains DSZ.EXE.

To dearchive with booz: booz x dszexe

Type "booz" without arguments to see the help message.

DSZ.ZOO        Contains DSZ.COM, DSZ.DOC, DESQview description files.

To dearchive with booz: booz x dsz

DGSZ.ZOO       Contains GSZ, DSZ, GSZ.DOC, and DESQview description

files.

To dearchive with booz: booz x dsz

RZSZ.ZOO contains ZMODEM "C" source code for COMPLETE

Unix/Xenix programs licensed for use transferring files

to/from licensed Omen Technology products.[1]

To dearchive RZSZ with booz: booz x rzsz

PUTSNP.EXE     Installs your unique registered serial number to enable

the extra value in DSZ.  (GCOLORS is used for GSZ.)

DSZBG.DOC      (in DSZBG.ZOO) Background information for GSZ and DSZ,

consisting of relevant parts of the ZCOMM User's Manual.

To dearchive with booz: booz x dszbg

__________

1. Technical support for rzsz is available on a paid consulting basis.

Chapter 20 Pg 62       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

20.4  Bulletin Board Registration

Note to the wise BBS operator: be sure you understand the DSZ restrict

command and how to use it.

Use and/or copying of DSZ by bulletin board sysops and/or systems is

allowed according to the following conditions:

I agree to feature the ZCOMM files (ZCOMMEXE.ZIP, ZCOMMDOC.ZIP,

ZCOMMHLP.ZIP) for downloading, and to announce the availability and

features of ZCOMM in any bulletins, help displays, or other displays

that mention communications programs.  It is my responsibility to

maintain reasonably current versions of the ZCOMM files on the BBS shown

above.

I understand that I am welcome to use DSZ on my bulletin board(s) from

the time I sign and mail this document as long as I abide by the

conditions above, including featuring the above mentioned ZCOMM files.

I shall distribute DSZ.COM only as part of the DSZ.ZIP EXACTLY as

provided by Omen Technology Inc, or "zipped" without any file additions,

deletions, or modifications.  If I distribute DSZEXE.ZOO, I shall also

maintain DSZ.ZIP in the same directory since DSZEXE.ZOO references files

in DSZ.ZIP.

If I see a modified DSZ archive or parts thereof in a file or collection

of files (e.g., archive) on a bulletin board, I shall replace such files

or archive members with the contents of DSZ.ZIP exactly as provided by

Omen Technology Inc.  I shall report instances of noncompliance to Omen

Technology.

I shall not allow any version of dsz to be sold, modified, patched,

hacked, disassembled, decompiled, or otherwise reverse engineered

without the prior written permission of Omen Technology Inc.

I understand that DSZ is not and never has been Copy Protected.  I fully

understand that "DSZ unprotect" files have no function whatsoever except

to aid and encourage the theft of Omen Technology's Intellectual

Property.

(For this BBS registration, a multi node bulletin board operating from a

single, common file system counts as one machine.)

The purpose of bulletin board registrations is to support and promote

ZCOMM and Professional-YAM by popularizing the ZMODEM protocol and

featuring the ZCOMM shareware files available for downloading.

BBS registrations must be submitted on the bbs_reg form with signature

and date, and must accompany a new paid DSZ or ZCOMM registration.  The

paid registration that must accompany your BBS registration can be one

submitted by one of your board's users.  Most boards have many

Chapter 20 Pg 63       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

unregistered DSZ users, so getting one of them to register their DSZ

should pose no difficulty.  Since the paid registration may be from one

of your board's users, you can get your BBS registered for free by

getting one of your users to register his/her DSZ at the same time.

Chapter 20 Pg 64       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

21.  CHANGES

Be sure to save your serial number/password (SNP) card and the putsnp

program you received from Omen Technology to use with future versions of

DSZ you may download!  (Use GCOLORS to serialize GSZ.)

Current GSZ and DSZ versions are maintained on EXEC-PC BBS, BIX, GEnie

(IBM Roundtable), CompuServe (IBMCOM and CONSULT SIGs) as well as Omen's

TeleGodzilla (503-621-3746).

DSZ is a popular program which has been the object of virus attacks.

Pirated/hacked versions of DSZ have cost users and DSZ's developer

thousands of dollars in lost time and other expenses, in addition to

injuring of Omen Technology's reputation.

Virus and Trojan code hidden in hacked/pirated copies of DSZ and

programs claiming to "register DSZ" have caused loss of data.

-rw-r--r--   1 caf      omen       53982 Jun  7 01:33 dsz.com

-rwxrwxr-x   1 caf      omen       61399 Jun  7 01:02 dsz.exe

PKZIP -vt 32 bit CRC: 8F0086D4   53982  dsz.com

PKZIP -vt 32 bit CRC: 0A1D5AF5   61399  dsz.exe

Your copy of DSZ should show the same CRC values.  ZIP uses a 32 bit CRC

compatible with the Professional-YAM and ZCOMM "crc" command.

pkzip -vt dsz

The following Pro-YAM/ZCOMM command gives the 32 bit CRC for DSZ.COM:

EXAMPLE: >>>c: crc dsz.com

If your copy shows a different CRC or file length, insist on a

legitimate copy and report the source of the bogus/pirated copy to Omen

Technology Inc.  Omen Technology has received reports of serious

problems caused by unauthorized patches to DSZ, and cannot assume

liability for the performance of the resultant code.

COM7 and COM8 are changed; see the "port" command above.

A problem in the 1/17/89 and 2/2/89 DSZ versions corrupts files under

unusual circumstances.  Please delete all instances of the 1/17/89 and

2-02-89 version.

There never was a "zmodem d" parameter in DSZ.  Older versions of DSZ

ignored attempts to set this nonexistent parameter with a message that

humans ignored.  DSZ now aborts when asked to set a nonexistent

Chapter 21 Pg 65       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

parameter.  There is a "d" parameter that controls time/date stamping of

files; Bulletin boards that need to disable time/date stamping of

received files should simply use "pd0" not preceded by a "z".

Good news for tired frog stompers!  The ZMODEM rz/sz programs are

available in native VMS flavors with wild cards, record format support,

etc..  Pre compiled VAX/VMS files are contained in the RZSZ.TLB

compressed text library, available in the TeleGodzilla (503-621-3746)

upgrade/vms subdirectory, Compuserve VAXFORUM, GEnie, and other bulletin

boards.  FTP access at ftp.cs.pdx.edu pub/zmodem/rzsz.tlb and Simtel20

mirrors.  (Get RZSZAXP.TLB for Alpha VMS systems.)

The DSZ and GSZ "t" command uses the DOS console driver for terminal

emulation.  Standard ANSI.SYS is limited to a small subset of ANSI

escape sequences; NANSI.SYS, FANSI-Console and other replacement console

drivers provide faster screen updates and better ANSI terminal

emulation.

DESQview users: make sure "Separate Shift State" is set to "N".  (Tnx to

Tim Trott for this info.)

Jun 7 1995: Added support for the Texas Insturments TL16C550C ACE with

AUTO-FLOW control.  This chip is identified as type 4.  GSZ displays

were optimized.

-h no longer spazzes.

Sep 18 1994: Supports the Startech ST16C650 ACE with 32 byte FIFOs and

on chip hardware flow control support.  This chip is identified as type

3.

July 17 1994: GSZ and DSZ now accept extended receiver defined character

escaping requests which can specify the all ones character FF to be

escaped.

April 26 1994: The ZMODEM k, K and J numeric parameters have been added

to help on machines with old style UARTS encountering LSR 02 serial inut

errors.

Feb 25 1994: The Windows multitasking added in the Jan 27 version has

been modified; it was overly generous.  The subchapter on Windows

operation has been changed.

Feb 02 1994: Fixed anomalies with YMODEM-g and time display.  Hardware

support was revised to accommodate a Hayes ESP board incompatibility

with the 16550 spec.

Jan 27 1994:  Added the "G" command to send an XON character to the

modem.  This is useful when calling dsz from MSKermit as MSKermit sends

an XOFF character to the modem.  Multitasking on OS/2 and Windows has

been improved.  Handshake slow is more aggressive in flushing disk

buffers.

Chapter 21 Pg 66       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

Nov 29 1993:  Rebuilt to correct zipfile authentication problems.

Detection of 7-bit and nontransparent paths improved.

Nov 27 1993:  Serial I/O has been tuned for more efficient operation

with internal modems using firmware driven virtual UARTs.

May 29 1993: The portx command now works with IRQ's up to 15, and,

optionally, nonstandard baudrate clocks.

EXAMPLE: dsz portx 3e8,15 rz -m

Newer UART chips can use raw bit rate sources higher than the standard

1.8432 MHz.  Some 386 and 486 machines support MobyBaud(TM) operation

above 115200 bps.

To support a nonstandard bit rate source, multiply 5760 by the new raw

bit rate source frequency in MHz divided by 1.84320.

EXAMPLE: dsz portx 3e8,15,57600   sp 38400 rz -m

supports the 18.432 MHz clock option on the Boffin Electronics "WinSport

II" 2 port serial board.

May 3 1993: SlugBait should now work for XMODEM and YMODEM.  Some

problems with file transfers over 7-bit paths were fixed (DSZ.EXE,

GSZ.EXE).

Nov 9 1992: Unwanted registration info message removed from registered

copies.  Enhancements to accommodate non standard XMODEM programs.

Corrected spurious automatic recognition of hardware handshake

configuration.

May 3 1992: Recompiled to correct bad file size/CRC values in DSZ.DOC

and DSZNEW, pruned some unused code.  No functional changes from April

27 vsn.

April 27 1992: The GSZWINDOW environment variable can be set to 1, 2 or

3 to leave the top 1, 2 or 3 lines unmolested.  DSZ.EXE is compiled on a

newer compiler and now supports file sharing.

Bogus/hacked/infected files with early 1992 revision dates have been

reported.

Dec 13 1991: DSZ and GSZ automatically support Doorway 2.2 without extra

commands.

The RZ.BAT and SZ.BAT files are supplied for convenience.  RZ.BAT allows

calling programs that emit an "rz" command to automatically start DSZ

from the DOS command prompt.  SZ.BAT is a convenient command interface

that complements RZ.BAT.  You may modify SZ.BAT and RZ.BAT with your

favorite options.

The GSZ file transfer display now shows a distinct color at the

locations where error retries occurred.  This gives a visual impression

Chapter 21 Pg 67       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

of the distribution of error retries.

Oct 15 1991:  To accommodate other vendors' programs which do not

provide a convenient way to control crash recovery, the "restrict"

command security check is not performed if the destination file does not

exist.

Oct 8 1991: Some versions of putsnp were confused by a data pattern in

the Oct 7 dsz.com, misplacing the installed serial number.  Oct 7 1991:

The "t" (talk) command now accepts -h and -H to enable local echo.  Oct

1 1991: Fixed an interaction between FIFO uarts and YMODEM-g termination

after all files have been sent.  Fixed problem of some options being

ignored on XMODEM and YMODEM downloads.  Fixed graphics display problem

involving more than 40 MB of files.  Sep 4 1991: GCOLORS now saves the

color scheme in a separate file GCOLORS.T.  Many small improvements in

the GSZ file xfer display.  GCOLORS adds a "m" menu choice to remove

color from the color scheme.

A conflict between crash recovery and user specified short packets has

been corrected.

DSZ.EXE and GSZ now accept the "c" option to force incoming files to use

a changed file name when a file with the same name exists.  DSZ.COM does

not support this.  The receiving program must support this new ZMODEM

option or file disposition will be undefined.

Aug 7 1991: Serial I/O modifications have solved a Wedge-O-Matic(R)

problem with one hardware configuration (antique 2400 bps internal

modem).  Hopefully this will solve similar non-replicatable problems for

other users.

A number of display fixes/enhancements to GSZ, including the addition of

the GSZWINDOW environment variable enabling BBS sysops to protect BBS

status lines at the bottom of the screen.

July 29 1991: Corrections to gcolors registration function.

July 28 1991: Some changes to GSZ and CGOLORS to improve human factors.

"GCOLORS -q" disables the flashing reminder of which field is being

modified.  This was added to help users with certain visual impairments.

A few users have reported hung transfers wither certain hardware and/or

software configurations.  We have not been able to replicate the

problem, but have added code that recovers from lost interrupts.

July 21 1991: Preview release of GSZ!!

July 3 1991: Spurious "Invalid Download Directory" message has been

Chapter 21 Pg 68       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

removed.  More hacks to accommodate AT&T's mutant "ymodemb" protocol.

AT&T may be the right choice, but their "ymodemb" is not the right

protocol.

June 13 1991: DSZLOG entries made by the 6/10 DSZ had no carriage

returns.  Please check the "pxN" and "z pxN" command descriptions for

important clarifications.

June 10 1991: A problem with sending with flow control has been fixed.

Intelligent Crash Recovery(TM) has been enhanced to prevent timeouts

when computers with different CPU speeds are used.

May 31 1991:  The "sb" command has been hacked to accommodate an AT&T

nonstandard "ymodemb" protocol.

The handling of hardware flow control has been hacked to prevent flow

control timeouts caused by TSR programs that poll the UART status

registers.

The size of the input buffer has been increased.

The "V" numeric parameter has been added to DSZ.COM to allow direct

video output in some instances.

April 29 1991: The "t" command no longer ignores options set with DSZOPT

and/or as flags to the "t" command.

April 16 1991: Operation of "handshake slow" has been enhanced when

receiving with ZMODEM CRC-32 and MobyTurbo(TM).  Users who had

previously found "handshake slow" unable to control disk related "Line

Status Register 02" overrun errors should find the new "handshake slow"

more effective on ZMODEM CRC-32 and MobyTurbo downloads.

DSZ.COM and DSZ.EXE now announce themselves as DSZ.COM and DSZ.EXE.

DSZ.DOC has more discussion of differences between .COM and .EXE flavor.

See the FILES chapter for expanded discussion of DSZ.COM vs DSZ.EXE.

April 4 1991:  DSZ now supports 115200 bps on suitable hardware.

DSZ.EXE throughputs of 113000 bps have been achieved with modest

trickery.  "Your mileage may vary."

The -I option has been added to the sz command.  The -I option causes

new versions (Mar 29 1991 or later) of VMS rz to store files with the

VMS Variable length (mrs=510) binary format.  Adding this record format

should prevent VMS Kermit servers from corrupting file contents.

Feb 28 1991:  The instructions for adding DSZ to ProComm Plus have been

updated to include PCP version 2.  Datastorm has not licensed ZMODEM-

90(TM) despite allegations that DSZ is now "built in" to ProComm Plus

version 2.

Chapter 21 Pg 69       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

The F command was added to leave the 16550A FIFO enabled on exit.  The

speed command now updates the effective baudrate used for time

estimates.

sz -e is more network friendly.  (If you don't know what sz -e does, you

don't want to know - trust me.)

Dec 16 1990:  Code changes to improve operation with the Intel

SatisFAXtion modem/fax board.

Nov 19 1990:  A number of small enhancements.

The sz -i and -u options were added.  The -i option is useful sending

files to VMS rz version 4.x, forcing VMS Stream_LF record format.  The

-u option unlinks (deletes) a file after it is successfully sent with

ZMODEM.

SlugBait has been fixed.

The Y numeric parameter (described above) has been added to allow FIFO

hackers to do their thing.

June 25 1990: When a ZMODEM block length is forced, the receiving DSZ

writes the data to disk before acknowledging the block.  With DSZ.EXE

and its maximum 16384 buffer size, you may be able to avoid overrun

(Line Status Register 02) problems receiving to slow disk drives by

using:

DSZ pB16384 z pl16384 rz

ZMODEM-90(TM) speeds up GEnie ZMODEM downloads; see DSZOPT.

The "sz" command had an undocumented fallback to YMODEM, this has been

removed for reliability reasons.

The SlugBait (-s) option now causes DSZ to return success (0 exit code)

if at least one file is transferred.

April 11 1990: Enhancements for more efficient multitasking under

DESQview and DESQview 386.  Code which prevented IRQ conflicts between

COM1/COM3 and COM2/COM4 has been removed to allow use of COM7 and COM8

without interference from DSZ running on COM1 or COM2..

ZIP replaces ARC as the standard download distribution format on new

files.

March 21 1990: DSZ now sends the CANCEL sequence if it runs out of disk

space while receiving a file with ZMODEM.

DSZ now defaults to "handshake both" if the clear to send signal is high

when DSZ selects the comm port regardless of the initial "baud rate".

Chapter 21 Pg 70       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

The P parameter can be set to -1 to disable the FIFO on the Western

Digital WD16550 chip.  The WD16550 FIFO has been observed to lose

characters when transmitting at slow baud rates.  A more elegant

software solution to this hardware problem may be discovered in the

future.

Feb 25 1990: Doco change only. The "R" numeric parameter or the

"restrict" command may be used to force partially received files to be

deleted.  These features have been in DSZ for a long time, but never got

into DSZ.DOC.

Feb 18 1990: DSZPORT now accepts "portx" style arguments in addition to

the traditional decimal number.  The -Wn option has been improved.

Jan-28-1990: Doco improvements, especially to the "In Case of

Difficulty" chapter.  BBS registration clarified for those that bother

to read it.  Qmodem batch files fixed.  No change to the DSZ software.

Jan-22-1990: ZMODEM-90(TM) extensions:

Receiver specified window size: The -Wn option operates similarly to the

-wn option.  The -Wn option is specified to the receiver, which sends

this information to the sender.  Pro-YAM, ZCOMM and DSZ versions

beginning with Jan 22 1990 support this.

Specified extra escaped characters: The "zmtxesc" and "zmrxesc" allow

extra characters to be escaped when sending (zmtxesc) or receiving

(zmrxesc).  Zmtxesc is effective sending to any ZMODEM program.  The sz

commands in Omen products after Jan 22 1990 except for DSZ.COM recognize

incoming "zmrxesc" requests.  The zmrxesc and zmtxesc string parameters

consist of one or more printing characters corresponding to the control

characters to be escaped.  To escape NULL:  set zmrxesc "@"

Since DSZ does not support string parameters, a -Q<string> option unique

to DSZ performs the same function as zmrxesc and zmtxesc.  If the

Q<string> option is used in DSZOPT, it must be last.  To escape NULL:

DSZ sz -Q@ file.ext

MobyTurbo(TM):

MobyTurbo optimizes throughput on GIF, ZIP, ARC, and similar files

without sacrificing ZMODEM's famous reliability on noisy lines.

MobyTurbo works with standard modems, error correcting modems with

hardware flow control, and over paths that support ZMODEM and do not

"eat" or otherwise respond to control characters.  This includes all

paths that support YMODEM-g (or would except for line noise).

MobyTurbo can be used for downloads over PC Pursuit if software flow

control is not turned on.

Activate MobyTurbo with the -m option to one or more of: sz command, rz

Chapter 21 Pg 71       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

command, or DSZOPT environment variable.

ZMODEM over 7 bit paths:  (Not supported in DSZ.COM)

A pair of ZMODEM-90 programs with 7-bit support operating over a 7 bit

path should detect that fact and switch to one of two 7-bit path

compatible encodings.  For testing, this can be forced with a "-E"

option.

Default 7-bit mode is RLE compression and 8th bit quoting similar to

Kermit but more efficient because not so many control characters are

quoted.  Use this default for text files.  Rubout and all control

characters can be escaped with the -e option.  Individual control

characters can be escaped with "zmtxesc" and "zmrxesc".

Pack-7 instead of quoting is activated with the -P option.  (-P has no

effect on 8 bit paths.) Pack-7 packs 4 bytes into 5 printing characters.

This is more efficient than quoting for files where 25% more 8th bits

appear than the number of characters saved by RLE compression.  (This

statement applies to GIF, ARC, ZOO, and ZIP files.  It may or may not

apply to other binary file formats, depending on the data.) Pack-7 will

match or beat standard Kermit on ZIP files, even when Kermit has use of

all 8 bits.

SUPK412.ZIP 75136 bytes sent at 2400 bps direct connect from an

Everex 1800 AT clone 8mHz/0ws to an 18 mHz Intel 386 motherboard.

8-Bit line

cps     Protocol/command

156     MS Kermit 3.00 to MS Kermit 3.00  (defaults)

182     MS Kermit 3.00 to MS Kermit 3.00  (windows=2 length=2000)

184     MS Kermit 3.00 to Pro-YAM 17.51 (windows=2 length=2000)

231     Pro-YAM 17.51 ZMODEM (defaults)

237     Pro-YAM 17.51 ZMODEM-90(TM) MobyTurbo(TM) (sz -m)

7-Bit line

cps     Protocol/command

156     MS Kermit 3.00 to MS Kermit 3.00  (windows=4 length=2000)

190     Pro-YAM 17.51 ZMODEM-90 PACK-7 (sz -EP)

Kermit transfers used 8 bit CRC, ZMODEM used 32 bit CRC.

************************************************************************

************************************************************************

WHAT TO LOOK FOR:

Performance     difference     with     MobyTurbo,     possible     side

effects/incompatibilities.   MobyTurbo  logic  tests  for  software flow

control incompatibilities: when does this test fail?

Chapter 21 Pg 72       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

Automatic detection of 7-bit paths.  Spurious detection of  7-bit  paths

and failure to detect 7-bit paths.  (DSZ.EXE only)

12-13-89  The Virus check was removed because of user confusion and slow

startup under certain conditions.  Code has been added to prevent and/or

recover from "line status register 18" errors.

10-28-89 A T may be appended to DSZOPT or "DSZ t  -T"  may  be  used  to

force  DSZ  to  exit  after  it has recognized a ZMODEM AutoDownload(TM)

request.

10-24-89  BBS registered copies (serial  numbers  8000-8999  and  20000-

23999) are now fully featured.

The BBS registration form has been  modified.   Please  read  this  form

carefully  before  applying  a  BBS  serial  number.   Use of BBS serial

numbers (4000, 20000 series) acknowledges acceptance of this contract.

The IMP/KMD compatibility file length information in the  YMODEM  header

generated by the sb command now uses the correct byte ordering.  Changes

to the XMODEM/YMODEM sz/sb commands were made  to  accommodate  programs

that ise CK or CKL.  (As predicted in 1985, networks sometimes interfere

with these sequences.)

Files passed over because of creation errors at the  receiving  end  are

now  counted  by the "sz" command as errors.  The receiving program must

be a 9/89 or later version for this to be effective.

The specified directory form of the rz and rb commands has been  enanced

to accommodate certain forms of erroneous syntax.

6-19-89 A documentation error (concerning DSZPORT) was corrected in  the

DOC file.

5-25-89  Corrects a protocol negotiation problem encountered when the rz

or sz -e option is used.

Host programs can now specify an alternate CANCEL sequence (not used for

DOS and Unix).

The ZMODEM "Slow Send" mode inserts a brief pause after  each  subpacket

when  invoked with the -F option, or automatically when the "sz" command

detects failing flow control.  The zmodem F numeric  parameter  (default

1) sets the pause in tenths of seconds.

4-24-89 The -rr option has been improved.  Long file compares  are  done

in  two  stages  to  save  time  when  the  first portions do not match.

Diagnostic code and undocumented commands have been removed to  decrease

the program size.

Chapter 21 Pg 73       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

The 4-18 version introduces ZMODEM variable length headers of up  to  16

bytes  information  field instead of the previous fixed 4.  Selection of

variable length header capability is automatic and downward  compatible.

Variable   length   headers   allow   future   extensions   to  ZMODEM's

capabilities.

ZMODEM Crash Recovery now understands the  -rr  option  to  compare  the

CRC-32   of  the  assumed  common  file  segment  before  continuing  an

incomplete file transfer.  The "zmodem R" numeric parameter may  be  set

to limit the number of bytes in the files that are compared; the default

R value of 0 allows all of the file to be compared, an  R  value  up  to

32000  (up  to 2 billion on 32 bit flavors) limits the comparison to the

first R bytes of the two files.  The ZMODEM -rr option is  supported  by

Omen Tech versions April 18 1989 and later.

When ZMODEM Crash Recovery detects incompatible source  and  destination

file  contents,  the  destination  file  is now truncated to zero length

before proceeding  with  the  transfer.   Crash  Recovery  requires  the

destination file to be no longer than the source file.

Error reporting on unregistered copies has been enhanced  to  point  the

user to solutions described in the relevant chapters in DSZ.DOC.

3-18-89 In order to work around a bug in the  new  TCOMM  bbs,  DSZ  now

defaults  to  "handshake  on"  if the speed is greater than 2400 and the

clear to send signal is high when DSZ selects the comm port.

The "o" command has been added to disconnect the modem after a  terminal

session:  "DSZ  t  o" disconnects the modem when the terminal session is

completed.

The "P" (capital P) numeric parameter (q.v.) has been added  to  display

and control the UART fifo on some clone chips.

BBS registrations postmarked after Feb 28 1989 must be accompanied by  a

paid  DSZ  registration.  The free program expired Feb 28 1989.  Outside

help has been hired to process and verify  applications  received  under

that program.

2-23-89 The term function now sends BREAK in response to the ALT-B  key.

Handling  of  the  Ctrl-Break  vector  has  been  changed so an untimely

keyboarded Ctrl-C or Ctrl-Break will not hang DOS.

2-15-89  DIRRX is enabled for BBS registrations (8xxx  numbers).   Logic

has  been  added  to  simulate  transmit  interrupts  on chips that have

problems in this area.  The "A" command does not  require  registration.

An  interaction between ONAME= or PREFIX= and the file count display has

been removed.  Time/date conversion routines have been enhanced to  deal

with  files with missing dates.  The "v" command has been added to allow

bbs programs to interrogate DSZ's serial number.

Chapter 21 Pg 74       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

2-06-89 A spurious error message at the completion of  YMODEM  downloads

with  a  download  directory  specified  on  the  command  line has been

eliminated.

2-05-89 DSZ now returns a non 0 status unless at least one file has been

transferred.    An   inappropriate   pathname/disk  specifier  check  on

specified download directories has been removed.

Previous versions:

SlugBait logic has been simplified, reporting any failed  file  transfer

where any of the last 1K of the file was sent with Q status.

Introduced ZMODEM RLE compression with the -Z option.   (registered  DSZ

copies  also  accept "Z" in the DSZOPT environment variable.) ZMODEM RLE

compression packs runs of the same character in two or  three  character

sequences.    Compression  is  useful  for  screen  dumps,  uncompressed

workspaces, compiler listings, and other  files  rich  with  strings  of

repeated  characters,  but  not  .GIF,  .ARC,  .ZOO and other compressed

files.  Compression percentages range  from  -1  per  cent  for  already

compressed  files  to  more  than  1000  per cent (10x speedup!) for the

Personal Computing text file benchmark(!).   When  sending  to  programs

that  do  not support ZMODEM compression, the -Z option is ignored.  The

-Z option is effective when specified to the sending program only.

"Handshake slow" has been added to disable the RTS signal to  the  modem

while writing to disk, even if the interrupt buffer is not chock full of

bytes.  This should help systems with  excessive  interrupt  latency  on

disk writes.

Setting the v (verbose) parameter to  1  or  greater  causes  serialized

copies to ring the bell when a transfer is done.

DSZ.DOC has been extensively rewritten and  reorganized.   DSZ  supports

the  advanced  buffering of the National NS16550AN, NS16550AFN and Intel

82510 serial interface devices.   Please  refer  to  the  subchapter  on

"Brain Damaged UARTs".

Chapter 22 Pg 75       DSZ Reference Rev Jun-7-95         Typeset 6-7-95

22.  ACKNOWLEDGEMENTS

32 bit CRC code courtesy Gary S. Brown.

ZMODEM-90(TM), MobyTurbo(TM), The High Reliability Software(TM), Demand

Upgrade(TM), TurboLearn Script Writer(TM), Cybernetic Data Recovery(TM),

ZMODEM AutoDownload(TM), Intelligent Crash Recovery(TM), Error

Containment(TM), Full Time Capture(TM), True YMODEM(TM),

OverThruster(TM), Password Guardian(TM), CryptoScript(TM), and

TurboDial(TM) are Omen Technology trademarks.

PowerCom was trademarked in 1985 by POLYTRON Corporation, Hillsboro

Oregon.

True YMODEM(TM) is an Omen Technology Trademark signifying YMODEM

protocol support that meets the full YMODEM specification.

DSZ and this documentation are Copyright 1991 by Omen Technology Inc,

all rights reserved.  DSZ is not and never has been Public Domain.

Every version of DSZ has included a Copyright Notice and a prohibition

against sale or modification.  This prohibition applies to any theft of

the intellectual property and trade secrets contained in DSZ by

decompilation, disassembly, or any other form of reverse engineering

including but not limited to profiling, tracing, data analysis, or

monitoring the operation of DSZ.

Omen Technology copyright applies to its expression of the encoding and

compression ideas expressed as code and byte streams used in the

ZMODEM-90 enhancements.

DSZ is not and never has been Copy Protected.  "DSZ unprotect" files

have no function whatsoever except to aid and encourage the theft of

Omen Technology's Intellectual Property.

Omen Technology Incorporated

The High Reliability Software

17505-V Northwest Sauvie Island Road

Portland Oregon 97231

Modem: 503-621-3746 Speed 1200,2400,V.32

FAX: 503-621-3735

CompuServe:70007,2304  GEnie:CAF   BIX:cforsberg