This file is part of ROMDump, a ROM dump utilty for the HP48sx Calculator.

Copyright (C) 1994  Eddie C. Dost  (ecd@dressler.de)

This program is free software; you can redistribute it and/or modify

it under the terms of the GNU General Public License as published by

the Free Software Foundation; either version 2 of the License, or

(at your option) any later version.

This program is distributed in the hope that it will be useful,

but WITHOUT ANY WARRANTY; without even the implied warranty of

MERCHANTABILITY or FITNESS FOR A PARTICULAR PURPOSE.  See the

GNU General Public License for more details.

You should have received a copy of the GNU General Public License

along with this program; if not, write to the Free Software

Foundation, Inc., 675 Mass Ave, Cambridge, MA 02139, USA.

This is ROMDump, a utility to dump the ROM of ANY HP48

======================================================

Usage:	(2:hxs, 1:hxs --> ) pop start address from level 2,

pop end address from level 1, and dump

ROM to the serial at 9600 baud using

lines of form "aaaaa:hhhhhhhhhhhhhhhh\n\r"

with 16 nibbles ROM data hhhhhhhhhhhhhhhh

at address aaaaa.

- Download the file ROMDump to your HP.

- To capture a complete ROM, start kermit on your computer,

set the line so that it fits your HP (use speed == 9600)

and type 'log session', then 'connect'.

- On your HP48 S/SX type '#0h #7FFF0h ROMDump' or on the G/GX

'#0h #FFFF0h ROMDump'. This will give you a complete ROM dump.

This will take about 15 minutes on the S/SX, 30 minutes on the G/GX.

- When done, type the kermit-Escape (usually CTRL-\) followed

by 'C' on your Computer. Say 'quit' to the kermit.

- Your ROM should now be in the file 'session.log'.

- To dump a certain range of ROM, enter the start-address in hex

and the end address in hex, and press 'ROMDump'.

(For example: '#7FFF0h #7FFFFh' ROMDump will output

"7FFF0:84054383D21467BC\n\r" on a Version A S/SX.)

I have included the assembler source (class version) for those

interested in fiddling with the hidden ROM...

Eddie C. Dost  (ecd@dressler.de)