---------------------------------------------------------------

LX-TEMPERATURE

Version 1.0 (March 2001)

(c) Stephan Lttjohann, Emmerich, Germany

Luettjohann@gmx.de

www.eddy.uni-duisburg.de/stephan

portions copyright, 1995, the PAL group

---------------------------------------------------------------

LX-Temperature is a program that controls a little thermometer

which can directly be plugged into the serial port of the

palmtop. It interprets measurement data coming from the

thermometer and calculates the actual temperature using the

data received from the thermometer.

If you are interested in how to built such a little thermometer

and how it works, have a look at my homepage.

LX-Temperature is freeware, so you may use it, copy it and give

it to friends, as long as all files included in the zip-file

are kept together.

Nevertheless I would be very happy to receive an email if you

use my program. Please let me know what you think about it.

WHAT FILES ARE INCLUDED ?

-------------------------

batt.lxt

calibr.exe

gnuplot.bat

lxtemp.cfg

lxtemp.exe

lxtemp.icn

lxtempab.exe

lxtempab.icn

temp.lxt

thermo.gnu

thermo.lxt

readme.doc

CALIBR.EXE

-----------

The calculation of the temperature mainly bases on two

parameters, which are called m and b in the lxtemp.cfg

configuration file. These parameter values depend on the

hardware setup of the thermometer as well as of the speed of

your palmtop and the speed of the program. If you want to know

more about these values, have a look at the theory of the

thermometer on my homepage.

To calibrate your thermometer, you should start lxtemp.exe and

choose calibration mode in the config dialog. Then take a

reference thermometer, e.g. a Hg-Thermometer, and measure by

pressing NOW two different temperatures which are far enough

away from each other. By pressing now at a certain temperature

in calibration mode, lxtemp will not show you a temperature,

but will show you a certain time. Write down the temperature of

your reference thermometer and the time shown by lxtemp. Then

do it with another temperature, which is not close to your

first temperature. After having done so, you have two

temperature values and two time values. Quit lxtemp and start

calibr.exe. Then you are quested to type in your two

temperatures and the two times. Doing so, calibr.exe will give

you values for m and b, which you can edit in lxtemp.cfg.

Note that calibration is not so easy. When you change your

thermometer to another temperature, make sure that you waited

long enough, so that the thermometer got to the same

temperature the surrounding air has. Press several times on

NOW, to see if the displayed time changes or stays stabile.

Probably you should press NOW at a certain temperature

several times and calculate the middle of all displayed time

values.

GNUPLOT.BAT

-----------

Using lxtemp you are able to measure a series of temperatures.

These temperature values can be written into a specified file

and if you want, they can be graphically displayed after the

measurement by gnuplot. Gnuplot.bat contains the command-line

of gnuplot. If you have gnuplot, you can edit gnuplot.bat to

let lxtemp know how to access your gnuplot. Note that the

gnuplot command line is followed by thermo.gnu. Thermo.gnu will

be generated by lxtemp and will tell gnuplot which file to

plot.

E.g:

a:\gnuplot\gnuplot.exe thermo.gnu

where thermo.gnu contains:

plot "temperat.dat" w l

wait

LXTEMP.CFG

----------

Lxtemp.cfg is the configuration file of lxtemp.exe and

lxtempab.exe. Some of the values in lxtemp.cfg can also be

changed by the config dialog of lxtemp.exe. The fundamental

parameters m and b, for example, can only be changed by editing

the lxtemp.cfg file.

LXTEMP.EXE

----------

Lxtemp.exe is the main program. Using this program, you can

measure the temperature by pressing NOW or measure a series by

pressing SERIES. How the series should be measured can be

specified by pressing CONFIG.

In the config dialog you can edit the number of measurements in

a series and the delay, i.e. the time in SECONDS between two

measurements. Note that the delay time does not include the

time of the measurement, only the time between two

measurements.

You can also specify the filename in which the data will be

saved, if gnuplot will be executed after the measurement of

the complete series and if calibration mode is active or not.

Dimension specifies, if the temperature is written to file in

F or in K.

LXTEMPAB.EXE

------------

Lxtempab.exe means "lxtemp in appointment book mode". If you

would like to measure the temperature e.g. every 30 minutes, it

would not be clever to use lxtemp and set the delay time to

30*60s=1800s, because in these 1800 seconds, the palmtop uses

energie or the palmtop turns off and will not turn on again for

the next measurement.

But to measure the temperature e.g. every 30 minutes, you can

use lxtempab.exe: Install lxtempab.exe under more applications

in system manager. Then define a macro that can start

lxtemp.exe. Now you can use your appointment book to execute

the new macro at certain times. Everytime the macro and

therefore lxtempab.exe is executed, lxtempab.exe measures a

temperature and stores it into the file which is specified as

Datafile in lxtemp.cfg or in CONFIG in the lxtemp.exe program.

Note that lxtempab.exe always appends the new temperature to

the data file. So, if you want to do a complete new series of

measurements, make sure that the datafile does not exist (then it

will be generated in the first measurement) or is empty.

Lxtempab.exe uses the same configuration values as lxtemp.exe

does. These values are dimension, datafile and the parameters m

and b, of course. The other configuration values are not used,

because delay e.g. is now handled by your appointment book

itself.

NOTE ABOUT MEASUREMENTS

-----------------------

As LX-Temperature also has to determine the actual voltage of

your palmtop to calculate the temperature, measurements under

1.57V of main battery are not so reliable. Nor should you

measure the temperature if your AC-adapter is plugged in or has

just been plugged in, because charging your palmtop heats the

serial port and therefore the thermometer.

If the measurement does not work, it could be that the serial

port is not turned on.

type serctl /w at the dos promt.

---------------------------------------------------------------

DISCLAIMER

---------------------------------------------------------------

This software is copyrighted freeware.

The author disclaims all warranties, expressed or implied.

The author assumes no liability for damages, direct or

consequential, which may result from the use of this software

and/or a thermometer built by the author's or anybody other's

instructions.

---------------------------------------------------------------