This is  a modified  version of the device driver DRIVER.SYS that

comes with DOS 3.2.  It works  exactly the  same, except  that it

will change  the step  rate to  the correct rate for 3.5" drives.

When it is accessed, it changes the step rate to 15, which is the

value that  works best  for my 3.5" drive. The last thing it does

is to set the step rate back to its original value. This  way the

step  rate  will  always  be  correct for the drive that is being

accessed.

Here are a few examples  of  CONFIG.SYS  files.  I  show  the -d1

option for  JRCONFIG for  clarity, it is the default value and is

not needed. You will also notice that I  use JRCONFIG  to set the

number of  drives to  the number of drives that are the same type

as the A: drive. The others are accessed thru the device drivers.

You will also notice that I frequently refer to two drive letters

for one physical drive. DOS will keep the drive letters straight,

and prompt you to change the disks when necessary.

Ex 1:

device=jrconfig.nrd -d1 -x

device=driver35.sys /d:1

device=lowrd.sys

This is for a 5.25" drive as A: and B:, a 3.5" drive as C:, and a

ramdisk as D:

Ex 2:

device=jrconfig.nrd -d1 -x

device=driver35.sys /d:1

device=driver35.sys /d:1

device=lowrd.sys

This is for a 5.25" drive as A: and B:,  a 3.5"  drive as  C: and

D:, and a ramdisk as E:

Ex 3:

device=jrconfig.nrd -d2 -x

device=lowrd.sys

device=driver35.sys /d:2

This is  for 2  5.25" drives as A: and B:, a ramdisk as C:, and a

3.5" drive as D:.

Ex 4:

device=jrconfig.nrd -d1 -x15

drivparm=/d:0

device=driver35.sys /d:1

device=lowrd.sys

device=driver35.sys /d:1

This is for a 3.5" drive as A: and B:,  a 5.25"  drive as  C: and

E:,  and  a  ramdisk  as  D:. The environment variable "drivparm"

tells DOS that the 0 drive (ie, the first drive) is a 720K drive.

There is a problem with the last example.  While the  -x15 option

will set  the proper  step rate for the 3.5" drive A: and B:, the

5.25" drive C: and E: will have the  wrong step  rate. To correct

this, here  is a DEBUG session that will change the step rate set

by DRIVER35.SYS, you may want to change the name to DRIVR525.SYS.

A>debug driver35.sys

-e14c

xxxx:014C  FF:df     (here you change the step rate from F to D)

-w

Writing 0497 bytes

-q

In case you are wondering about  example 4,  a 3.5"  drive can be

put inside the jr, and used as the boot disk.

If you  have any  questions, you  can leave  me mail on the Metro

PCjr BBS  301-468-0984.

Larry Newcomb