The file contains Frequently Asked Questions useful for Troubleshooting

Table of Contents:

SECTION 1 - GENERAL QUESTIONS

-No Sound

-No Sound for an attached CD-ROM

-Resolving conflicts with multi-serial cards

-Connecting two joysticks

-Changing Interrupt Levels

SECTION 2 - DOS

-DOS games with General Midi

-DOS games, problem running

SECTION 3 - WINDOWS

-Windows startup "Initialization" error

-Changes made to windows environment

-Changes for Band-In-A-Box

-Win95, sounds repeat continuously

SECTION 4 - OS2

-OS/2 startup '**RM:IRQ7**' error

-MIDI:2 does not play any sounds

-MIDI:2 WaveTable, adjusting the volume

SECTION 1 - GENERAL QUESTIONS

Q1  No sound

A1  Check your computer's configuration (using the Reference Disk) and

be sure the "Parallel Port Arbitration Level" is "Disabled".  This

procedure is detailed in the sound card's manual.

Be sure you have your speakers/headset plugged into the proper jack.

The connector arrangements are:

Audio In  (3.5mm stereo)

Mic   In  (3.5mm stereo or mono)

Audio Out (3.5mm stereo)

Gameport  (DB15F)

If the software you are using controls the sound volume, make sure

it is adjusted to a level that can be heard.

Q2  No Sound from a CD-ROM drive

A2  CD-ROM drives generally have three cables attached to them

(1). Power cable

(2). Control cable (usually SCSI for PS/2)

(3). Audio cable

FOR EXTERNAL CD-ROM

Make sure you have an audio cable from the external CD-ROM drive

to the sound card.  If you are using the CD-ROM headphone jack,

be sure the volume control on the CD-ROM headphone jack is turned

up.  Since the CD-ROM drive is external, you must be using the

"Audio Line In" jack of the sound card to get the sound from the

CD-ROM drive into the sound card.  Make sure the volume setting

for the LINE in the mixer is set to a level that can be heard.

To set the volume in Windows, use the Windows mixer application

in the sound card's folder.  To set the volume in DOS or OS/2

use the DOSMIXER program with the -Lxx option, where xx is the

volume setting from 0 (minimum) to 15 (maximum).

FOR INTERNAL CD-ROM

Make sure you have an audio cable from the internal CD-ROM drive

to the sound card.  The internal cable should connect to one

of the sound card's CD-ROM connectors.  Make sure the volume

setting for the CD-ROM in the mixer is set to a level that can

be heard.  To set the volume in Windows, use the Windows mixer

application in the sound card's folder.  To set the volume in

DOS or OS/2 use the command line DOSMIXER with the -Cxx option,

where xx is the volume setting from 0 (minimum) to 15 (maximum).

Q3  Some Async/Parallel cards do not fully decode their address (improper

design) and can cause conflicts with the sound card resulting

in no sound being hear or sparatic playback.

A3  IBM defines Serial 1 - Serial 8 and LPT as follows:

DEVICE                  ADDRESS        IRQ

-------------------------------------------

Serial 1 (Com 1)        3F8 - 388       4

Serial 2 (Com 2)        2F8 - 2FF       3

Com 3         3E8 - 3EF       4

Com 4         2E8 - 2EF       3

Serial 3                3220-3227       4

Serial 4                3228-322F       3

Serial 5                4220-4227       4

Serial 6                4228-422F       3

Serial 7                5220-5227       4

Serial 8                5228-522F       3

LPT1                    3BC-3BF         7

LPT2                    378-37B         5

LPT3                    278-27B         7

Some "Dual Async Parallel Adapter/2" are designed improperly

and do not do a full decode of the address space.  For example,

if you set the Async card up for Serial 5 it should use address

4220-4227.  The Async card does not decode the upper address, so

instead of responding to 4220-4227, the Async card responds to

x220-x227 (x=anything).  This conflicts with the sound card that

uses 0220-0227.  This conflict may not show up in the system

configuration using the Reference Disk for the computer.

Solution is to use COM1 - COM4 only, then a conflict cannot occur.

Q4  How do I connect two joysticks

A4  You will need to purchase a MIDI/Joystick breakout cable and a

Joystick "Y" cable.  The Game port on the sound card has both

MIDI and Joystick signals.  The MIDI/Joystick breakout cable will

split the signals into MIDI-IN, MIDI-OUT, and JOYSTICK.  Then

the Joystick "Y" cable will allow two joysticks to operate.

Q5  Changing the Interrupt Level of the Sound Card.

(Normally fixes Parallel port devices conflict with the sound card)

A5  These conflicts relate to interrupt sharing.  Micro Channel and

PCI allowing sharing of interrupts (ISA do not).  PS/2 systems use

IRQ 7 for the parallel port, shared with the sound card.  Some software

drivers assume an ISA bus and do not allow sharing.  You may

have to determine which parallel port device drivers are causing

the problem and only enable these devices when needed.

To try and resolve conflicts with the parallel port, you should

first read the sections in this file for your particular operating

system and see if any of the suggestions will help.

If none of the suggestions will help, you can check to see if

your sound card configuration will support another interrupt.

To do this, boot your computer from the sound card's DISK-1.

A message will appear on the screen indicating the address, dma,

and interrupt channels.  If interrupt 5 and 7 are indicated then

the sound card can use 5 and 7, otherwise the sound card can only

use interrupt 7.  If the sound card only supports interrupt 7 then

you must resolve the problem with the parallel port device drivers.

If the sound card supports 5 and 7, then you can try using interrupt

5 for the sound card.

Only make the following changes if your sound card supports 5 and 7.

To use IRQ-5, you need to make the following modifications:

a. Change @5150.adf file replacing "int 7" with "int 5"

(you must re-configure your system after updating the ADF)

b. DOS AUTOEXEC.BAT, change DOSMIXER statement from "-7" to "-5"

c. Win31 SYSTEM.INI, change [auddrive.drv] from "int=7" to "int=5"

d. OS2 CONFIG.SYS, change device=ES1688DD.SYS from "/I:7" to "/I:5"

e. Win95, use device manager to change interrupt from 7 to 5

f. WinNT, re-install and select interrupt 5

SECTION 2 - DOS

Q1  DOS game does not work with General Midi

A1  Run "DOSMIXER -M15 -W15" before the game.  When the sound card

powers up, the MPU401 port is disabled.  "DOSMIXER" enables the

General Midi MPU401 port.  The -M15 and -W15 set the Master

volume control to 15 (maximum) and the WaveTable volume to

15 (maximum).  In the game setup be sure to select

"General Midi" at 330 for the sound music.  You will need the

sound card's WaveTable (standard on Sound-32, option to Sound-16).

Q2  DOS game does not run

A2  Run "DOSMIXER -G".  Some DOS games, such as DOOM and Heretic

require the sound card to be initialized to a particular mode.

The "DOSMIXER -G" performs this initialization and should be

run everytime before playing the game or performing setup for

the game.  Other games, such as RISE OF THE TRIAD do not require

the "-G" option.  If a particular game is giving you problems,

try the "-G" option.  If a game requires the DOSMIXER -G, create

a BAT file that automatically runs the DOSMIXER then the game.

The DOSMIXER does not stay resident in memory, it simply initializes

the sound card to a mode desired by the game.  Read the file

DOSMIXER.DOC (copied into your sound card's directory

when you install DOS support) for more detail.

SECTION 3 - WINDOWS

Q1  Windows startup error

ES1688WN.386 Initialization Error

Unable to loacate a useable IRQ channel

A1  Another device driver is loading that uses IRQ7

Most likely a left over device driver from another sound card

or soundblaster drivers.  Look for vspd.386 or vsbpd.386 and

REM out these lines.

Q2  What Changes does the Windows install make, and how can

I check to see if the software installed properly?

A2  After installing windows support, the file BTINSTALL.LOG

is created in the sound cards directory.  This "LOG" file

contains a complete log of all the changes made to your

windows files to install sound support.  The LOG file

will indicate if a particular part of the installation

was successful.

Q3 Changes for Band-in-a-Box

A3 The standard install of the windows drivers is good for

most applications.  Band-in-a-box uses interrupts for MIDI.

The following changes are to add interrupts for MIDI.

(1). Edit SYSTEM.INI file, [DRIVERS] section

Change: 'MIDI1=AUDMPIO.DRV'

To: 'MIDI1=AUDMPU.DRV'

(2). Edit SYSTEM.INI file, [AUDDRIVE.SYS] section

Change: 'MPU401=NO'

To: 'MPU401=YES'

Change: 'INTMPU401=7'

To: 'INTMPU401=5'

Note: These changes assumes the sound card uses interrupt 7 (default)

In this case, INT=7 and INTMPU401=5

If you change the sound card to use interrupt 5, then

In this case, INT=5 and INTMPU401=7

Refer to SECTION-1 of this readme for changing interrupts.

Q4 After loading Win95 drivers, you restart your computer and you get a continuous

repeating sound.

A4 Generally this is due to you not loading the DOS support under Win95.  Refer

to WIN95.DOC file.

SECTION 4 - OS2

Q1  OS/2 startup error

**RM:IRQ7**

SYS1201 The device driver C:\mmos2\ES1688DD.SYS specified in the device

statement on line xx of the CONFIG.SYS file was not installed. Line xx

is ignored.

Press Enter to continue...

A1  If Parallel Port Arbitration is Enabled, you must Disable it.

Use your computer's Reference Disk, Go into the system configuration

and disable Parallel Port Arbitration.  See sound card's manual for

more detail.

Also check to see if you are using Interrupts for printing.

Look for the OS/2 CONFIG.SYS statement:

Basedev=print02.sys /IRQ

Change the statement to:

Basedev=print02.sys

The /IRQ parameter enables interrupts for printing.  Using interrupts

for printing is not required for OS/2 and the OS/2 print device driver

using interrupts, conflicts with the sound card device driver.

Q2  OS/2 MIDI:2 player does not play any sounds

A2  The OS/2 MIDI:2 player in the MultiMedia folder is for playing

Midi songs using the WaveTable music synthesizer.  You must

have the WaveTable option to hear any music from the MIDI:2

The WaveTable is included on the Sound-32 card and is an

upgrade to the Sound-16 card.

Q3  How do I adjust the volume level of MIDI:2 WaveTable

A3  Go into a DOS session and use DOSMIXER

to adjust the sound level of the MIDI:2 with WaveTable.  The

format is

DOSMIXER - Wxx

W stands for Wavetable

xx is the volume setting from 0-15

(0=quite,  15=loud)

For Example,

DOSMIXER -W5

Sets WaveTable volume to maximum

DOSMIXER -W7

Sets WaveTable volume to about half maximum

The adjusting of the WaveTable sound volume using the MIDI:2

player (much like the MIDI and the Digital Audio player)

is not yet available.  This feature may be offered in a

software update.  The Master Volume control (icon in MultiMedia)

controls the volume of all the sound sources including the

MIDI:2.