STANDARD MICROSYSTEMS CORPORATION

EtherCard PLUS Family LAN Adapters

For IBM LAN Server NDIS Environments

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1992 Standard Microsystems Corporation.

All rights reserved.

1.  DOS LAN REQUESTER

---------------------

1. At the DOS prompt, insert the SMC(or IBM) LAN Support

Program(LSP) Version 1.30 diskette into your disk drive.

2. Type: dxmaid. Press Enter.

3. Press Enter twice.

The Environment Information screen appears.

4. Specify the following values for the fields on this screen:

Configuration for this computer?                Yes

Use existing configuration information?         No

Do you have adapter option diskettes?           Yes

Are you configuring for two adapters?           No

Source for LSP                                  A:\

Target for new configuration                    C:\lsp

5. Press Enter to store your choices.

The program prompts you to install the option diskette.

6. Install the SuperDisk Drivers diskette in your source drive

and specify the path: A:\NDIS. Then press Enter.

The necessary files are copied from the SuperDisk to your

target directory on the hard disk.

7. Press Enter three times.

8. Insert the LSP diskette and press Enter.

The Current Configuration screen appears with the SMC EtherCard

PLUS adapter and protocols already selected ("IBM IEEE 802.2"

and "IBM OS/2 NETBIOS").

9. Press F4. LSP is installed.

10. Use a text editor to edit the CONFIG.SYS file. Specify at least

20 files in the FILES= line, like this:

files=20

Once you have saved the CONFIG.SYS file, reset the computer.

11. Insert the DOS LAN Requester Install Diskette.

12. Type: install. Press Enter.

13. Press Enter twice more.

The program prompts you for the pathname of the DOS LAN

Requester directory.

14. Specify the pathname for the DOS LAN Requester directory.

(C:\DOSLAN is the default.)  Press Enter.

The program asks you which services you require.

15. Highlight either the "Send messages" or "Send, View, Edit, and

Log messages" option. (Normally, you select the second option.)

Press Enter to confirm your choice.

16. Specify the machine's ID and the domain ID. The domain ID must

be the domain server ID and must be the same for all

workstations on the network.

17. Follow the prompts to complete the installation.

2.   OS/2 LAN REQUESTER

-----------------------

1. Insert the SMC(or IBM) LAN Adapter Protocol Support(LAPS) Version

1.0 diskette into your disk drive.

2. Type: LAPS and press Enter.

The IBM logo appears.

3. Select the INSTALL button.

4. Specify the target drive (usually C:\) and press Enter and OK.

5. Select the Configure button.

6. Select the "Configure LAN transports" and press the Continue

button.

The Configure Workstation screen appears.

7. From the Network Adapters window select the "SMC Ethernet

Adapter for OS/2" and press the Add button.

This adapter name appears in the Current Configuration window.

8. From the Protocols window select both "IBM IEEE 802.2" and

"IBM OS/2 NETBIOS" options. (You may also need to select

the "IBM NetWare Requester" option depending on your network.)

Press the Add button after each selection and that option appears

in the Current Configuration window below the chosen driver.

9. Select OK.

10. Select Exit.

The CONFIG.SYS Update screen appears.

11. Specify the pathname where the CONFIG.SYS file resides.

12. Select the Configure button.

13. Press OK.

14. Select Exit.

15. Reset the computer. Use the OS/2 Shutdown process for an orderly

exit from the program.

16. Bring up the computer again, enter OS/2 and insert the OS/2

Requester Install diskette in your diskette drive (A:\).

17. From the OS/2 prompt, type: laninst. Press Enter.

18. Select OK at the IBM logo.

19. Specify the "Advanced" option.

The Install Type screen appears.

20. Specify "Install or Configure this workstation."

21. Designate your fixed disk (usually C:\).

22. Select "Install or remove a component."

23. Highlight all other components in the list you want to install.

Use the arrow keys to choose the component and press the space

bar to highlight it. Press the Install button and the word

"Installed" appears in the Status column. Once you have

selected all the components you require for this installation,

select the OK button.

24. Select the "Configure a component" option and press OK.

25. Select a component to configure from the list and press the

Configure button.

26. Specify the requester name and the domain name for that

component. Press OK.

27. If you want the server to start automatically when one of its

workstations is started, select YES.

28. Select the requester services you require and press OK. (Choose

either "Messenger" or "Message pop-up.")

The Configure screen reappears with the word "Installed"

displayed in the Status column next to the selected component.

29. Continue to configure each component in the list until you are

finished.

30. Select the "LAN Adapter and Protocol Support" option.

The Options screen appears.

31. Specify "Copy additional network adapter drivers" and press OK.

32. Specify the path for the optional driver diskette: A:\NDIS.

33. Insert the SuperDisk Drivers diskette in your A:\ drive and

press OK.

34. Select the "Configure workstation" option.

The Configure Workstation screen appears.

35. From the Network Adapter window, select the adapter name

appropriate for your adapter:

"Standard Microsystems EtherCard PLUS Micro Channel"

"Standard Microsystems EtherCard PLUS Software"

"Standard Microsystems EtherCard PLUS Jumpered"

36. Press the Add button and your choice appears in the Current

Configuration window.

37. From the Protocols window, select the protocol options you

require. These usually include "IBM IEEE 802.2" and "IBM

OS/2 NETBIOS." (You might also select the "IBM NetWare

Requester" option depending on your requirements.)

Press the Add button after each of your selections and it

appears below your adapter selection in the Current

Configuration window.

38. Press OK.

39. Select the "Configuration complete" option.

40. Select the LAN adapter this configuration applies to. If there

is only one adapter in this computer, select "Adapter 0" and

press OK.

41. Press OK at the Configure screen.

42. Select the "Apply the changes" option and press OK.

43. Follow the prompts to copy all files from the Requester diskettes

to your hard disk.

3.  OS/2 LAN SERVER INSTRUCTIONS

--------------------------------

The following section provides you with instructions for

installation of LAN Server on an OS/2 server. This includes

instructions on modifying the CONFIG.SYS and PROTOCOL.INI files to

configure the specific driver.

1. Load OS/2 according to your OS/2 Installation manual.

2. Insert the LAN Server #1 diskette into your A:\ drive.

3. Open an OS/2 window. At the [A:\] prompt type: laninst. Press

Enter.

The IBM logo appears.

4. Select OK or press Enter.  The Installation/Configuration

screen appears.

5. Select the "Advanced" option.

The Install Type screen appears.

6. Select the "Install or configure this workstation" option.

The Server Type screen appears.

7. Select the "Domain Controller" option if this is the only

server in the network or the dominant server in a multi-server

network.

Select the "Additional server" option if there is already

a domain controller in the network.

8. Press the OK button.

The Installation and Configuration screen reappears.

9. Select the "Install or remove a component" option and press OK.

The Advanced Server Installation/Configuration screen appears.

10. Highlight each component you want installed using the arrow

keys and press the space bar.  The Install button is activated

under the Component window.  Press the Install button.  The

word "Install" appears in the Action column.

Note: Several components are automatically selected

for installation when this screen appears. The word

"Install" or "Install (required)" appears in the

Action column of each of these components. These

components are required elements for installation of

this driver.

(To deselect a selected component, highlight the component,

press the space bar, then choose the Cancel Action button.

The word "Install" disappears from the Action column.)

11. When you have specified all the components you wish to

install, select the OK button underneath the Component

window.

The Installation and Configuration window reappears with

the "Configure a component" option highlighted.

12. Press OK. The Configure menu appears.

This menu lists the components and their status. You

must configure each of the components specified as

"required" in the Status column.

13. Select one of the components using your arrow keys and

press the Configure button.

The Server and Domain names window appears.

14. Type in the server name and the domain name you require.

The names must be different and the domain name must be

assigned to all workstations in this network. Press the

OK button to record your choices.

You are asked whether you want to start the server

automatically when the workstation is started.

15. Select Yes.

The Select Server Services screen appears.

16. Select as many of the Server Services items and Requester

Services items as you require. Press OK to record your choices.

You are returned to the Configure screen.

17. Select the "LAN Adapter and Protocol Support" option.

The program prompts you to insert the Network Adapter

Drivers diskette.

18. Insert the LAN Server Network Adapter Drivers diskette

in drive A:\ and press OK.

All appropriate files are copied to the server's hard disk.

The Options screen appears.

19. Select the "Copy additional network adapter drivers"

option and press OK.

20. Insert the SuperDisk Drivers diskette and specify the full path

for the NDIS driver files: A:\NDIS. Press OK twice.

The Options menu appears.

21. Select the "Configure workstation" option.

The File screen appears.

22. Select the "Default Protocol" option.

23. Select "Personal copy from fixed disk or diskette" and type in

the path name: A:\NDIS and press OK.

The Configure Workstation screen appears.

24. Select the appropriate SMC adapter from the list. In this case,

select either "Standard Microsystems EtherCard PLUS Software"

if you have an ISA-bus computer or "Standard Microsystems

EtherCard PLUS Micro Channel" for a Micro Channel computer.

25. Press the Add button, then Yes.

The selected driver appears in the Current Configuration

window.

26. Go to the Protocols window and select the "IBM IEEE 802.2"

option and press the Add button.

27. Select the "IBM NETBIOS" option and press the Add button.

Both protocol selections appear under the adapter selection

in the Current Configuration window.

28. Select the OK button.

29. Select the "Configuration Complete" option.

The LAN Adapters screen appears.

30. Select "Adapter 0" for this configuration. If there is one

adapter in this computer, only the Adapter 0 option will be

available.

If more than one adapter is present in this computer, select

the appropriate adapter number for this computer.

You are returned to the Configure screen.

31. Select OK.

The Installation and Configuration screen appears.

32. Select the "Apply the changes" option and press OK.

33. Follow the prompts to finish this installation.

EtherCard PLUS, TokenCard, and SuperDisk are trademarks, and SMC and

Standard Microsystems are registered trademarks of Standard Microsystems

Corporation. Other trademarks mentioned herein belong to their respective

companies.