COMRESET version 1.0 of 12/3/90

This simple program is used to initialize serial ports on an IBM

PC-compatible computer.  It was designed specifically because Microsoft

Windows 3.0 running in 386 Enhanced mode does not fully reset the COM

ports if you use serial communications applications under Windows.  Run

COMRESET to reinitialize the ports installed in your machine after you

exit Windows, if you are having trouble with communications applications

that normally work correctly but fail after you run Windows on your

system.

List the numbers of the COM ports you want to reset on the COMRESET

command line.  (Generally, you should reset all the ports that are

installed in your system.)  Include only the number, and separate each

with a space.  COMRESET recognizes only ports COM1 through COM4 and will

exit with an error message (and without doing anything) if you specify a

port that is out of range.  If a COM port is specified but does not

actually exist in your system, COMRESET will note it with an error

message, but other existing ports you may have specified will still be

reset.

Example: To reset ports 1, 2 and 4, enter the command:

COMRESET 1 2 4

at the MS-DOS prompt.

COMRESET assumes that the serial ports are at the "standard" addresses

and interrupts for COM1 through COM4:

Device      Port      IRQ

-------------------------

COM1       03f8h        4

COM2       02f8h        3

COM3       03e8h        4

COM4       02e8h        3

Custom versions of the program to address other ports can be produced

on request.

This program is a product of:

FBN Productions

917 W. Columbia Ave.

Champaign, IL 61821

This is a free program, and you may distribute it as you desire.  For

further information or source code, contact FBN Productions at the

address above or through the FBN BBS at (217) 359-2874.