(c) Copyright 1992 Genashor Corp.  All Rights Reserved.

This file describes the SIMIC demonstration files.  Some additional

information can be obtained by reading the "SIMIC Engineering Guide

for IC Design Verification" by Genashor Corp.

Only a small percentage of SIMIC features are described in these

demonstrations.  The "SIMIC User's Guide" by Genashor Corp describes

SIMIC's range of capabilities.

Each Demonstration consists of a network description, in a file with

extension .net.  Most Demonstrations also have a second file, with

extension .run, which contains a number of SIMIC commands to

automatically perform the simulation.  Each .run file contains

commentary to explain each step.

A simulation is run by issuing the command:

simic <command_file>

where <command_file> is the name of the "run" file for that

demonstration.  For example, if a demonstration consisting of two

files: example1.net and example1.run, exercise this demonstration

with the command:

simic example1

Alternatively, the same operation can be accomplished by entering the

command:

simic

SIMIC will issue its prompt, ">>:", requesting interactive input. In

response to this prompt, simply type the command:

execute file=example1

SIMIC, by default, is case insensitive.  The demonstration files use

mixed case only to improve readability.  It is a good idea to make a

hardcopy of each file for viewing while running the demonstrations.

Network Description Overview:

a) Global Delay Table. Delay characteristics are specified

in the !Delay section of the file.  Each delay relation

is assigned a unique name, allowing multiple element

instances to share delay characteristics through symbolic

references.  Delay tables can be designed for different

operating conditions, and a specific table can chosen for

circuit compilation.  Typical tables might include pre-

and post- radiation hardening characteristics, or the

effects of different supply voltages.  Delays can be

specified as absolute values, or delay vs. loading curves.

Delay vs. loading curves may be specified in intercept/slope

format or two point format (delays at two different loading

conditions).  SIMIC adds all loading on each signal (wiring

capacitance and capacitance at all driven load pins), and

extrapolates the delay for this total loading from the

referenced delay curve.  For any element, MINIMUM, TYPICAL

and MAXIMUM values may be specified, and then chosen

and/or scaled interactively.

b) Annotation.  These include !Documentation section(s), which

allow clean addition of large amounts of text, and Comments,

beginning with the Comment keyword (C=), for single line

commentary.  In addition, Remarks (R=) allow information to

be conveyed to the user during circuit compilation (for

example, to warn the user about constraints on using a given

macro (TYPE)).

c) Electrical Characteristics. Specifications such as drive

strength (ODrive) and loading (Ilod).  These and other

characteristics can be viewed in the listing (.lst) file

created by the circuit compiler.

d) Network Topology. The circuit's netlist is specified in the

!Logical section of the file. SIMIC supports a general

hierarchical network description language in which each

macro (subcircuit) is defined by a TYPE statement, which

specifies the macro's input, output, and bidirectional (bus)

ports (and optionally, their electrical characteristics),

followed by PART statements, which specify the macro's

internal elements and their interconnections. The internal

elements can either be SIMIC primitives or instances of

other macros.

See the file snl.txt for descriptions of the topological constructs

and the SIMIC primitives used in the demonstration testcases.

Testcase #1 - The Bus Hold Circuit. (bushold.net and bushold.run)

Run Command Description:

a) Define File=bushold

This command specifies that all files will have the default

name "bushold".  Contents of files are distinguished by the

extension attached to this name.  For example, in this

demonstration, SIMIC will generate a "listing" file of the

compiled circuit, as no name for this file was specified,

the information will be placed in a file named "bushold.lst".

b) Get Type=bushold Report=all

The "Get" command is used to compile/retrieve circuit

descriptions, timing, and loading information.

"Type=bushold" specifies that SIMIC should compile the

circuit named "bushold".  "Report=all" specifies that a

"listing" file should be generated and all information

should be included.  This listing file will contain a

description of the flattened circuit and the electrical

characteristics of each element.

c) Define Wall.4 = 1000 @200 0000 @210 0011 @260 0001

Apply Patterns=Wall

Creates and applies input stimulus to the circuit.

SIMIC has a very flexible input stimulus language.  It

allows hierarchical patterns to be described, containing

loops, absolute and relative positioning of patterns,

stimulus grouping, etc. "Define Wall.4" specifies the

creation of a 4-bit wide pattern sequence named Wall.  The

"W" in Wall specifies that the patterns are temporal (Each

pattern is positioned in time).  This form is the most

frequently used by other simulators, but is only one of

three forms (and typically the least used) in SIMIC.  In

this command, four input states (1000, 0000, 0011, and

0001) are described and  applied at times (0, 200, 210, and

260) respectively.  "Apply Patterns=Wall" attaches this

sequence to the primary inputs (EnA, DataA, EnB, DataB)

respectively (The order defined in the network description).

d) Print List=EnA,DataA,EnB,DataB*Bus*E Change:

No Print Pstep:

Specifies what, when and how to output signal information

during simulation.  The "Print" command is used to output

information to the terminal (stdout).  The "Write" command

is identical to "Print", except this command sends the

output to a file, with default extension ".wrt".  Write

and Print commands operate independently, and may coexist

in the same simulation run.

"List=" indicates that a list of signals to output will

follow.  The asterisks (*) specify that a blank vertical

column should be inserted in order to make the output more

readable.  "Change:" indicates that SIMIC should output

whenever any one of the signals on the list changes state.

"No Print Pstep:" specifies that SIMIC should NOT output

each time the circuit stabilizes (all internal activity

ceases -- this is the default condition causing Print and

Write output).

SIMIC uses a single-character representation for each of the

15 possible combinations of signal strength and value.

You may force SIMIC to display a four character

representation (0, 1, X, and Z), suppressing strength

information by adding the command:

Print Values=Levels

prior to the simulate command.

e) No Warn Unstable:

SIMIC will, by default, issue a warning message if the

circuit leaves fundamental mode of operation (the circuit

is still responding to an input change when a new input

change occurs).  Since loss of fundamental mode of operation

is not a problem for this  circuit, the warning message is

not meaningful here.

f) Simulate

This tells SIMIC to perform the simulation.

g) Quit

This ends the SIMIC session.

Testcase #2 - Paralleled Devices. (parallel.net)

This demonstrates SIMIC's ability to merge electrical characteristics

of simple combinatorial gates and switches.  In this circuit two

buffers are placed in parallel.  This demonstration will also walk

you through a totally interactive session in SIMIC.

a) Enter SIMIC.

Do this with the command:

simic

at your system's prompt.  The SIMIC session should begin

and SIMIC should issue the prompt:

>>:

indicating that SIMIC is requesting command input.

b) Specify a default file name.  File names for SIMIC input

or output are then derived from this name, by default.

If not specified, the default name is "noname".  In this

example, use the name "parallel":

define file=parallel

If you mistyped this command, just reenter it with the

correct value to replace the mistyped one.

c) Compile the circuit description.  We also would like a

file containing the flattened circuit description and its

simulation properties.  This is done with the command:

get type=parallel report=all

We could quit this session and look at the listing file,

parallel.lst, to see the resulting information.  We can

also interrogate the circuit properties of interest

interactively.  Examples are:

?delay list:

which will display the rise and fall values for all signals

and:

?loading list:

which will display the loading information for all signals.

You will note that the AND gate generating signal C has

larger delays than the equivalent ANDs that are paralleled

and generates signal B.

d) Leave SIMIC session with the command:

quit

Testcase #3 - Dynamic Loading. (dynamic.net and dynamic.run)

This demonstrates SIMIC's ability to adjust element delays (even those

undergoing a state transition) based on the current and dynamic loading

conditions provided by ideal switch elements.

Run Command Descriptions:

a) Define File=dynamic

Specifies the default file name, as in the previous examples.

b) Get Type=dynamic

Requests SIMIC to compile the circuit description.

c) Define pin.1= do 2 (0 1)

Define pen.1.2= 0 1

Apply Pattern=pin List=in

Apply Pattern=pen List=b

Defines and applies stimulus values.  In this case the

stimuli are defined as simulate-until-stable patterns.

Each pattern is applied only when the circuit has finished

responding to the previous stimulus.  This method of

stimulus application:

1) Prevents leaving fundamental mode of operation,

2) allows identification of slow paths easily, and

3) makes available a number of sophisticated

troubleshooting techniques uniquely found in SIMIC.

"Define pin.1= do 2 (0 1)" defines a pattern sequence

called "pin" that will be applied to one signal.  In this

pattern the sequence (0 1) will be applied twice.

"Define pen.1.2= 0 1" defines a pattern sequence called

"pen" that will be applied to one signal and whose default

duration for each vector is two patterns.  The sequence

(0 1) will be applied.  Note that the default duration

specifies that the sequence will really be (0 0 1 1).

"Apply Pattern=pin List=in" attaches the sequence "pin"

to signal "in", and "Apply Pattern=pen List=b" attaches

the sequence "pen" to signal "b".

d) Print List=in*b*a*out Change:

Describes which signals to output, and when output should

occur.  In this case, We wish to display the values of the

signals in, b, a, out.  Again, the asterisk (*) indicates

that a blank vertical column is requested for readability,

and "Change:" indicates that we want output whenever any

signal on the list changes.

e) Simulate

Initiates simulation.

f) Quit

Ends the SIMIC session.

Testcases #4 and #6 combined - Conflicts and Conflict resolution.

This demonstrates SIMIC's ability to detect, and, if possible,

resolve conflicts on wire-ties.

Run command description:

a) Define File=conflict

Defines the default file name.

b) Get Type=Conflict

Compiles the circuit called "Conflict".

c) Define Pa.2= 10 X1

Define Pb.2= 11

Apply Pattern=Pa List=EnA,DataA

Apply Pattern=Pb List=EnB,DataB

Defines and applies the stimulus to the input signals.

As before, the stimulus mode is simulate-until-stable

patterns.  In pattern #1 part A is driving a logic-0,

while part B is driving a logic-1, a "real" conflict

situation.  During simulation, notice the explicit

conflict message, indicating the condition, and the

signals that are in conflict.  In Pattern #2 part A

is either driving a logic-1 or is tristating, due to the

unknown logic value on its enable.  Since part B is still

driving a logic-1, the "conflict" can be resolved as a

logic-1.

d) Print List=EnA,DataA*EnB,DataB*Bus*C

Outputs the specified signals whenever the circuit becomes

stable.

e) Simulate

Initiate simulation.

f) Quit

End SIMIC session.

Testcase #5 - Oscillations. (oscil.net and oscil.run)

This demonstrates SIMIC's ability to trap an oscillation (Excess

activity on a signal for a given circuit input state), and squelch

it (by turning it into an X).  This easily uncovers Meta-stable

states in the circuit.

Run Command Descriptions:

a) Define file=oscil

Defines a default filename.

b) Get Type=oscil

Compiles the circuit named oscil

c) Define Pab.2= 00 11

Apply Pattern=Pab

Defines a pattern sequence and applies it to the input

signals.  Again, we are using simulate-until-stable format.

d) Print List=a,b*q,nq Change:

Specifies to output the listed signals whenever their values

change.

e) Simulate

Do the simulation.

f) Quit

Leave SIMIC.

Testcase #7 Timing Verification - (ftv.net and ftv.run)

This testcase demonstrates SIMIC's unique capability to detect timing

violations functionally.

Network Description Highlights:

a) Timing hazard information.  Information describing timing

checks may be added to all SIMIC flip-flops and D-Latch

primitives.  These are contained within a timing-check

block (Timing-checks= begin; ... end;) Each timing check

may be described as an constant value, or as a load-

dependent value.  In addition, you can control whether a

timing violation message is issued, or whether the flip-flop

state should be set to unknown (X), on a per instance/

per timing-check basis.  In this example, the pulse-width

limit on the negative reset input is set at 20.

Run Command Description:

a) Define File=ftv

Defines the default file name.

b) Get Type=ftv

Compile the circuit called ftv.

c) ?Checks Part=q

Have SIMIC list the timing-check limits and options set

for part Q.

d) Define Pat.3=000 011

Apply Pattern=Pat

Define Period=1000

Define Tclock.RZ=250,750

Define Treset.RO=500,510

Define Tdata.NRZ=0

Apply Timing=Tclock List=c

Apply Timing=Treset List=nr

Apply Timing=Tdata List=d

You should be familiar with the first two commands from

previous examples.  It seems exactly like the simulate-

until-stable format discussed previously.  However, in

this example, we will demonstrate the third method for

describing stimulus, Timing Generators.  This method

emulates modern Automatic Test Equipment (ATE).  Testers

require that a master clock period be define, and timing-

generators be assigned to inputs to produce waveforms

relative to a master clock period.  In our example,

we describe the period with the "Define Period" command,

and each timing generator with the "Define T..." command.

Define Period=1000

Sets the master period to 1000 time-units.

Define Tclock.RZ=250,750

Defines a RZ (Return to Zero) timing generator.  If the

pattern is a 0, the waveform remains at a logic-0.

If the pattern is a 1, then the waveform goes high at

time 250 and then low again at time 750 (relative to the

start of the period).

Define Treset.RO=500,510

Defines a RO (Return to One) timing generator.  If the

pattern is a 1, the waveform remains at a logic-1.  If

the pattern is a 0, then the waveform goes low at time

500 and then high again at time 510 (relative to the

start of the period).

Define Tdata.NRZ=0

This defines an NRZ (Non-Return to Zero) timing

generator.  The waveform obtains the pattern's value at

time 0 (relative to the start of the period).  If no

timing generator is specified for an input, this is SIMIC's

default.

SIMIC also allows description of Drive Envelopes (when an

input changes from drive to tristate and tristate to drive),

supports time-set switching (changing timing generators

on an input during simulation), and strobe definitions

(when to sense for an output or in/out value).

In this demonstration, we could have used SIMIC's waveform

description to accomplish the same task.  The equivalent

waveform description for the timing generator waveforms

would be:

Define Wdata.1.1000= 0 1

Define Wreset.1= 1 &500 do 2 (0 &10 1 &1000)

Define Wclock.1.500= 0 &250 do 2 (1 0)

Apply Pattern=Wdata List=d

Apply Pattern=Wreset List=nr

Apply Pattern=Wclock List=c

Where the & indicates that the following integer represents

the duration for the previous pattern.  For example the

definition for Wreset reads:

"Apply a logic-1 at time 0 and hold it for 500 time-units,

followed by the repetition of the following sequence twice:

Apply a logic-0 and hold it for 10 time-units followed by

a logic-1 held for 1000 time-units."

e) Print List=nr,d,c*q Change:

No Print Pstep:

Specifying to print the listed signals when any one of them

change, and do not output each time the circuit becomes

stable.

f) Simulate

Simulate the circuit.

g) Restore Tnum=0

This command returns the circuit to an uninitialized

(pre-simulation state).  We are doing this in order to

show what would happen if there were no functional

timing checks.

h) No Warn Part=q Pw:

Turn off the warnings for any pulse-width violation on

part Q.

i) No Xpropagate Part=q Pw:

Suppress setting the flip flop for a functional violation

on part Q.

j) Simulate

Simulate the circuit (again).

k) Quit

Leave the SIMIC session.

Testcase #8 - Interactive Debugging (div7.net and div7.run).

In this demonstration we provide an interactive debugging session

encapsulated in a run file.  This provides a demonstration of how

easily SIMIC can isolate problems in the circuit.

Run Command Descriptions:

a) Define File=div7

Define the default name for files.

b) Get type=divide_by_7

Compile the circuit.

c) Define Preset.1= 0 1

Define Pclock.1= 0 do 8 (1 0)

Apply Pattern=Preset List=reset

Apply Pattern=Pclock List=clock

Define and apply the stimulus.  We used simulate-until-

stable patterns here.

d) Define Vq.Posint=q4,q2,q1

Group scalar signals q4, q2, and q1 into an array called

"Vq". When displaying this array, it will be done as a

positive integer (Posint).  Arrays can also be explicitly

defined in the network description.  SIMIC recognizes

arrays in all commands and acts on them collectively.

Other display formats include (binary, octal, hexadecimal,

1's complement, and 2's complement).

e) Print List=clock,reset*q4,q2,q1*Vq

Specify the simulation output.  Note here the use of our

defined array, Vq.

f) No Xpropagate Spike:

This option prohibits detected spikes from propagating as

a pulse of unknown value.  In effect, all spikes will be

filtered from the simulation.  This will demonstrate how

most other simulators would handle this circuit.

g) Simulate

Initiate simulation.  The things to notice in the simulation

results are:

1) Our defined array, Vq.  The header indicates this

signal as *Q to indicate that this signal does not

exist in the network description.

2) The simulation results.  In this simulation, the

circuit appears to be operating properly.

3) The message:

2 Spike messages suppressed.

at the end of the simulation.  This indicates that

spikes occurred during simulation, but we did not

request that they be reported.

h) Restore Tnum=0

Xpropagate Spike:

To check to see if the spike could cause problems with this

circuit, we will re-simulate with the spike propagate turned

back on.  The "Restore Tnum=0" command directs SIMIC to

restart the simulation at its uninitialized state.  The

"Xpropagate Spike:" command counteracts the previously

issued "No Xpropagate Spike:" command.

i) Simulate

Initiate re-simulation of the circuit.  Notice that q1 went

to X at test #7, when the counter should go from count 3 to

4.

j) Restore Tnum=0

Break Memlatch:

Set up for another round of simulation.  In this round,

we set a breakpoint whenever an X is latched into a

memory element (The T flip-flops in this circuit).

k) Simulate

Initiate simulation again.  SIMIC stops and issues a

breakpoint message, that the flip-flop latched in an

X when the NS input (not-set) went X:

3 B     7> MEMLATCH CONDITION (NS AT 'X')[TNCF]: Q1

l) Restore Tnum=*

Trace List: Expand:

Instructs SIMIC to roll back to the start of this test

(previously stable point) with the Restore command.

Also, turn on activity tracing on all signals "Trace List:".

The "Expand:" option instructs SIMIC to include causality

information with the trace.

m) Simulate

Initiate simulation.  Notice the event trace, "E", messages

followed by causality, "C" information.

Scanning the Trace information, we see that the master rank

of Q1 (Q1.1) went unknown, caused by SKIP_2 going unknown.

SKIP_2 went unknown because NQ1_Q2 went unknown, triggered

by NQ2 and Q1 changing too close for NQ1_Q1 to adequately

respond.

We have quickly and easily identified the problem.  Notice

we are frozen at the breakpoint again.  We can replay this

small piece of the simulation as many times as necessary

until the problem is isolated.

SIMIC has a large repertoire of interactive debugging tools.

Here are some you may want to try:

Display value of signal Q1:

Look List=Q1

Display value of signal CL4 and its input drivers:

Look Input: List=cl4

Display any signals in an unknown state:

Look X:

Display delays of all signals:

?Delay List:

Here are some other commands that are useful when

troubleshooting real designs:

Display charge storage value for signal A:

?Decay List=a

Display load values for signals B and C:

?Loading List=b,c

Checkpoint the simulation every 100 stable points:

Save Pstep=100

Stop simulation if signal D goes unknown:

Break X=d

Warn if any signal spikes.

Warn Spike:

There are many more options that make troubleshooting

large circuit easy in SIMIC.

n) Quit

Leave SIMIC session.