Tips and Tricks using FSUUCP Version 1.51

By Christopher J. Ambler, Terranet Communications / UUPlus Development

----------------

- Introduction -

----------------

The UUCP suite of programs can be very tricky to set up and operate. With

this in mind, here are some tips and tricks for the operation of FSUUCP.

Most of the material here represents the most frequently asked questions,

as well as some 'insider' hints that might be found to be helpful.

-------------------------------------------------

- Larger Packet Sizes, and Increased Throughput -

-------------------------------------------------

The g protocol that FSUUCP UUCICO.EXE uses will support packet sizes from

64 bytes up to 4096 (4K) bytes, in intervals of powers-of-2 (eg. 64, 128, 256,

512, 1024, 2048, and 4096). Selecting the proper packet size can increase your

throughput by as much as 100 CPS, but can be somewhat tricky, so here are a

few hints and guidelines.

First, let's answer the question, "Why?" With 64-byte packets, the overhead

in the g protocol is considerable. With each packet, there is a header, and

an acknowledgement sent back. Simple math will show that if each packet is

1K in size, there will be less ack packets to be sent, and less overhead per

file, as there will be less packets to send.

There are two considerations to keep in mind when selecting packet sizes.

First, can the remote side handle sizes greater than 64 bytes in the first

place? Most older UUCP implementations have 64-byte packet sizes hardcoded.

Since original versions of UUCP (since the early 70's!) used only 64-byte

packets, versions that came later didn't bother to include the capability to

select other sizes. Over time, the 64-byte packet became the de-facto

standard. With the modernization of modem technology, the need for larger

packets has become more pressing, and so newer UUCP implementations have begun

to support them.

A common question is, "why won't larger packet sizes work?" The first and

most common answer will be because the host provider is running an older UUCP

that doesn't support them. Unfortunately, that isn't always the case. Another

common problem occurs when the host provider *is* running a newer UUCP, but has

not configured their modem pool to run efficiently with larger packets. Keep in

mind that with 4K packets, the computer on their end is streaming 4K of data to

their modem in continuous chunks. With port speeds in excess of 57.6KBPS, that

is much more data than the modem can handle in so short a time. The result is

an overflowed buffer on the host side, and checksum errors on your side. Often,

with a hardware configuration like this, 256-byte packets is the theoretical

limit that can be expected from your provider. Thankfully, as UUCP service

providers learn more about their setups (remember, this is a new field!), they

are becoming able to support better buffering and larger packets.

The simple way to check if your host provider supports larger packet sizes

is to simply change the value of the g_packet configuration value in your

systems file, and run UUCICO. The line in your systems file should look like:

configure g_packet 4096

(note that 4096 is a selection for 4K packets. You can put any valid

packet selection value here).

If they do not support the larget packet size, you can expect colourful

errors in the form of checksum errors, packet-header xor errors, or even a

complete protocol lockup. Simply reset your computer, and try again with a

different packet size, or return to 64-byte packets. If you discover that only

64-byte packets work, you can be fairly sure that they are running an older

UUCP package. If you can get packet sizes up to 256 or 512 to run, but not

higher, they probably have a modem buffering problem. In the former case, you

may want to politely suggest that they look into upgrading their UUCP. They

can run Taylor UUCP, which is a freely-available UUCP replacement.

=== Selecting The Packet Size That's Right For You ===

Selecting the optimum packet size is mainly trial-and-error. Try each size

that works well with your provider, and keep a record of the CPS rate on your

transfers. Try to poll when you have large files, as larger files report CPS

more accurately. 100K to 200K news batches are ideal for testing this.

In tests, we found better performance at different packet sizes at different

providers. Most, however, seem to get the best performance at either 256-byte

or 1K-byte packets. Your mileage will vary, of course.

------------------------------------------------

- Disk Buffers, UARTS, and Interrupts (oh my!) -

------------------------------------------------

=== UARTS and Speed Considerations ===

One common problem experienced with UUCP transfers by FSUUCP is loss of

characters due to UART overrun. The problem is caused when a character coming

into the computer from the modem cannot be processed by the UART because

there is already a character in the UART that has not yet been processed. This

sounds a lot more complicated than it really is.

There are a few causes of this, and most are easilly solved. The most common

reason is simply a slow UART. Most older computers with original 8250 UART

chips cannot handle port speeds over 9600, and some poorly designed 8250 boards

have difficulty at 9600! The first solution to try, then, is to lower your

portspeed value in your .MDM file.

=== The 16550 UART, and Why It's Good For You ===

The advent of the 16550 UART, with its 16 character buffer, has helped to

reduce the occurance of this problem. The 16550, by virtue of being a faster

UART, can handle port speeds up to 57600, but the real problem solver is the

16 character FIFO buffer. Older UARTS, like the 8250 and 16450, can handle

only 1 character at a time. If another character comes in before the UART is

done processing the first one, a _UART_Overrun_ will occur, causing an error.

The 16550, however, can accept up to 16 characters before having the process

them. This has two distinct advantages. First, obviously, overruns are less

likely to occur. Second, since the UART has to signal the processor to take

the characters less often, transfer speeds tend to increase. Taking advantage

of this buffering is as simple as adding a value to the 16550trigger variable

in your .MDM file. Acceptable values are 1 (which will cause the 16550 to

operate as any other UART), 4, 8, and 14. This value specifies the number

of characters that the UART will store before signaling the processor (with

an interrupt) to process the characters. We recommend a setting of 8. You

can also specify the keyword "disable" to turn this feature off.

=== Loss of Data Due to Disk Writes ===

A very annoying "feature" of PC hardware is the interrupt shutoff during

disk writes. This has the unwanted effect of preventing the UART from

signaling the processor that a character has come in. On slow machines,

this interrupt shutoff time can easilly be longer than a complete character

cycle, and often can prevent 2 or more characters from being seen. This

problem is easy to identify: if you notice that you get "UART Overrun"

errors, and a "Checksum Error" immediately following, take a look at your

hard disk activity light. If you see it flash at the same time as these

errors, you can confirm that this is your problem.

The solution is more of a workaround than a fix, as there is nothing that

can be done about the hardware. There are, however, a few things that can

help make this problem less of an impact. First, and easiest, is to slow

down the portspeed. If you are connecting at 9600 to your host, there is

no reason to have your portspeed variable (in your .MDM file) set to 38400!

Try 19200, or even 9600. In most cases, if your portspeed was set too high,

this will eliminate the problem alltogether. If it wasn't set too high, or

lowering it has not solved the problem, you may want to increase your disk

buffer. The disk buffer is the amount of data that will be stored before

a write to disk is made. By default, MSDOS buffers very little data. UUCICO

has the option to set its own disk buffer, and store up to 32K of data

before writing to disk. If most of your transferred files are less than

32K in size, you'll never notice this problem, as there is no modem

traffic between files, where the disk write would then occur (the buffer

is flushed after each file). In the worst case, the buffer will flush much

less often with a large buffer. The variable in question is "diskbuffer" and

is found in the .MDM file. Note that this variable takes a value in hex (base

16), and the "0x" that is commonly found in front of a hex number is optional.

It's recommended to keep it in, however, to remind you that the number is,

in fact hex. A value of 0x4000, for example, is a 16K buffer. 0x8000, the

maximum value, is a 32K buffer.

------------------------------------

- Alternate Drivers for UUCICO.EXE -

------------------------------------

UUCICO has the option of using its own internal communication drivers, or

accessing any FOSSIL-compliant drivers you may have loaded. The decision is

made based on the "driver" variable in your .MDM file. The two valid options,

"internal" and "fossil", determine which to use.

------------------------------

- How Can FSUUCP Be Sped Up? -

------------------------------

Transfer rates can be increased by optimizing the packet size, if your

host provider allows this. This is the #1 best way to increase the speed

of your transfers, aside from buying a faster modem. Of course, buying

a faster modem is always a good option, too!

The most commonly asked speed question is, "How can I get UUXQT.EXE to

process my mail and news faster? I've got TONS of news, and it takes quite

a long time!" The answer is, unfortunately, not much. Keep in mind the

volume of traffic that you're processing. With a large newsfeed, there is

quite a bit of processing going on: decompressing, splitting up articles,

determining the newsgroups to place them in, updating incoming information,

copying the news article into the proper directory, and cleaning up after

the processing. Comparing the time it takes to process news on your FSUUCP

system with the time it takes to do the same job under a UNIX system running

UUCP shows that FSUUCP is actually faster!

With that said, however, we have found that a disk cache works wonders

for the processing time of UUXQT. With the amount of disk I/O that UUXQT

performs, a 2-meg disk cache can avoid a large number of disk accesses and

group them into a single large write operation.

WARNING: disk caches are potentially dangerous! A power outage during

a cached operation can result in data loss, and corruption of your Usenet

and Internet Mail traffic! If you are going to increase your speed through

a disk cache, we strongly recommend some kind of UPS power backup. It's a

small investment to make to insure that you work quickly and safely.

A last suggestion is to create a ramdisk and point your temporary

directory at it. This can have a very beneficial effect upon speed.

-----------------------------------

- What About Memory Requirements? -

-----------------------------------

In older versions of FSUUCP (prior to 1.50), the compression utilities were

imbedded into the utilities that used them (UUXQT.EXE, BATCHER.EXE, etc) and

required quite a bit of memory. Decompression, for example, could require

up to 610K of free memory to run! This, being unacceptable to most users,

has been changed in version 1.50. COMP.EXE and DECOMP.EXE are now called

as external programs from the utilities that need them.

-----------------------------------------------------

- How Can I Get UUCICO To Retry If My Host Is Busy? -

-----------------------------------------------------

UUCICO has the ability to schedule retries in the case of a busy host,

a failed connection, or an inability to dial for various reasons. In your

systems file, add the lines:

configure retries 3

configure retrytime 30,60,90

This will instruct UUCICO to retry the system for which the configure

pertains up to 3 times before exiting. After each call, UUCICO will pause

the amount of seconds in the retrytime value before trying again. In the

above example, UUCICO will make a total of 4 calls (the initial call as

well as 3 retries). After the initial call, it will pause for 30 seconds

before trying again, 60 seconds after the first retry, and 90 seconds

after the second retry. After the third retry, it will give up and exit.

You may configure up to 10 retries. If you have less retrytime arguments

than the number of retries that you specified, the last value will be

repeated. So, if your configure lines look like:

configure retries 8

configure retrytime 30,60,90,120

then it will make a total of 9 calls before giving up, and after the

third retry, and for all subsequent retries, it will wait 120 seconds

before trying again.

Of course, if UUCICO gets through and the protocol is started, there

will be no further retries.

----------------------------------------------

- What Do Those Messages In UUCICO.EXE Mean? -

----------------------------------------------

UUCICO, the UUCP protocol transfer engine, can report some pretty confusing

messages at times. Here is a list of some of the more important messages and

their meanings:

No modem config file specified

Could not open modem config file

Illegal IRQ specified

No UART detected

Unknown com port type

Initialization Error

Unable to classify UART, or no UART present

Error initializing modem

These messages indicate a problem starting up UUCICO. They should

be self-explanatory.

waiting for shere

waiting for ROK message

These messages indicate that UUCICO is simply waiting for the

appropriate messages from the remote.

RMSG: flushing for sync

RMSG: timeout in sync phase

RMSG: timeout in data phase

"RMSG" is the section of UUCICO responsible for communicating with the

remote site before the actual protocol begins. These timeout messages

indicate that the remote has not yet sent either a sync or the data

expected. It could mean a bad connection, or simply a slow remote.

The "flushing for synch" means simply that the system is letting

data go by until it syncs up with the remote.

Shere error

"Shere" is the message that the remote sends to identify itself. This

error indicates that UUCICO had a problem parsing the Shere message.

Host name error

Simply, the host name that the remote site identified itself with in

the Shere message did not match the host name that you specified with

the -s parameter.

Startup: Invalid R Message

The "R Message" is the reply from the remote telling you that everything

is OK, and we can start the protocol. This error indicates that UUCICO

had a problem parsing the R Message.

waiting for protocol

UUCICO is waiting for the remote site to identify the protocols it

speaks

Startup: No Common Protocol

This will almost assuredly never come up. It indicates that UUCICO and

the remote site could not find a protocol in common. Since all UUCP

packages speak 'g' protocol, without exception, this should never

happen.

ROK: Taylor UUCP limiting feature enabled

ROK: Remote ULIMIT set to [amount] bytes

These messages indicate features that are enabled during startup

Warning: INITA & INITC windows don't match

During startup, the INITA and INITC packets should contain the same

value for the number of packet windows to use. This warning simply

means that they didn't match. UUCICO will use the value in INITC.

Bad speed value in systems file

This indicates that the speed value in your systems file was not

a valid value (perhaps a negative number, or some huge number).

No Carrier

Modem Error

No Dialtone

Called number is busy

No Answer

Called number is answered with voice

Call timeout

Unknown modem result

These are responses that might be seen during the connection attempt to

your host provider, immediately after dialing. They are conditions the

modem may report.

g Init: heap empty, aborting

g Init: heap corrupt, aborting

g Init: heap error, aborting

These are messages that might come up while initializing the g protocol

driver. The indicate errors with UUCICO.

Illegal packet size, use 64-4096, not single-digits.

During startup, the packet size you request (or the default, if you

do not specifically request a packet size) is verified to be legal.

If not, you will get this message.

Packet Error: Header Deficient

Packet Error: bad header xor

Packet Error: packet data deficient

Packet Error: Checksum error

As the transfer runs, there are a few conditions that can cause an

incoming packet to be rejected. The above messages indicate that one

of these conditions has occurred. Header deficient indicates that there

were not enough characters received to construct a valid header. Bad

header xor means that the header's checksum (a simple xor) failed,

indicating a garbled transmission. Packet data deficient means that

not enough characters were recieved to construct a full packet.

Checksum error means that the packet's checksum did not match the

calculated checksum, and the packet must be rejected. The most common

cause for these errors is line noise.

Please insure that CD does NOT track DTR. This can usually be set

with the &C1 command to your modem in the INIT string.

Quite a few modems are shipped with CD tracking DTR as the default.

This means that CD will be high if DTR is high. Since DTR is raised

when UUCICO initializes the modem, UUCICO would then think that it is

immediately connected. UUCICO, then, checks to make sure that they do

not track by raising, lowering, and rasing DTR again, and checking to

see if CD did the same. If so, you will get this message. AT&C1 is

almost always the command to have your modem disengage the CD/DTR

tracking.

Memory Allocation Error, GPOLLVIEW

Memory Allocation Error, GPROTOCOL

OpenDriver: MEMORY ALLOCATION ERROR, outPacket

OpenDriver: MEMORY ALLOCATION ERROR, outPacket

OpenDriver: MEMORY ALLOCATION ERROR, rawData

OpenDriver: MEMORY ALLOCATION ERROR, outbuf

OpenDriver: MEMORY ALLOCATION ERROR, inbuf

The above are memory allocation errors during the allocation of the

buffers for UUCICO. They indicate a severe shortage of memory.

Remote site does not have an entry for you! Contact the

administrator of [sitename] to solve this problem.

This common message means that your host provider has not yet set up

your UUCP account.

Remote site thinks that it is already talking to us. Perhaps it is.

UUCICO will not talk to a site that thinks it is already talking to you.

This could be caused by the host provider failing to disconnect even

when you have, or you might be running a transfer on another machine.

You may only have one UUCICO running at any time.

Remote site will call back

This indicates that UUCICO was told by your host provider that it would

rather call you back. Since UUCICO does not support auto-answer of calls,

you should contact your host provider and inform them that you are getting

this message.

Remote site reports back sequence number. Since we don't support

sequencing, please inform the administrator of [sitename] to

turn off sequencing

You appear to be using the wrong login name for this UUCP account

Contact the administrator of [sitename] and tell them that

you are receiving RLOGIN messages on your UUCP feed

These messages should be self-explanatory.

Driver: Data out of sequence: got [value] expected [value]

Driver: data out of window: got [value] window ([value] - [value])

These are common errors after a checksum error has occured. They mean

that the packets received were out of order, or out of the expected

window. If a packet is requested to be resent, there will usually be

up to 6 packets already "in the pipe" that are no longer valid. These

messages indicate that they came in.

FATAL ERROR: incoming packet larger than buffer

This means that the remote site sent a packet larger than they said

they would (and hence, larger than the buffer that we allocated)