Tandy DAC Programming Guide

version 1.8

September 30, 1996

This guide is by:

Jeffrey L. Hayes

130 Forest Ave., Lot 1

Orono, Me. 04473

(207) 866-7903

Internet:  tvdog@agate.net

The information in this guide is based on the _Tandy 1000 TL Technical

Reference Manual_, Ralf Brown's interrupt list, the Tlslsnd package by

Bruce M. Terry, Jr., John Ball's "Tandy Sound Programming Information," the

PSSJ Digital Sound Toolkit by Frank Durda IV, and miscellaneous sources.

If you have additional information, or you see an error that should be

corrected, please call or write (email preferred).  Particularly, if you

have technical information about a Tandy model with the DAC, and what you

have does not agree with this, let me know.  Please distribute this guide,

and please include Tandy DAC support in your programs!

All programs and documentation included with this package are hereby placed

in the public domain.

Contents

--------

1.  Introduction

2.  The Machines

3.  BIOS Interrupts

4.  I/O Ports

5.  Setting the Recording Rate

6.  Playing in Direct Mode

7.  Program Examples

8.  Other Sources

1.  Introduction

On July 27, 1988, the Tandy Corporation announced two new computers, the

8086-based 1000SL and the 80286-based 1000TL.  These systems contained a

number of innovations remarkable for the time, one of which was the

capability of recording and playing digitized sound.  Tandy's news release

declared:

Tandy's innovative reputation continues with the 1000 TL, as the

company also introduces high fidelity audio "input and output"

technology to the PC industry.

Through the use of internal digital to analog and analog to digital

converter circuitry, the 1000 TL is the first MS-DOS personal computer

to record, store and playback speech or sound.

With this innovation, Tandy has given software publishers one of the

most exciting new tools yet with which to create quality products not

available to the industry today.

... and all that most PC-compatibles could do straight from the factory was

beep.  The same audio DAC circuitry was used in several subsequent Tandy

models, until Tandy started including SoundBlaster capability instead.

(The Tandy DAC preceded the introduction of the SoundBlaster by a year.)

Unfortunately, software publishers other than Tandy itself do not appear to

have made much use of these "exciting new tools" - at least, publishers of

free- and shareware have not.  Users have been sold on the sound

capabilities, only to discover later that DeskMate Music and Sound are

(almost) the only applications available that support their sound chip.

There is a Windows 3.1 sound driver for the Tandy chip, available from

Radio Shack, but many machines with the Tandy DAC can't run Windows 3.1.

There were also several games written to support the chip, some of which

will not run on newer models due to bugs in the chip detection routines.

Music and Sound are fine programs, but they suffer from the usual Tandy

curse:  they are proprietary.  Sound .snd files are playable on a Tandy and

no other machine (Conv2snd v2.00 or later will convert between .snd and

.wav, and Snd2sam, in this package, will convert .snd files to Amiga .mod

samples).  Music likewise creates song modules that no other machine can

use, and in this case there is nothing to convert a standard song format

such as .mod to .sng, or vice versa.

Furthermore, both programs run only under DeskMate.  On the 1000SL and TL,

the core of DeskMate is in ROM and is tied to DOS 3.3, also in ROM.  Hence,

upgrading DOS results in losing DeskMate and consequently Music and Sound.

One can obtain the standalone version of DeskMate and use Music and Sound

with that, but only at additional expense.

Confronted with this lack of software support for the built-in sound, users

desiring sound capability better than that supplied by the PC speaker

(programs that use the generic PC speaker for sound output will, of course,

run) have had to purchase Soundblaster cards.  Personally, it offends me

that I have to use one of my computer's scarce expansion slots to get good

sound, when it already has perfectly adequate (though no longer state-of-

the-art) sound capability built in.

The offense is all the more offensive when one considers that the original

Soundblaster card is no better than the built-in sound.  Both the Tandy DAC

and the Soundblaster play 8-bit mono, but the Tandy can play at 44kHz and

record at up to 22kHz (higher rates are possible, but it becomes difficult

to set the rate precisely at those speeds), while the Soundblaster can only

play at 22kHz and record at 13kHz.  The Tandy sound chip also has built-in

BIOS support on those machines that have it, making it relatively easy to

program for simple applications.  (Tandy's use Phoenix BIOSes.)

2.  The Machines

The following Tandy models include the Tandy DAC:  1000SL, SL/2, TL, TL/2,

TL/3, RL, RL-HD, RLX, RLX-HD, RLX-B, RLX-HD-B, and RSX, and the entire

2500-series.  (Thanks to Ken Udut for this information.)  The Sensation

model I has DAC support for output only.  At the time some of these models

were produced (1988-89), Tandy was the number-one manufacturer of PC's in

the U.S.  The 1000RSX and the 2500SX were in stores as of 1993.  Tandy's

are very reliable machines, and hundreds of thousands of machines with the

Tandy DAC are still in service.

The 2500's vary from the 2500XL, a 10MHz 286, to the 2500SX/33, a 33MHz

80386.  All models came standard with at least 1 meg of RAM, and all can be

upgraded to at least 9 megs.  The XL and XL/2 are 286's, the rest are

386's.  The 2500SX and submodels use the HT chipset.  All models came

standard with at least basic VGA.  OS/2 was packaged with some; all will

run Windows 3.1.  The Sensation model I is a 25MHz 486SX (it has an

Overdrive socket).

The processors used in the 1000's vary from the 8MHz 8086 in the 1000SL to

the 25MHz 80386 in the 1000RSX.  Except for the 1000SL, SL/2, RL, and RL-

HD, Tandy's that have the Tandy DAC have at least an 80286.  The SL-, TL-,

and RL-series are XT-compatibles.  The RSX is an AT-compatible.  The RLX is

a borderline case, being an AT on the motherboard, but having an (that's

right, *an*) 8-bit expansion slot.

The 1000RLX's have built-in VGA with 256k RAM (not expandable).  The

1000RSX has SVGA (AcuMos AVGA2 with 256k upgradable to 512k).  All models

can be upgraded to VGA.  The SL's, TL's, and RL's have proprietary video

which is a superset of CGA.  Specifically, they have Tandy Video II, which

is a superset of Tandy Video I (used in the earlier 1000-series), which is

compatible with PC-Jr video, which is a superset of CGA.  The video on

these machines is comparable to what one would get from an EGA card with a

CGA monitor attached (640x200 16-colors).  Tandy Video II also supports

Hercules graphics when a mono monitor is attached.  (There are some tools

for handling Tandy video on my ftp site.)

All of the 1000-series with the Tandy DAC have a 101-key keyboard, though

on some the keyboard is of the unidirectional XT type (don't count on being

able to send commands to the keyboard).  Earlier models without the DAC

used an 84-key keyboard with some nonstandard scan codes, but the scan

codes are standard on the newer models.  (The Tandy 1000-series FAQ lists

the scan codes for the old keyboard.)

In addition to the DAC, there is a 3-voice tone and noise generator

compatible with the Texas Instruments SN76496 used on the IBM PCjr.  Tandy

offered some enhancements over the PCjr 3-voice chip, including the

capability of using the DAC as a 4th tone channel and the ability to play

tones an octave lower than the TI chip.  The presence of this 3-voice tone

capability has caused some confusion.  The Tandy DAC does not have 3

voices.  While the 3-voice circuitry is on the same IC as the DAC, they are

separate functions.  Older 1000's without the DAC still have the 3-voice

circuitry, but without the 4th channel or the lower octave.

Some (all?) of the computers under consideration have a volume control

knob.

The DAC uses DMA channel 1, IRQ 7.  IRQ 7 is shared with the printer; the

BIOS takes account of this fact.  The PSSJ expansion card (see below) can

be jumpered for different DMA channels and/or IRQ's, but it is rare.

All Tandy models with the DAC came with DOS 3.3 or later.  There is a rumor

that it is impossible to upgrade the DOS version on some 1000 models; this

rumor is false, and by now most have been upgraded.  The 1000RSX, all

models of the 2500-series, and the Sensation can run Windows 3.1; earlier

models can run Windows up to 3.0 (upgrades being required in most cases for

adequate performance).  OS/2 will run only on the 1000RSX, the 2500-series,

and the Sensation.

In addition to the above, there was a PSSJ expansion card made that could

be installed in any IBM-compatible.  (PSSJ being the name of the IC that

contains the sound circuitry - "Printer, Serial, Sound, and Joystick.")

The card came with a loadable driver that provided the missing BIOS

support.  PSSJ cards are very rare, however; few were made.  There were

some models sold under the Panasonic name that have the sound circuitry,

and a few custom systems made for Hallmark and other companies that were

clones of the 1000SL and TL.  Some 1000AX's sold at Walmart may have the

sound circuitry as well.  (This information from Frank Durda.)

3.  BIOS Interrupts

The BIOS provides access to the DAC on Tandy's that have the chip, using

Int 1Ah.  This is the easiest way to program the chip.  The BIOS announces

completion of a sound request by calling out to Int 15h.  See the comments

following the list.

--------------------------------------------------------------

Int 1Ah AH=7Fh                  Get Sound Chip ID

(Tandy 2500 only)

Use:

AL = location for chip ID info:

0 = on stack

1 = in buffer at ES:BX

if AL=1 then

ES:BX -> buffer for chip ID info

Returns:

Sound chip ID string (???).

Int 1Ah AH=80h                  Set sound multiplexer for audio source

(PC-Jr / Tandy 1000 only)

Use:

AL = source:

0 = 8253 channel 2

1 = cassette interface

2 = audio in on I/O board

3 = sound chip

Returns nothing.

(This function is a no-op on the 1000TL.)

Int 1Ah AH=81h                  Get DAC base I/O port

Use:

AL = 0

Returns:

AX = base I/O port of DAC (00C4h on most 1000's, 01E4h on 2500's,

0304h on the PSSJ expansion card).

CF clear if sound not busy, set if busy.

(Returned AX is >= 8000h if the DAC is not present; use this call to

detect the DAC - see below.  The 3-voice chip is at (DACbase - 4) if

this call is supported, 0C0h otherwise.)

Int 1Ah AH=82h                  Record sound

Use:

ES:BX -> buffer for sound

CX = length of buffer

DX = transfer rate (1-4095)

Returns:

AH = 0

CF clear if sound not busy, set if busy.

Int 1Ah AH=83h                  Play sound

Use:

ES:BX -> buffer for sound

CX = length of buffer

DX = transfer rate (1-4095)

AL = volume (0-7, 0 = silence)

Returns:

AH = 0

CF clear if sound not busy, set if busy.

Int 1Ah AH=84h                  Stop sound

Use nothing.

Returns nothing.

Int 1Ah AH=85h                  Reinitialize joystick

(Not supported on all systems)

Use nothing.

Returns nothing.

--------------------------------------------------------------

I don't know the format for the ID string returned by Int 1Ah AH=7Fh on the

2500-series.

Int 1Ah AX=8003h should be called before using the 3-voice tone and noise

generator on the Tandy 1000 or PC-Jr.  It is not necessary to call this

interrupt to use the DAC.  Alternately, set bits 5 and 6 high and bit 0 low

at port 61h on older machines without the DAC before using the 3-voice

chip (this is what the BIOS code does).

Use Int 1Ah AH=81h to detect the presence of a Tandy DAC.  To do this,

first issue Int 1Ah AX=8003h.  This is "Set Multiplexer for Audio Source"

on the 1000-series and IBM PC-Jr.  As shown above, the audio source is

being set for the 3-voice tone generator.  On the TL or other models with

the DAC, this function, if supported, is a no-op; all audio (from whatever

source) is mixed.  At any rate, it does no harm.  The real purpose is to

detect the presence of PCMCIA Socket Services, which is present if the

interrupt returns CX=5353h.  If PCMCIA services is installed, the DAC can't

be used because of a software interrupt conflict, so the detection routine

should report that the DAC is not present in this case.  Then issue Int 1Ah

AH=8100h.  If AX < 8000h on return, a Tandy DAC is probably present, and

the value returned in AX is the base I/O port of the DAC.  You can confirm

this by testing port (DAC base)+2.  You should be able to set and clear all

bits.  If so, the DAC is there.

Generally, it is not necessary to program I/O ports to use the DAC;

however, it _is_ necessary to do so if you wish to use the DAC in direct

(non-DMA) mode (see below).  It is possible to get better sound by

programming the hardware, however, particularly if you wish to record or

play sound for an extended period of time (i.e., the sound won't fit in

64k).  The base port may be 0C4h, 1E4h, or 304h, as noted above.

If the DAC is present, the address of the 3-voice tone and noise generator

is computed by subtracting 4 from the base DAC port address returned by Int

1Ah AH=81h; i.e., the 3-voice chip is at 0C0h, 1E0h, or 300h.  If the call

is not supported, the 3-voice chip may still be there.  All models of the

Tandy 1000, as well as the IBM PCjr, have the 3-voice chip, even if they

don't have the DAC.  On older Tandy 1000 models with the 3-voice circuitry,

the byte at FFFF:000Eh is 0FFh and the byte at FC00:0h is 21h, while on the

PCjr the byte at FFFF:000Eh is 0FDh.  The 3-voice tone and noise generator

is at port 0C0h on the older machines.  (See Piano.asm.)

The BIOS calls out to Int 15h with AX=91FBh when sound DMA is complete

(i.e., when Int 1Ah functions 82h, 83h, and 84h are complete).  Your

program should hook this interrupt if you are using the BIOS sound

functions and you need to know when the request is completed.  The Int 1Ah

AH=82h, 83h, and 84h calls return immediately, continuing to play, record

or halt sound I/O in the background.

Sound buffers passed to Int 1Ah AH=82h or 83h do not have to be on 64k

boundaries.  The BIOS masks the DMA-across-64k problem from the program.

The value returned in the carry flag by the various sound functions is not

to be relied on.  In general, if CF is set, it means that the sound is

busy, but if CF is clear, it does *not* mean that the sound is available.

Initializing the sound chip seems to be the main problem to be overcome

when using the Tandy DAC.  One must call AH=84h and wait for the BIOS

callout to be sure; see Dac_init.asm, included with this package.  (Thanks

to Bruce M. Terry, Jr., for his information on this.)  Int 1Ah AH=84h is

listed above as returning nothing, but in fact it "returns" a BIOS callout

to Int 15h when complete.

Note that the above method of initializing the DAC will hang systems

without the chip.  It is important to detect the chip before initializing

it.

The above method of initializing the DAC causes a small click, owing to the

fact that what the BIOS does is actually to play a very short (10-byte)

sound and wait for it to finish.  (The sound is not even baseline - it's 10

zero bytes.)

The DAC should be finalized before the program exits, normally by the same

method as it is initialized.  If this is not done, the Tandy BIOS will stop

DMA at the sound chip, but not at the DMA controller, causing problems for

some devices using DMA channel 1 (e.g., SoundBlaster).  If you are

programming I/O ports directly, you can program them directly to initialize

and finalize the sound chip as well - see Recwav.asm and Playwav.asm.

The transfer rate values in DX used by Int 1Ah functions 82h and 83h are

dividers used to clock the sound chip.  Unfortunately, the values used are

different in each function, and the values used with function 82h depend on

the model involved as well as the code being executed.  For function 83h,

compute the divider as follows:

divider := 3579545 / sampling_rate_in_Hz

The DAC is clocked at 3 times the PIT rate.  Parenthetically, the divider

for function 83h is exactly the same as the period value in an Amiga music

module.

Tandy says that for function 82h, you should compute the divider as

follows:

divider_for_82h := divider_for_83h / 10     (on a 1000TL)

divider_for_82h := divider_for_83h / 11.5   (on an 1000SL)

Unfortunately, the above seems to cause sounds to play back slightly faster

(about one semitone higher in pitch) than they record.  The true

relationship on the TL is found experimentally to be approximately:

divider_for_82h := (divider_for_83h - 12.5) / 10

For the SL, I compute the recording divider as follows (for no reason other

than symmetry with the above):

divider_for_82h := (divider_for_83h - 12.5) / 11.5

Later models included an improved version of the sound chip that used a

double buffer when recording.  For them:

divider_for_82h := divider_for_83h / 10

Without the double buffer on the sound chip, the processor speed effects

the recording rate (the SL is an 8086, and the TL is a 286).  See the

section below for more on dividers and sampling rates.

You can detect the presence of a double-buffered chip as follows (this

information from the PSSJ Digital Sound Toolkit, see bottom of this file).

First check for the presence of the DAC - if it isn't there, you don't need

this information :-).  Set, then clear, bit 4 of port (DAC base)+3 (i.e.,

port 0C7h, 01E7, or 307h), reading back the bit each time, leaving the

other bits alone.  If you are able to both set and clear the bit (and have

it stick), the DAC is an old version without double buffering; otherwise,

it has a double buffer.  See Recwav.asm.

If the DAC does not have a double buffer, you can detect the system type as

follows (this is from the PDST also).  Issue Int 15h, AH=0C0h (system ID).

The call should return with carry clear.  If so, ES:BX addresses machine ID

information.  On the SL, the word at ES:[BX+2] is 00FFh.  On the TL, that

word is either 01FFh or 05FFh.  For further confirmation, you can verify

that the byte at FFFF:Eh is 0FFh and that the byte at FC00:0h is 21h.

These two bytes identify the system as a Tandy 1000-series.

If Int 15h, AH=0C0h returns with carry set, indicating the BIOS does not

support the call, or if it returns with carry clear and the word at

ES:[BX+2] is not one of those three, it's an old version of the PSSJ

expansion card, installed in some unknown system.  Frankly, I don't know

how you set the recording divider in that case, since the appropriate

divider depends on the processor speed and number of wait states.  I just

use TL timings in that case.

Int 1Ah AH=84h may be used at any time to stop sound input or output.

Int 1Ah AH=85h is not supported on the 1000TL.  I don't know what systems

have it.

For (fairly) seamless playing and recording of long sounds, Int 1Ah can be

called from within the Int 15h handler.  Minor clicks can be heard by this

method.  If they are objectionable, you can program the sound hardware

directly to play long sounds.

Of the programs in this package, only Testdiv uses the BIOS for sound I/O.

4.  I/O Ports

As noted above, it may not be necessary to program I/O ports to use the

Tandy DAC, particularly if you only want to record or play short snippets

of sound.  One case where it is necessary is if you want to perform output

byte-by-byte rather than using DMA (see "Playing in Direct Mode," below).

The 3-voice chip also lacks BIOS support and must be programmed directly.

The sound circuitry (3-voice tone and noise generator and DAC) use a range

of 8 I/O ports, 0C0h-0C7h, 1E0h-1E7h, or 300h-307h.  See the above section

on "BIOS Interrupts" for how to determine the port addresses.

--------------------------------------------------------------

Port 0C0h (1E0h, 300h)          SN76496 3-voice tone and noise generator

Write only

Bits

7 6 5 4 3 2 1 0       Function

---------------       --------

1 x x 0 y y y y       Set tone frequency, channel xx (0-2).  yyyy = low 4

bits of divider.

0 w z z z z z z       (following the above) zzzzzz = high 6 bits of divider.

If extra divide enabled (port 0C4h bit 6), w = high

(11th) bit of divider (otherwise, w is a "don't

care" bit).

1 x x 1 y y y y       Set tone attenuation, channel xx (0-2).  yyyy =

attenuation (1's complement of volume).  Attenuation

1111b = no sound.  Approximately 2 dB per level.

1 1 1 0 x y z z       Set noise type/frequency.  x is a "don't care" bit.

y = periodic/white noise code:  0 = periodic noise,

1 = white noise.  zz = frequency:  0 = N/512, 1 =

N/1024, 2 = N/2048, 3 = Tone 3, where N = 3579545.

0 is highest noise frequency (see below).

1 1 1 1 y y y y       Set noise attenuation.  Attenuation 1111b = no sound.

Port 0C4h (1E4h, 304h)          DAC Mode Register

Bits 0-2 and 4-6 are read/write

Bit(s)        Meaning

------        -------

0-1         DAC function select:  0 = joystick, 1 = sound channel

(fourth tone channel, used with 3-voice chip), 2 =

successive approximation (sound input), 3 = direct write

to DAC (sound output).

2           DMA enable (if bit 1 = 1).  0 = DMA disabled, 1 = DMA enabled.

3 (read)    1 = DMA interrupt has occurred.

3 (write)   DMA interrupt clear.  0 = DMA interrupt held clear, 1 = DMA

interrupt allowed.  When DMA interrupt occurs, this bit

must be set low, then set high to allow another interrupt.

4           DMA interrupt enable.  0 = DMA EOP interrupt disabled, 1 =

DMA EOP interrupt enabled.

5           Sound divider sync enable.  0 = synchronization disabled, 1 =

synchronization enabled.  See below.

6           Sound chip extra divide enable.  0 = extra divide disabled

(10-bit dividers for 3-voice tone generator), 1 = extra

divide enabled (11-bit dividers).

7 (read)    0 = Successive approximation done.  When polling for input,

indicates that a byte of sound is ready to read at port

0C5h (1E5h, 305h).

7 (write)   reserved

Port 0C5h (1E5h, 305h)          Sound Control Register (if 0C4h bits 0-1 = 1)

Read/write

Bit(s)        Meaning

------        -------

0-2         DAC sound channel duty cycle (pulse width):

0 = 6.25% duty cycle

1 = 12.5% duty cycle

2 = 18.75% duty cycle

3 = 25.0% duty cycle

4 = 31.25% duty cycle

5 = 37.5% duty cycle

6 = 43.75% duty cycle

7 = 50.0% duty cycle

3-5         reserved

6-7         DAC waveshape select

0 = pulse

1 = ramp

2 = triangle

3 = reserved

Port 0C5h (1E5h, 305h)          Direct Read/Write Register (if 0C4h bit 1 = 1)

Read/write

If using polling with sound input, read sound data from this port.  If

doing sound output, read current output byte from here, or write a byte

to here for output to the DAC.

Port 0C6h (1E6h, 306h)          DAC Frequency LSB (if 0C4h bits 0-1 <> 0)

Read/write

Least significant 8 bits of divider for frequency of DAC tone when used

as a sound channel (4th tone with 3-voice).  Least significant 8 bits of

divider for frequency of sound DMA when doing sound input or output.  See

below.

Port 0C7h (1E7h, 307h)          DAC Amplitude/Frequency MSB (if 0C4h bits

Read/write                        0-1 <> 0)

Bit(s)        Meaning

------        -------

0-3         Most significant 4 bits of divider for frequency of DAC when

used as a sound channel.  Most significant 4 bits of divider

for frequency of sound DMA when doing sound input or output.

See below.

4           Officially "reserved" - used to determine the chip type

(see below).

5-7         Sound amplitude (if port 0C4h bit 0 = 1).  7 levels, appx.

3 dB per level.  0 = no sound.

--------------------------------------------------------------

On older Tandy 1000's and on the IBM PCjr, bits 5 and 6 at port 61h must be

set to enable the 3-voice chip (bit 6 enables the keyboard and must always

be high).  Bit 0 at this port should be low (this disables timer channel 2

output to the speaker).  These bits can be set/cleared by calling Int 1Ah,

AX=8003h (see above).  Do not change the other bits; on the 1000TL, setting

bit 4 disables the speaker.  It is not necessary to program port 61h on

newer systems with the DAC.

There are various ways of disabling the sound chip and/or the speaker,

depending on the system.  On the 1000TL, the speaker is disabled by setting

bit 4 at port 61h, and the sound chip is disabled by clearing bit 2 at port

0FFEBh.  On the 2500SX, the sound chip can be disabled by setting bit 6 at

port 73h, then clearing bit 4 at port 1F9h - or by clearing bit 2 at port

1FBh.  The speaker and sound chip will normally be enabled, however.

The 3-voice tone and noise generator is addressed at any of a set of 4 I/O

ports.  In other words, if the chip is at 0C0h, the same data can be

written to 0C1h, 0C2h, or 0C3h instead, to the same effect.

The tone frequency for the 3-voice chip is 111,860Hz divided by the 10- or

11-bit divider.  To get 11 bits of division, one must set bit 6 at port

0C4h (1E4h, 304h); this bit is clear by default.  11-bit dividers are only

available on systems with the DAC installed.

With periodic noise code 0, frequency 0 gives a tone at 466Hz (A# above

middle C), frequency 1 gives a tone at 233Hz (A# below middle C), frequency

2 gives a tone at 116.5Hz (second A# below middle C), and frequency 3 gives

a series of clicks.  With white noise code 1, frequencies 0, 1 and 2 give

white noises at successively lower frequencies, and frequency 3 gives an

intermittent noise something like radio static.

Bit 5 at port 0C4h (1E4h, 304h), if set, will cause all 3-voice tone

frequency registers to be reloaded when port 0C6h (1E6h, 306h) or port 0C7h

(1E7h, 307h) is written to.  This makes it possible to guarantee that the

tone frequencies are in phase when using the DAC as a fourth tone voice.

This bit is low by default.

The base frequency for the 12-bit divider programmed in ports 0C6h (1E6h,

306h) and 0C7h (1E7h, 307h) varies according to the DAC function selected

by bits 0-1 at port 0C4h (1E4h, 304h).  When using the DAC as a sound

channel, a base frequency of 111,860Hz is divided to obtain the output tone

frequency.  When using the DAC for sound output with DMA, the base

frequency is 3,579,545Hz.  For sound input with DMA, well, it depends (see

the section on "BIOS Interrupts" above, and the section on "Setting the

Recording Rate" below).

The volume programmed in port 0C7h (1E7h, 307h) is used for sound output,

either digitized sound or tone channel.  The volume is ignored for sound

input.

Bit 4 of the DAC Amplitude/Frequency MSB register (port 0C7h, 1E7, or 307h)

is used to determine whether the DAC has a double buffer for recording.  If

not, it is possible to both set and clear this bit - see the section on

"BIOS Interrupts" above for more information on this.

To get the best sound quality out of the DAC, you will need to program I/O

ports (I've changed my opinion on this ;-).  It is, however, a fairly

intricate process; refer to Recwav.asm and Playwav.asm.  One problem with

playback via the BIOS is that an audible click is produced when

initializing the chip, then another when starting playback, then another

when finalizing the chip.  The clicks can be (mostly) avoided by

programming I/O ports to initialize and finalize the chip and by ramping

the chip from the speaker's rest position up to the baseline and back down

again; the method for doing so is black art, however, and depends on the

chip version, and in Playwav.asm I simply adapted Frank Durda's code

(reportedly based on experimentation at Tandy).

The drawback with recording via the BIOS is that calling Int 1Ah AH=82h

from inside the Int 15h handler (or worse, polling Int 1Ah AH=81h) causes

an audible waver in the recorded sound.  The waver can be reduced by

recording in autoinitialize DMA mode, but you can only do that by

programming I/O ports.  When programming the chip directly to record, you

must be sure to read a sample before you start to prime the approximator

and read another when you're done to drain it.  The first 2-3 samples

recorded will be wrong because of the time it takes the approximator to get

warmed up, so they should be thrown away.  See Recwav.asm.  Note:  The

waver is not completely eliminated by this method, at least on the older

version of the PSSJ without the double-buffer for recording.  On the older

chip, the approximator will not start to convert the next sample until

either the processor or the DMA chip reads the previous one, so variations

in processor/DMA activity (for example, disk access) will cause a waver.

It will, however, be less that what you get from using the BIOS, at least

if you're careful about it (see "Setting the Recording Rate" below).

Note that providing a stairstep wave for the joystick comparators is a

function of the DAC.  You can't play digitized sound at the same time you

read the joystick (although you could use the 3-voice tone and noise

generator).  If the joystick is enabled in the DAC Mode Register (by

default it is at startup), the joystick is programmed like any standard

joystick.  Joystick programming information is widely available; see the

Game Programmer's Encyclopedia for example (available on the Internet -

search archie for "pcgpe10.zip").  One of the reasons for properly

finalizing the sound chip when you're done with it is to make sure the

joystick gets enabled.  It is not difficult to enable the joystick if some

poorly-written program has left it disabled, however; see Setdac.asm.

5.  Setting the Recording Rate

The above note on setting the recording divider on the 1000SL and TL is

only an approximation.  There is no stable relationship between the

recording divider and the playback divider, or between the recording

divider and the sampling rate, on machines without the double buffer on the

sound chip (see Recwav.asm for some code to detect a double-buffered chip).

The situation for recording is thus different from that for playing - the

formula for the playback divider given above is quite reliable.  See

Divider.doc for some experimental data.  Recwav starts with an initial

"guess" at what the divider should be, then times the DAC at various

dividers to set the recording rate.  That method works fairly well.

The 1000SL and 1000TL, produced in 1988, are the only Tandy models that do

not have a double-buffered DAC (most PSSJ cards are also not double-

buffered).  Without the double buffer on record, the approximator will not

begin to calculate the next sample until the previous one has been read,

either directly by the processor from I/O port 0C5h (1E5h, 305h), or by the

DMA chip.  The result is that the recording rate is not a constant, but

depends on the instructions executed by the processor and the consequent

amount of time it takes the processor to relinquish the bus to the DMA.

Hence, disk I/O in particular will cause the recording rate to temporarily

change, resulting in a waver in the recorded sound.  To keep this to a

minimum, Recwav (like the PSSJ Digital Sound Toolkit) limits disk writes to

2k and takes steps to make sure that the writes will be on sector

boundaries.  When recording a pure tone, the waver can still be heard, but

with most sounds, it shouldn't be obtrusive.

This package includes a program, Testdiv, to record and play back short

snippets of sound using various recording and playback dividers.  You can

use Testdiv to hear what different divider values sound like.  The program

is largely self-explanatory.  The sound buffer can be saved to disk as file

Testsnd.ub, a headerless unsigned PCM sound file (Raw2wav can convert it to

.wav if you want).  Note that if you select "Increment playback divider and

play" or "Decrement playback divider and play" from the menu, the playback

divider will be changed before playing, but the new value won't be

displayed until after playback is done.

6.  Playing in Direct Mode

It is possible to play sounds on the Tandy DAC in direct mode, that is,

byte by byte, if that is what your program requires.  Direct writes to the

DAC are not supported by the BIOS, so it is necessary to program the DAC

registers directly to enter this mode.  Once the chip is programmed for

it, however, the Tandy DAC can be treated as just a dumb homemade DAC made

out of a bunch of resistors and attached to a parallel port (or a Covox

Speech Thing, which is a glorified version of the same).

There are obvious advantages and obvious disadvantages to direct mode.  DMA

mode access to the DAC through the BIOS is cleaner, faster, and easier to

program than byte-by-byte access, and it supports higher sampling rates and

better sound quality.  DMA mode access through the hardware, especially

when used with autoinitialize mode DMA, produces the best possible sound

quality.  On the other hand, if you are considering adding Tandy DAC

support to an existing application that already supports homemade or Covox

DAC's, direct mode may be the easiest way for you to do so.

This package includes a program, Setdac.asm, which sets the Tandy DAC for

direct mode playback.  Setdac doesn't do anything by itself; it is designed

to be run in a batch file before another program that uses a homemade or

Covox DAC.  Use of Setdac is described in detail in Tspak.doc.  Once Setdac

is run, any program can write 8-bit unsigned PCM data to the DAC's I/O port

byte-by-byte to generate sounds.  At noted in Tspak.doc, Setdac only

provides for direct mode output of sound.  It is possible to program the

Tandy DAC for direct mode input if anyone can come up with a reason for

doing it (perhaps to implement voice activation), but one can't switch from

input to output or vice versa without reprogramming the chip (the

information above should be helpful if you want to try it).  Refer to

Setdac.asm when reading the following.

The DAC is programmed for direct mode in several steps.  First, detect the

Tandy DAC as described above.  The value returned in AX for Int 1Ah,

AX=8100h is the base I/O port of the DAC.  Verify that the value returned

addresses a set of read/write I/O ports by writing to the port at (base+2)

and reading back from the port.  You should be able to set and clear all

bits.  (It is a good idea to make this check if you're going to program

directly to hardware at all.)  Next, initialize the DAC by polling Int 1Ah

functions 84h and 81h, as for DMA through the BIOS.

Next, program the DAC itself.  The DAC Mode Register is its base I/O port.

Its bit mapping (for writes) is as follows.  Bits 0-2 and 4-6 are

read/write.  To write to the DAC byte-by-byte, bits 0 and 1 of the base

port must be set, bits 2-4 must be clear, and bits 5-7 should be left

alone or clear.

Finally, set the output volume.  The ports at (base+2) and (base+3) are

used to do this.  These ports are read/write.  When using DMA with the

Tandy DAC, port (base+2) and bits 0-3 of port (base+3) are used as a

divider to determine the DMA transfer rate (see above).  I don't know

whether they matter with direct writes or not; I thought it best to set

them to 0.  Bit 4 of port (base+3) is reserved; I set it to zero as well.

Bits 5-7 of port (base+3) determine the output volume; this is the value

programmed in AL for Int 1Ah function 83h.  In Setdac, I set the volume to

the maximum; as noted above, most Tandy's have a volume control knob - and

most programs provide volume control.

If all went well, the DAC is now set to receive 8-bit unsigned samples at

port (base+1), just as if it were another dumb DAC.

7.  Program Examples

Playwav.com is my RIFF WAVE player for the Tandy DAC.  It is an example of

a working application that uses the chip for output with DMA.  This version

of Playwav has been completely rewritten in assembler; it programs I/O

ports on the DAC rather than using the BIOS for better sound quality.  (It

is possible to get better quality still, at least at low sampling rates, by

oversampling - the method used by DeskMate Sound.pdm.)

Recwav.com is a RIFF WAVE recorder for the Tandy DAC.  It is an example of

an application that uses the chip for DMA-mode input.  This version of

Recwav likewise bypasses the BIOS and programs the sound hardware directly.

Setdac.com sets up the DAC either to perform direct-mode (byte-by-byte)

output (before running another program) or for joystick mode (afterwards).

It doesn't perform any output itself (there's a small click when it's

setting up the chip); it is meant to be run in a batch file before or after

another program that uses a DAC on a parallel port.  It can also patch the

BIOS parallel port list to put the DAC in it.

Piano.com is a little time-waster demonstrating the use of the Tandy 1000 /

PC-Jr 3-voice tone generator (a Tandy DAC is not required).  The latest

version has gone through some modification to get it to run on the PCjr as

well as Tandy's (PCjr's have weird keyboards).

Testdiv.exe is an example of a program that uses the BIOS for sound input

and output.  It records and plays back sounds at various divider values.

Programs in this package are written in either Turbo Pascal v5.0 or A86

assembler (or both).  A86 is a shareware assembler by Eric Isaacson,

available on Simtel.  Assembler programs and subroutines in this package

will require modification to be assembled by Masm or Tasm.  .exe programs

written in assembler are linked with the Microsoft Object Linker.  There is

a short .mak file included for each executable file; the .mak file is a

text file describing how to compile or assemble and link the program.

8.  Other Sources

Bruce M. Terry, Jr's., Tlslsnd.asm is a set of subroutines providing access

to the DAC chip.  It was a major source of information for this guide.  The

routines are designed to be called from Microsoft QuickC.  The package is

freeware.

John Ball's Playsnd.exe is a program to play .voc, .wav, .iff, and Tandy

.snd files on either the Tandy DAC or the PC speaker.  His documentation

and letters were an important source, especially regarding the 2500-series.

John has recently decided to release the source code for Playsnd into the

public domain.  He also has a program, Sbplay.exe, to play the same types

of files on the Soundblaster or PC speaker, or on most any soundcard using

the Digpak drivers (see below).

Joseph Albrecht's Tandysnd.com is a TSR providing access to the 3-voice

tone and noise generator through the BIOS video interrupt (Int 10h).

Interface modules for Turbo Pascal, Microsoft QuickBasic, and Spectra

PowerBasic are provided.  This program is freeware.

John W. Ratcliffe's IBM Digitized Sound Package ("Digpak") contains a TSR,

Tansltl.com, which can be called to play sounds (at 9kHz) in the background

on the Tandy DAC, and another Tanhxtx.com, which does the same on the 3-

voice chip.  This driver set seems to support just about every soundcard

ever made, using a standard interface.  There is also a program, Play.exe,

to play sounds in a batch file.  The latest version of this package is free

for non-commercial use (commercial use is $500).  There is glue code for

Borland C and information on the assembler interface.

I have a program called Play3voi that plays digitized sound on the 3-voice

chip as well.  It comes with source if you want to see how I did it.

All of the above are available on my ftp site:

ftp://ftp.agate.net/users/01935/

http://www.agate.net/~tvdog/

Frank Durda's PSSJ Digital Sound Toolkit is derived from the Tandy Digital

Sound Toolkit, a library of sound functions that Tandy distributed with the

DeskMate Software Development Kit.  The PDST is a library linkable with

Microsoft C or compatible.  It is gigantic, and if you want to see some

tight coding at the hardware level, this is where to look (it comes with

source code).  Frank was one of the programmers at Tandy who wrote the TDSL

and the DeskMate Sound and Music applications, and he probably knows more

about this stuff than anybody.  He does not want his work posted on an ftp

site, so I would have to email it to you (he does not, AFAIK, object to

that).  He is planning to put his work on his own Internet site shortly.