EMS41.BIN

A memory manager for the Micro Mainframe 5150 EMS card

by J.L. Hayes

This is the documentation file for the driver, EMS41.BIN.  This file concerns

matters specific to this implementation of the LIM EMS 4.1 standard.  For

general information concerning LIM EMS and version 4.1, consult the official

specification, available on SIMTEL as file LIMEMS41.DOC.

This driver is free and may be copied and distributed at will, provided that

this file is included, unaltered, in the distribution.  If you try EMS41.BIN,

you are requested to send a brief comment giving your opinion of it to me at:

tvdog@delphi.com.  Source code for the driver is available on request.

Contents

--------

1.  Introduction

2.  Installation

3.  Limitations

4.  Nonstandard Features and Compatibility

5.  Reentrancy

Introduction

------------

EMS41.BIN is a device driver that implements the Lotus/Intel/Microsoft Expanded

Memory Standard, version 4.1, on the Micro Mainframe 5150T expanded memory card.

The 5150 is an older EMS card commonly used on Tandy's; it was originally sold

with a driver for version 3.2.  Some newer software (reportedly including DOS

version 6) will not work properly with version 3.2 EMS, hence the need to

upgrade the driver.

Be it noted that Micro Mainframe sells a driver for version 4.0 for $49.95.  I

have never seen that driver.  I called them to ask about it and was told that it

was not really version 4.0, that it would only identify itself as such.  I

thought that fifty dollars was a lot to spend and not get the real thing, so I

wrote this instead.  If you decide you want MMF's driver, they can be reached

at:

Micro Mainframe

120 Blue Ravine Road #2

Folsom, Ca. 95630

ph. (916) 985-7501

This driver is based on (1) the LIM EMS 4.1 standard; (2) a disassembly of Micro

Mainframe's version 3.2 driver; and (3) Douglas Boling's EMS40.SYS EMS emulator

for extended memory from _PC Magazine_, 1989.  Of these, the standard was by far

the most important source of information.  Micro Mainframe's old driver turns

out not to be very well written, at least not very efficient.  It may have been

written in a high-level language.  Nevertheless, it provided important

information on such things as the port addresses of the page mapping registers

and the nonstandard functions.  EMS40.SYS is a miserable, bug-ridden horror, a

menace to any system on which it runs.  It, too, provided some information,

though, helping to clarify some minor points of the standard.

Although the above sources furnished information useful to me while writing this

driver, the driver code is mine.  In particular, no code from Micro Mainframe's

driver is included.

This driver offers the following features:

- Support for all 28 standard functions in LIM EMS version 4.1

- Compatibility with Micro Mainframe's LIM 3.2 driver, including the

nonstandard EMM functions and device driver functions

- Support for /Q and /C options from MMF version 3.2

- Works with all utilities included with MMF version 3.2

- Supports up to 4 EMS cards with up to 8 MB of RAM; cards need not be

jumpered in order

- Provides 1 system and 224 user handles, and 20 internal mapping context

save areas

- Long memory test executes in one pass and is about 1/3 faster than

with MMF version 3.2

- Driver is reentrant, for use by TSR's (see notes below)

- Compatibility mode can be turned on or off under program control

- Best of all, it's free!

Installation

------------

All that is necessary to install EMS41.BIN is to place a line in your CONFIG.SYS

file, like this:

DEVICE=C:\DOS\EMS41.BIN

Substitute drive and path as appropriate.

When loaded, the driver displays the following:

LIM EMS 4.1 driver for Micro Mainframe 5150 EMS cards

- freeware by J.L. Hayes

If the long memory test is to be performed (the default), there follows a

graphical display of the status of the expanded memory.  This display is similar

to what MMF's 3.2 driver displays, but simpler.  In any case, it is self-

explanatory.

Following the memory test display, the number of boards installed, the number of

EMS pages installed, and the compatibility mode setting are displayed.  If no

expanded memory is found in the system, the driver aborts installation with an

error message (oddly enough, MMF's driver remains installed, but inoperative, in

this case).

Two options may be given on the DEVICE= line, the same two available from Micro

Mainframe.  As with MMF, you can specify /C, /Q, /C /Q, /QC, or whatever.

/Q will skip the long memory test and perform a "quick" memory test instead.

The long test tests every single byte of expanded memory, overwriting it all.

It takes a few seconds, although it's quicker with this driver than with MMF's.

The quick test tests only the first word of each page of expanded memory and is

nondestructive; it is also almost instantaneous.  If /Q is used together with

EMSRAMDK.BIN, the RAM disk will persist across warm boots.

/C sets "compatibility" mode.  This disables the three nonstandard expanded

memory manager functions that Micro Mainframe included with their driver.  I

recommend that you use compatibility mode; MMF's utilities will work without the

nonstandard functions anyway (the only case where they won't is if you have no

file handles available).  If you want to use the nonstandard functions in your

own programs, this driver permits your program to temporarily turn compatibility

mode off and turn it back on again (see below).  The problem with leaving the

nonstandard functions active is that these functions are used for different

things by other EMS manufacturers, particularly AST.  Furthermore, the LIM 4.1

standard recommends support for at least one of these AST functions, so a

program would be within its rights to assume that that function, if valid, was

the AST function and not the Micro Mainframe one.

Limitations

-----------

The LIM EMS 4.1 standard defines some functions that support special hardware

features, if those features are present on a particular EMS card.  Since not all

EMS hardware will support these features, means are also specified for the

driver to inform applications about the expanded memory manager's limitations,

and default behavior is defined for cases where a particular feature is not

available.  Contrary to what may be believed, the LIM 4.1 standard does not

require special hardware to run, and a memory manager can be written to meet the

standard that does not use EMS hardware at all (e.g., a disk-based EMS

emulation).  Such a memory manager would be unable to support all features

defined in the standard, but it would be compliant in that it meets the

specification for default behavior in cases where a feature is unsupported.

That is the situation with this driver, since the Micro Mainframe card simply

doesn't have some of the hardware needed to perform particular functions.

Alternate mapping register sets are not supported and are emulated in software

according to the standard.  Some EMS cards have several sets of page mapping

registers, enabling operating system software to switch mapping contexts

instantaneously.  The 5150 has only one set of mapping registers.

Alternate DMA register sets are not supported.  It is not possible to emulate

this in software, and the standard provides a means to query the memory manager

about this capability.  This driver answers such a query in the negative.  Some

EMS cards have the capability of "locking in" a set of page mapping registers

for use by a DMA channel, while applications use another set to access other EMS

pages.

Nonvolatile attribute is not supported.  This refers to the capability of

keeping EMS pages allocated to a handle across warm boots.  I don't know whether

it's really possible to do this with a Micro Mainframe card or not, but this

driver won't do it.  The LIM 4.1 standard notes that RAM refresh signals may be

disabled for an extended period during a warm boot, possibly corrupting memory

contents, so a memory manager is not supposed to provide this feature unless the

card has its own RAM refresh circuitry to ensure that the contents of expanded

memory will remain stable.  It is of course true that EMSRAMDK.BIN will retain

the contents of a RAM disk across warm boots, but this does not rely on the

nonvolatile attribute, which did not exist in version 3.2.  Furthermore, Micro

Mainframe does not guarantee that the contents of the RAM disk will not be

corrupted by a warm boot.  It appears that this aspect of EMSRAMDK.BIN is more

of a safety feature, not to be generally relied on, and that the 5150 does not

have the special refresh circuitry.  At least that's my guess.  The standard

states that software wanting to use the nonvolatile attribute should inquire

about its availability first; this driver says "no."

Some EMS cards have the capability of backfilling conventional memory on systems

whose motherboards are not fully populated with 640k.  The 5150 does not have

this feature, and this driver will so indicate to the appropriate query.  EMS on

this card is accessed only through a 64KB window at 0D0000h.

Some EMS cards have the capability of switching expanded memory in units smaller

than 16KB.  Hardware pages on the 5150 are 16KB long, and finer granularity is

not available.

Nonstandard Features and Compatibility

--------------------------------------

The rest of this file is technical in nature, so if you're not a programmer, you

might as well stop here.

Micro Mainframe added three memory manager functions not defined in the LIM EMS

version 3.2 standard.  It is also possible for a program to read from the

EMMXXXX0 device to get pointers to internal device driver data.  These functions

provide access to certain internal data structures in the driver.  This driver

reproduces the data structures involved, although they do not perform the same

function as they do with MMF's old driver.  LIM version 4.1 is very different

internally from version 3.2, and a great deal of additional state information is

required, so MMF's nonstandard features do not provide the same view of this

driver as they do for MMF's original 3.2 driver.  Nevertheless, if treated as

read-only the data is valid, and MMF's utilities can use the nonstandard

features.

The following nonstandard Int 67h functions are defined.  These functions are

disabled and return AH=84h, invalid function, when compatibility mode is set.

AH=60h.  Get Page Owner Table.  Caller passes a pointer to a 1-kilobyte buffer

in ES:DI.  The memory manager fills the buffer with an array of 512 words, one

for each possible 16-kilobyte EMS page.  The codes for the words are:  0FFFFh,

meaning that the page is bad or not installed; 0, meaning that the page is

available and unallocated; and 80xxh, meaning that the page is allocated by

handle xx.  The pages start at card 0, bank 0, go up to card 0, bank 7, then

proceed with card 1 and so forth.  With MMF's 3.2 driver, you could assume

that the 8th entry containing 8003h was logical page 7 for handle number 3.

With this driver, that is no longer true, and the logical pages for a handle

need not be stored on the card(s) in any particular order.

AH=61h.  Get Context Save Area Contents.  Caller passes a pointer to a 381-byte

buffer in ES:DI.  The memory manager writes a byte giving the number of

installed EMS cards at the beginning of the buffer, then copies out the

contents of the internal mapping context save area (i.e., the one used with

functions AH=47h and AH=48h).  The context save area consists of 20 19-byte

records of the following format:

offset    length           contents

0       byte     flag:  0 = context allocated to handle; 0FFh =

context not in use

1       word     handle number if allocated

3      16 bytes  page mapping register contents, in the following

order:  208h, 4208h, 8208h, 0C208h, 20Ah, 420Ah,

820Ah, 0C20Ah, 20Ch, 420Ch, 820Ch, 0C20Ch, 20Eh,

420Eh, 820Eh, 0C20Eh

AH=62h.  Get I/O Port and Value Array.  Caller specifies a physical page number

in AL, a handle in DX, and a pointer to a buffer in ES:DI.  The memory manager

fills the buffer with an array of 3-byte records, one for each logical page

allocated to the specified handle, where each record consists of a word I/O

port address followed by a byte data value.  The port and data value are those

that would be used with an OUT instruction to map the logical page for the

handle into the specified physical page.  Note that it is not that simple,

however; any logical page that is currently mapped into the physical page has

to be mapped out first, or you could wind up mapping two different logical

pages into the same physical address space.  I strongly recommend that

programs not mess around with the page mapping registers; let the memory

manager handle that.

None of the Micro Mainframe utilities uses this function, and I'm somewhat

at a loss to figure what it's for.  I finally concluded that it could be used

for diagnostics.  The I/O port and data byte can be used to determine the card

and location on the card of the physical memory that contains the logical

page, in the following way.  The two most significant bits of the port number

are the physical page.  Bits 1 and 2 of the port number are the card number

(0 if you only have one card and you jumpered it right).  Bits 4 through 6 of

the data byte are the bank number, and bits 0 through 3 are the number of the

page within the bank.  Armed with this information, you could write a program

to test EMS for intermittent failures and display failing banks, if you were

so inclined.

In addition to the three nonstandard memory manager functions, Micro Mainframe

permits access to the expanded memory manager through the DOS device driver

interface.  This is the method preferred by MMF's utilities for gaining access

to the nonstandard features, and this method will work in compatibility mode as

well.

Although the LIM EMS standard specifies that the expanded memory manager be

structured as a character device with the name "EMMXXXX0," the standard does not

specify that you should actually be able to read from or write to the device as

you can with, say, CON or AUX.  With Micro Mainframe's 3.2 driver, and with this

driver, you can.

Reading from the EMMXXXX0 device returns up to 10 bytes:

bytes                 contents

0,1      segment address of the device driver

2,3      offset in the driver segment of a word giving the number of

expanded memory boards installed

4,5      offset in the driver segment of the page owner table

6,7      offset in the driver segment of the internal context save area

8       ASCII letter "B"

9       ASCII letter "W"

If more than 10 bytes are read, reading wraps back around to the beginning.  If

fewer than 10 bytes are read, the next read will return data beginning with the

first unread byte.  The letters "B" and "W" are used by MMF's utilities to check

the validity of the data read.

Writing to the EMMXXXX0 device resets the read pointer to the beginning of

the 10 bytes.  To make sure you read the right data, a read should be preceded

by a write.  The actual data written to the device is ignored.

It is obvious that reading from the EMMXXXX0 device provides information

similar to Int 67h functions 60h and 61h.  There is one difference, though:  the

pointer returned by the device refers to the original copy in the device

driver's memory, while the interrupt fills the caller's buffer with a copy.

However, this may be a distinction without a difference if as recommended you

treat the data as read-only.

As a single exception to the idea of treating driver data as read-only, I

mentioned above that this driver (but not MMF's old one) permits programs to

turn compatibility mode on and off.  Note that bytes 2 and 3 returned by the

EMMXXXX0 device give the offset in the driver segment of a word giving the

number of installed EMS cards.  Well, I put the compatibility mode flag in the

byte immediately preceding that word.  Two values are valid for the flag:  0,

meaning that compatibility mode is off and nonstandard functions are available;

and 0FFh, meaning that compatibility mode is on and nonstandard functions cannot

be used.  A program can write as well as read the flag to turn compatibility

mode on and off.

I recommend that compatibility mode be on as a general rule and turned off

as needed by programs that want to use the nonstandard functions.  The reason is

that AST defines AH=60h as Get Physical Window Array, and AH=61h as Generic

Accelerator Card Support.  To make matters worse, Get Physical Window Array

returns an array through ES:DI just like MMF's Get Page Owner Table.  Also, the

LIM 4.1 standard recommends that support for AST's version of AH=61h be provided

by the memory manager.  (This driver doesn't.  I called AST to see what their

function AH=61h did, but no one knew.)  Micro Mainframe notes that DesqView and

PC-Tools can crash without compatibility mode, so apparently there are programs

that will get confused.

Reentrancy

----------

LIM EMS versions 3.2 and 4.1 both require that the expanded memory manager never

be busy, but always be ready to process requests.  Neither of the two drivers I

looked at is reentrant, however, so in general it seems that a program shouldn't

rely on an arbitrary memory manager having this feature.  Micro Mainframe's

version 3.2 driver stores the caller's registers in a static data area, so a

reentrant request will clobber the first caller's registers.  The _PC Magazine_

driver stores the caller's registers on the stack, but reentrant requests can

still fail since not all critical code is protected from interrupts.  For

example, it is possible for one process to request that a page be allocated, the

EMM to check and see that one is available, then for a second process to jump in

and "steal" the page before the first process gets it.

This driver is reentrant, within reason.  Unreasonable things include chiefly

reentrant requests involving the same handle, and reentrant move/exchange

operations involving memory that is already being moved/exchanged.  What do you

think should happen if a request to allocate additional pages to a handle is

interrupted by a request to deallocate the handle?  Well, nothing sensible will

happen, and the memory manager will get messed up so that you will have to

reboot to fix things.  In general, a TSR that uses expanded memory is required

to (a) allocate and use its own handle(s), (b) save the mapping context and

restore it before returning, and (c) use its own conventional memory.  Note also

that all data structures used with EMM functions must be in memory that won't be

mapped out (i.e., when a pointer is passed to the memory manager, the pointer

should address conventional memory).  The only exceptions to this are the Alter

Map and Jump and Alter Map and Call functions, where the target of the jump or

call is allowed to be in expanded memory.