SK-NET FDDI Driver for Windows NT

Installation

August 1994

Software version 2.01 beta 01

SKFMNT.SYS is the driver for the SK-NET FDDI MCA adapter

under the first release of Windows NT on Intel processors.

The driver corresponds to the NDIS 3.0 Specification as delivered with

the first release of the Windows NT DDK.

Files needed

In the root of the installation diskette

DISK3		Indicator.

In a single directory

OEMNADSK.HLP	Help file for SK-NET drivers.

OEMSETUP.INF	Setup script for the driver.

SKFMNT.SYS	The driver.

How to install

After having configured your adapter, please start Windows NT. There,

start NETWORK from the CONTROL PANEL, choose ADD ADAPTER and then

<OTHER> and type in the path to OEMSETUP.INF.  Choose the driver

presented and select the appropriate configuration.  Then choose OK

and restart your system.  (Protocol software should be installed.)

Troubleshooting

If you encounter problems using the SK-NET FDDI adapter under

Windows NT, please re-read carefully the instructions above and the

documentation supplied with the system. Additionally, you should look

into your system log file (using the event viewer) and check it for

entries from SKFMNT. The last 5 DWORDs of the logging data are:

Slot number,

Interrupt number,

Dma arbitration level,

Base I/O port and

Vendor code (3 bytes) / adapter id (1 byte).

of the adapter that caused the error.

Following is a list of some events, possible reasons (may be

incomplete) and suggestions to fix your problem.

Event ID:	5001 -	Could not allocate the resources necessary

for operation.

Reason(s):	Not enough RAM or pagefile too small.

Help:		...

---------------

Event ID:	5003 -	Could not find an adapter.

Reason(s):	Wrong slot.

Help:		Check configuration.

---------------

Event ID:	5004 -	Could not connect to the interrupt number

supplied.

Reason(s):	The selected interrupt is already in use.

Help:		Select free interrupt.

---------------

Event ID:	5007 -	Timed out during an operation.

Reason(s):	Wrong interrupt, dma channel or base address.

Cable not connected.

Help:		Run the diagnostic program supplied with the

adapter or start a driver under DOS (with the same

resource usage) and look if it works.

Check cabling.

---------------

Event ID:	5009 -	Does not support the configuration supplied.

Reason(s):	Shared interrupt configured.

Help:		The driver does not support interrupt sharing.

Please re-configure the adapter to not using shared

interrupts.

---------------

Event ID:	5014 -	The adapter is disabled.

The driver cannot open the adapter.

Reason(s):	The adapter was not enabled at system startup.

Help:		First, check the system configuration. If it looks ok,

run the diagnostic program supplied with the adapter.

If your special problem is not listed or the error persists, please

send us your system log file or at least the event id number and the

logging data (as dwords) of the entries written by SKFMNT.