ZPC Assembly Source Code Corrections

The  following  correction  should be made to your  ZPC.ASM  file

regardless of the dates on your ZPC files.  Load the file ZPC.ASM

into your editor, and locate the label WROLP:.  A few lines down,

locate the following code:

IF      LEVEL1

MOV     CURPOS,DX               ;UPDATE CURSOR

ENDIF

IF      LEVEL2 OR LEVEL3

MOV     DS,BIOSRAM

MOV     CURPOS,DX

PUSH    CS

POP     DS

ENDIF

Remove these lines at this one location only.

The  following  corrections apply if your ZPC3.COM file is  dated

BEFORE 10-4-85.   To correct your ZPC source files first load the

file ZPC.ASM into your editor, and locate the label SETMOD1:, and

add the line with * in the comment shown below:

SETMOD1:CALL    CUROFF                  ;TURN CURSOR OFF

MOV     XORFLG,0                ;* KILL XOR FLAG

Load  in the file DOS.ACM,  and change the lines from  the  label

MYSYS: to the label FUNCOK: so they look like this:

MYSYS:  CMP     CS:PCFLG,0              ;PC MODE ON?

JZ      MYEXIT                  ;IF NOT, EXIT

OR      AH,AH                   ;FUNCTION ZERO?

JZ      MYEXIT                  ;YES, GO TO DOS

CMP     AH,13                   ;SUPPORTED FUNCTION?

JC      FUNCOK                  ;YES

CMP     AH,63                   ;XENIX READ?

JZ      READOK

IF      LEVEL3

CMP     AH,48H                  ;FREE MEMORY?

JZ      FIXSIZ                  ;IF SO, FIX RESULT

ENDIF

CMP     AH,64                   ;XENIX WRITE?

JNZ     MYEXIT                  ;UNSUPPORTED FUNC.

CMP     BX,1                    ;STD OUT?

JNZ     MYEXIT

JMP     SHORT FUNCOK

READOK: CMP     BX,0                    ;STD IN?

JNZ     MYEXIT

FUNCOK: PUSH    BX                      ;SAVE BX

Locate  the comment "TABLE OF I/O ROUTINES",  and add these lines

before it.

IF      LEVEL3

;       FIX MEMORY SIZE ON FREE MEMORY CALL

FIXSIZ: CMP     BX,0FFFFH               ;ASK FOR MAX?

JNZ     MYEXIT                  ;IF NOT, EXIT

PUSHF                           ;FAKE DOS INT.

PUSH    CS

MOV     AX,OFFSET FIXRET

PUSH    AX

MOV     AH,48H

JMP     MYEXIT                  ;CALL DOS FUNC. 48H

FIXRET: CMP     BX,1000H                ;AT LEAST 1000H?

JC      FIXEX                   ;NO, LEAVE ALONE

SUB     BX,1000H                ;ALLOW FOR VRAM

STC

FIXEX:  STI

RETFD                           ;RETURN

DW      2

ENDIF

;       TABLE OF I/O ROUTINES 1-12

Locate  the  label XREAD:,  and delete all lines from it  to  the

label  XRET:,  except the one containing JMP  XREADSI,  as  shown

below:

XREAD:  JMP     XREADSI                 ;ELSE, SPECIAL CASE

XRET:   CLI

MOV     SS,CS:STKS              ;RESTORE STACK

MOV     SP,CS:STK

POP     BX

STI

RETFD

DW      2                       ;FLAGS SKIPPED

XREADSI:PUSH    CX

Locate the label XWRITE:, and delete all lines between it and the

label XWRIT1 except the ones shown below:

XWRITE: PUSH    DI

PUSH    BX

MOV     DI,OFFSET MYCNOUT       ;ASSUME STD OUTPUT

XWRIT1: PUSH    SI

Load the file KEY.ACM, and locate the line shown below:

MOV     BX,OFFSET KEYTBL2       ;ELSE, USE ALT

Change it to look like this:

MOV     BX,OFFSET (KEYTBL2-40H) ;ELSE, USE ALT

Load  the  file PIXEL.ACM,  and locate the comment "TEST FOR  XOR

BIT".  If the instruction there is TEST AL,8, change it to

TEST    AL,80H                  ;TEST FOR XOR BIT

Locate  the label READDOT:,  and change the lines from it to  the

comment "GET BYTE POSITION" so that they look like  this.   Note:

depending  on the date of your PIXEL.ACM file,  these changes may

have already been made.

READDOT:MOV     CSFLG,0                 ;DISABLE CURSOR

PUSH    BX

PUSH    CX

PUSH    DX

IF      LEVEL3

MOV     ES,PCRAM                ;POINT TO PC VIDEO

CALL    GVADR1                  ;GET VIDEO ADDRESS

ELSE

CALL    GVADR                   ;GET VIDEO ADDRESS

ENDIF

MOV     AH,DL                   ;PUT POSITION IN AH

CMP     MODE,6                  ;HI RES MODE?

JZ      HIRES1                  ;GO DO IT

SHL     AH,1                    ;DOUBLE POSITION

INC     AH                      ;ADD 1

MOV     AL,DL                   ;GET BYTE POSITION

A  few lines below the above change,  locate the comment  "INVERT

IT".  If the instruction there is "NOT DL", change it to this:

NOT     AL                      ;INVERT IT

Locate  the label GVADR:,  and change the code from there to  the

comment "SAVE ROW" so that it looks like this:

GVADR:  MOV     AX,VIDRAM               ;POINT TO VIDEO RAM

ADD     AX,100H                 ;SKIP TEXT AREA

MOV     ES,AX

GVADR1: XOR     DH,DH                   ;ENSURE ROW IN RNGE

MOV     SI,DX                   ;SAVE ROW

Assembly Corrections for MS-DOS Version 3

If you would like to alter your assembly files so that ZPC can be

assembled for use under either version 2 or version 3 of  MS-DOS,

load the file COND.ACM,  and add this line as the last one in the

file:

DOS3    EQU     TRUE                    ;ASSEMBLE FOR DOS VERS. 3

Load the file ZPC.ASM,  and add these lines near the beginning of

the file, with the other EQUate statements.

IF      DOS3 AND LEVEL3

CODE1   EQU     0A326H                  ;CODE ALTERED IN MS-DOS

CODE2   EQU     2

CODE3   EQU     0C22BH

ELSE

CODE1   EQU     8926H                   ;CODE ALTERED IN MS-DOS

CODE2   EQU     236H

CODE3   EQU     0

ENDIF

Locate the label NSZCHK:, and change the lines from the MOV AX,CS

statement above it to the label to look like this:

MOV     AX,CS

STOSW

ENDIF

IF      DOS3 AND LEVEL3

MOV     AL,90H

STOSB

ENDIF

NSZCHK: MOV     AX,3

CALL    SETMODE                 ;SET DEFAULT VIDEO MODE

Locate  the  label NSZCHK1:,  and change the lines from  the  MOV

ES,BIOSRAM instruction above it to the label to look like this:

MOV     ES,BIOSRAM              ;ELSE, POINT TO BIOS RAM

MOV     AX,CODE1

STOSW                           ;RESTORE OLD SIZE CODE

MOV     AX,CODE2

STOSW

ENDIF

IF      LEVEL3 AND NOT DOS3

MOV     AL,LOW CODE3

STOSB

ENDIF

IF      LEVEL3 AND DOS3

MOV     AX,CODE3

STOSW

ENDIF

NSZCHK1:MOV     AL,VPVAL

OUT     VRPORT,AL               ;SET OLD VR PORT VALUE

RET

Locate  the label SIZCHK:,  and change the code from there to the

RETF statement to look like this:

SIZCHK:

ENDIF

IF      LEVEL3 AND NOT DOS3

CMP     SI,CS:MEMSV             ;CHECK SIZE

JBE     SIZOK                   ;IT'S OK

MOV     SI,CS:MEMSV             ;ELSE, LIMIT IT

SIZOK:  MOV     ES:MEMLIM,SI            ;SAVE NEW SIZE

RETF

ENDIF

IF      LEVEL3 AND DOS3

CMP     AX,CS:MEMSV             ;CHECK SIZE

JBE     SIZOK                   ;IT'S OK

MOV     AX,CS:MEMSV             ;ELSE, LIMIT IT

SIZOK:  MOV     ES:MEMLIM,AX            ;SAVE NEW SIZE

SUB     AX,DX

RETF

ENDIF

Locate the label SSRCH:,  and change the code from one line above

it to the label SCFND: to look like this:

MOV     AX,CODE1                ;SEARCH FOR THIS

SSRCH:  REPNZ   SCASW                   ;DO THE SEARCH

JNZ     SHSRCH                  ;NOT FOUND, SHIFT SEARCH

CMP     ES:WORD PTR [DI],CODE2  ;IS THIS IT?

JNZ     SSRCH                   ;NO

CMP     ES:BYTE PTR 2[DI],LOW CODE3     ;CHECK LAST BYTE

JNZ     SSRCH                   ;NO

JMP     SHORT SCFND             ;FOUND SIZE CODE

SHSRCH: MOV     DI,4001H                ;START SEARCH HERE, NOW

MOV     CX,4000H

SSRCH1: REPNZ   SCASW                   ;DO THE SEARCH

JNZ     SCNFND                  ;CODE NOT FOUND

CMP     ES:WORD PTR [DI],CODE2  ;IS THIS IT?

JNZ     SSRCH1                  ;NO

CMP     ES:BYTE PTR 2[DI],LOW CODE3     ;CHECK LAST BYTE

JNZ     SSRCH1                  ;NOT IT

SCFND:  DEC     DI                      ;BACK UP TO CODE START

#########################################