RANCHO TECHNOLOGY'S ASPI MANAGER FOR THE RT1000C MULTIMODE SCSI HOST ADAPTER.

=============================================================================

Manual ASPI manager installation

---------------------------------

To install this ASPI manager, simply add the following line

to your CONFIG.SYS file:

DEVICE=<path>\RTASPI1C.SYS /Timeout=15 Scan Info Busy_Retry

RTASPI1C.SYS is for use with the RT1000C Multimode SCSI Host Adapter in the

I/O-mapped and memory-mapped modes depending on whether or not the J8 jumper

at SW1 on the RT1000C is ON. The RTASPI1C.SYS supports up to 6 host adapters

and up to 7 CD-ROMs on each host adapter.

To add more RT1000C host adapters:

_ Make sure the I/O or memory address of one host adapter is

different from that of another.

_ Then, from the hard disk in which CorelSCSI was installed,

type "INSTALL" to update the CONFIG.SYS and AUTOEXEC.BAT files.

To use all the utilities for Floptical drives and hard drives included

with the product diskette, make sure that you add the installed

subdirectory to the `PATH' statement in AUTOEXEC.BAT file.

Example:

PATH = C:\RT1000C

Please report any problems with Rancho's ASPI manager to:

Rancho Technology Inc.

10783 Bell Court

Rancho Cucamonga, CA 91730

(909) 987-3966

FAX: (909) 989-2365